3GPP TSG-RAN WG4 Meeting #90bis	R4-1903894

# Xi'an, China, 8  - 12 April 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR 38.133 (B.2.4.2) Correction of CSI-RS based L1-RSRP reporting condition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-04-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Unit of CSI-RS based L1-RSRP reporting is conditin is wrong. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the unit of CSI-RS based L1-RSRP reporting from SCSSSB to SCSCSI-RS Add the condition of SCSCSI-RS =60kHz for both FR1 and FR2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CSI-RS based L1-RSRP reporting test is based on the wrong condition. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | B.2.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS38.533... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------------------- Beginning of Change ------------------------------------------------------------

### B.2.4.2	Conditions for CSI-RS based L1-RSRP reporting

This section defines the following conditions for NR L1-RSRP measurement reporting and corresponding procedures performed based on CSI-RS: CSI-RS_RP and CSI-RS Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.4.2-1 for FR1 NR cells.

The conditions are defined in Table B.2.4.2-2 for FR2 NR cells.

Table B.2.4.2-1: Conditions for intra-frequency measurements in FR1

| Parameter | NR operating band groups Note1 |  |  | Minimum CSI-RS RSRP |  |  |  |  | CSI-RS Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | dBm / SCSCSI-RS |  |  |  |  | dB |
|  |  |  | SCSCSI-RS = 15 kHz |  |  | SCSCSI-RS = 30 kHz |  | SCSCSI-RS = 60 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A |  | -127 |  |  | -124 |  | -121 | -3 |
|  | NR_FDD_FR1_B |  | -126.5 | -123.5 | -120.5 |  |
|  | NR_TDD_FR1_C |  | -126 |  |  | -123 |  | -120 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  | -125.5 |  |  | -122.5 |  | -119.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  | -125 |  |  | -122 |  | -119 |  |
|  | NR_FDD_FR1_G |  | -124 |  |  | -121 |  | -118 |  |
|  | NR_FDD_FR1_H |  | -123.5 |  |  | -120.5 |  | -117.5 |  |
| NOTE 1: NR operating band groups are defined in Section 3.5.2. |  |  |  |  |  |  |

Table B.2.4.2-2: Conditions for intra-frequency measurements in FR2

| Parameter | NR operating band groups Note1 |  | Minimum CSI-RS_RSRP |  |  | CSI-RS Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSCSI-RS |  |  | dB |
|  |  |  | SCSCSI-RS = 60 kHz |  | SCSCSI-RS = 120 kHz |  |
| Conditions | NR_TDD_FR2_A | TBD | TBD | TBD |
|  | NR_TDD_FR2_B | TBD | TBD |  |
|  | NR_TDD_FR2_F | TBD | TBD |  |
|  | NR_TDD_FR2_G | TBD | TBD |  |
|  | NR_TDD_FR2_T | TBD | TBD |  |
|  | NR_TDD_FR2_Y | TBD | TBD |  |
| NOTE 1: NR operating band groups are defined in Section 3.5.3. |  |  |  |  |

------------------------------------------------------------- End of change ------------------------------------------------------------