3GPP TSG-RAN WG4 Meeting #90b                 	 R4-1903913

Xi’an, China, April 8th– 12th, 2019

Agenda Item:	6.11.3.2.5

Source:	MediaTek Inc.

Title:	Rough and fine beam assumptions for FR2 RRM test cases

Document for:		Discussion

- Introduction

In this paper, we provide our view on whether rough beam or fine beam should be assumed for each FR2 RRM test case.

- Discussion

RAN4 is now discussing FR2 test case setup. One of the important discussion point is on whether we should assume UE is using rough beam or fine beam to pass each test case. The selection of the rough beam and fine beam mainly has two impact on the test case setup. One is on Noc level in the reference point, and the other is on the beamforming margin for L3 and L1 RSRP accuracy tests.

Regarding the Noc level, if we assume UE is using rough beam in a certain test case, we will apply a higher Noc level to guarantee the SNR observed in UE’s baseband will not be impacted by UE’s noise figure. For the test case with fine beam, the Noc level will be lower. If UE uses rough beam in this test case, it will observe a lower SNR in the baseband, and the probability to fail the test case increases.

In our view, UE will use rough beam for the following operations

- Cell search

- L3 measurement

- MIB and SIB decoding for non-serving cell

- RACH based on L3 measurements

Therefore, if UE has to perform one of above operations in one of the steps in the test case, then the Noc of this test case should be set based on rough beam.

For test case not involved with any of above operations, we can assume UE is using fine beam, e.g., RLM, BFR, BWP switch.

Note that the core requirement for the activation of the 1st SCell in the FR2 band is still ongoing. Whether UE is assumed to use rough or fine beam is pending on the final conclusion.

Proposal 1: Whether UE is assumed to use rough or fine beam for the test case of 1st SCell activation in the FR2 band is pending on the conclusion of core requirement.

Proposal 2: UE is assumed to use rough beam for the following operations

- Cell search

- L3 measurement

- MIB and SIB decoding for non-serving cell

- RACH based on layer 3 measurements

Otherwise, UE is assumed to use fine beam.

In Table 1, we provide our view on the rough/fine beam assumption for each test case.

Table 1 Suggestion on rough and fine beam for each test case

| Category | Sub-category | # | Test purpose | Critical RS or PHY channel which has direct impact to performance | Rough or fine beam |
| --- | --- | --- | --- | --- | --- |
| L3 measurement | delay | 1 | EN-DC cell search and L1 measurement period | - SSB for cell search and measurement | Rough N.A, if no such a test case in FR2. |
|  |  | 2 | SA cell search and L1 measurement period |  |  |
|  |  | 18A | EN-DC NR inter-frequency measurement |  |  |
|  |  | 18B | SA NR inter-frequency measurement |  |  |
|  |  | 47 | E-UTRAN inter-RAT NR cell search and measurement delay |  |  |
|  |  | 15 | EN-DC SFTD measurement delay | - SSB for cell search and timing estimation |  |
|  |  | 17A | Serving NR PSCell and target E-UTRA inter-frequency measurement with LTE PCell | - All performance-related RSs are in FR1 |  |
|  |  | 17B | NR Pcell with target inter-RAT E-UTRA measurement |  |  |
|  | accuracy | 11 | Intra-frequency RSRP accuracy for FR1 and FR2 | - SSB for measurement |  |
|  |  | 19 | Inter-frequency RSRP accuracy for FR1 and FR2 |  |  |
|  |  | 31 | Intra-freq RSRQ accuracy for FR1 and FR2 |  |  |
|  |  | 32 | Inter-freq RSRQ accuracy for FR1 and FR2 |  |  |
|  |  | 40 | SA/EN-DC SS-SINR measurement accuracies |  |  |
|  |  | 49 | E-UTRAN inter-RAT NR measurement accuracy |  |  |
|  |  | 30 | EN-DC SFTD measurement accuracy | - SSB for cell search and timing estimation |  |
|  |  | 43A | SA NR inter-RAT E-UTRAN RSRP measurement accuracy test cases | - All performance-related RSs are in FR1 |  |
|  |  | 43B | SA NR inter-RAT E-UTRAN RSRQ measurement accuracy test cases |  |  |
|  |  | 43C | SA NR inter-RAT E-UTRAN SINR measurement accuracy test cases |  |  |
| timing | Tx timing | 3 | EN-DC Timing accuracy and adjustment | - SSB for sync and SRS spatial association | Fine SMTC should not overlap all SSB occasions |
|  |  | 4 | SA Timing accuracy and adjustment |  |  |
|  | TA | 5 | EN-DC TA accuracy | - SSB for sync - PDSCH/PDCCH for MAC-CE decoding |  |
|  |  | 6 | SA TA accuracy |  |  |
|  | MTTD | 38A | EN-DC MTTD |  |  |
|  |  | 38B | NR CA MTTD |  |  |
| RLM/BFD/CBD/L1-RSRP | RLM | 7 | EN-DC SSB RLM for PSCell IS and OOS | - SSB for OOS/INS | Fine SMTC should not overlap all SSB occasions |
|  |  | 9 | SA SSB RLM for PCell IS and OOS |  |  |
|  |  | 13A | EN-DC CSI RLM for PSCell | - CSI-RS for OOS/INS |  |
|  |  | 13B | SA CSI RLM for PCell |  |  |
|  | L1-RSRP | 29A | Beam management: L1-RSRP reporting | - SSB - CSI-RS |  |
|  | BFD/CBD | 29B | Beam management: Beam failure detection and link recovery procedure |  |  |
|  | scheduling restriction | 25 | EN-DC/SA SSB RLM scheduling restriction and impact on mobility | - PDSCH/PDCCH for DCI and PDSCH decoding |  |
|  |  | 39 | EN-DC/SA beam failure detection and recovery and scheduling restriction | - SSB - CSI-RS - PDSCH/PDCCH for DCI and PDSCH decoding |  |
| CONNECTED mobility | RACH | 10 | Random access | - SSB for RACH - PDSCH/PDCCH for RAR | Rough N.A, if no such a test case in FR2. |
|  | Handover | 26A | NR-NR Handovers | - SSB for sync, MIB decoding and RACH - PDSCH/PDCCH for RRC command decoding |  |
|  |  | 26B | NR handovers to other RATs |  |  |
|  |  | 48 | E-UTRAN inter-RAT NR handover |  |  |
|  | Re-establishment | 27 | RRC Re-establishment | - SSB for cell search, measurement and RACH |  |
|  | Re-direction | 28 | RRC Release with redirection to NR/E-UTRAN | - SSB for cell search, measurement and RACH - PDSCH/PDCCH for decoding UL grant |  |
| IDLE/INACTIVE mobility | reselection | 38 | SA RRC_Idle/inactive cell reselection NR to NR (FR1) | - SSB for cell search, measurement and RACH - PDSCH/PDCCH for RMSI | Rough N.A, if no such a test case in FR2. |
|  |  | 39 | SA RRC Idle/inactive cell reselection NR to E-UTRAN (FR1) |  |  |
|  |  | 46 | E-UTRAN cell reselection to NR target cell |  |  |
| Signal characteristic | Delay: SCell | 12 | EN-DC SCell activation/deactivation delay | - SSB for cell search and sync - CSI-RS for CQI reporting | TBD, pending on core requirement |
|  | Delay: BWP | 35 | BWP switching delay | - PDSCH/PDCCH for DCI and PDSCH decoding | Fine |
|  | Delay: PSCell | 36 | NR PSCell addition and release in EN-DC | - SSB for sync and RACH | Rough |
|  | Delay: UL | 37 | UL carrier RRC reconfiguration delay | - PDSCH/PDCCH for RRC command decoding | N.A, if no such a test case in FR2. |
|  | Delay TCI | 42 | TCI switch delay | - SSB - CSI-RS | Fine |
|  | interruption | 14A | EN-DC interruptions due to DRX transition | - PDSCH/PDCCH for determining missed ACK/NACK | Fine |
|  |  | 21A | SA interruptions at SCell addition/release/activation/deactivation | - SSB for cell search, sync - CSI-RS for CQ reporting - PDSCH/PDCCH for determining missed ACK/NACK | TBD on aggressor, pending on core requirement Fine on victim |
|  |  | 14B | EN-DC interruptions due to deactivated SCell operations | - SSB for cell search, sync, measurement - PDSCH/PDCCH for determining missed ACK/NACK |  |
|  |  | 20C | EN-DC interruptions due to measurement on deactivated SCell |  |  |
|  |  | 21D | SA interruptions due to measurement on deactivated SCell |  |  |
|  |  | 20B | EN-DC interruptions due to active BWP switching | - PDSCH/PDCCH for DCI decoding and determining missed ACK/NACK | Fine |
|  |  | 21C | SA interruptions due to Active BWP switching |  |  |
|  |  | 34 | BWP switching interruptions on E-UTRA serving cells in EN-DC | - N.A. No cross FR1-FR2 test |  |
|  |  | 20A | EN-DC interruptions at UL carrier RRC reconfiguration | - N.A. test case deleted | N.A, if no such a test case in FR2. |
|  |  | 21B | SA interruptions at UL carrier RRC reconfiguration |  |  |

- Summary

In this paper, we provide our view on whether rough beam or fine beam should be assumed for each FR2 RRM test case. We have below high-level proposals.

Proposal 1: Whether UE is assumed to use rough or fine beam for the test case of 1st SCell activation in the FR2 band is pending on the conclusion of core requirement.

Proposal 2: UE is assumed to use rough beam for the following operations

- Cell search

- L3 measurement

- MIB and SIB decoding for non-serving cell

- RACH based on layer 3 measurements

Otherwise, UE is assumed to use fine beam.