3GPP TSG-RAN WG4 Meeting#90bis                	R4-1903919

Xi’an, China, April 8th – 12th, 2019

Source:	Nokia, Nokia Shanghai Bell

Title:	Updated simulation results for L1-RSRP accuracy measurements for FR1

Agenda item:	6.11.2.2.1

Document for:	Discussion

Introduction

Already in RAN4 meeting in Gothenburg RAN4 agreed on simulation assumptions for L1-RSRP accuracy measurements in [1]. In the last couple of meetings simulation results based on the agreed assumptions results have been provided to RAN4 e.g. in [3]. In Athens meeting RAN4 again agreed a set of assumptions for simulations [2] to facilitate determination of the CSI-RS measurement bandwidth as well as CSI-RS and SSB based L1-RSRP measurement accuracy.

In this paper we provide our simulation results for FR1. Results for FR2 are presented in [4].

Discussion

Following 2 section include the simulation results for the SSB-based and CSI-RS based L1-RSRP measurements for FR1, respectively.

CSI-RS-based based L1-RSRP simulation result

Following results are for the CSI-RS based L1-RSRP simulation result.

AWGN, 4GHz, 20ms CSI-RS periodicity

Carrier frequency: 4GHz

SCS: 15KHz, 30KHz

SNR: -3dB

CSI-RS periodicity: 20ms

D: 3

| samples | SCS [KHz] | Measurement BW [PRBs] | 5 % | 50 % | 90 % | 95 % | absolute accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 15 | 24 | -1,03 | 0,68 | 1,78 | 2,06 | 2,06 |
| 1 | 15 | 48 | -0,74 | 0,48 | 1,3 | 1,51 | 1,51 |
| 1 | 15 | 96 | -0,49 | 0,34 | 0,93 | 1,1 | 1,1 |
| 1 | 30 | 24 | -0,99 | 0,69 | 1,76 | 2,07 | 2,07 |
| 1 | 30 | 48 | -0,7 | 0,48 | 1,29 | 1,49 | 1,49 |
| 1 | 30 | 96 | -0,5 | 0,34 | 0,94 | 1,08 | 1,08 |

SSB-based based L1-RSRP simulation result

Following results are for the SSB based L1-RSRP simulation result for FR1.

AWGN, 4GHz, 25RB

Carrier frequency: 4GHz

SCS: 15KHz, 30KHz

SNR: -3dB

SSB periodicity: 20ms

| samples | SCS [KHz] | Measurement BW [PRBs] | 5 % | 50 % | 90 % | 95 % | absolute accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 15 | 25 | -0,77 | 0,49 | 1,35 | 1,61 | 1,61 |
| 1 | 30 | 25 | -0,78 | 0,51 | 1,36 | 1,59 | 1,59 |

SSB periodicity: 40ms

| samples | SCS [KHz] | Measurement BW [PRBs] | 5 % | 50 % | 90 % | 95 % | absolute accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 15 | 25 | -0,77 | 0,49 | 1,38 | 1,63 | 1,63 |
| 1 | 30 | 25 | -0,81 | 0,5 | 1,42 | 1,65 | 1,65 |

General observations

From the results it is clear that for an AWGN channel:

- The carrier frequency only has marginal impact on the measurement accuracy (compare to [4]).

- There is only marginal impact on the accuracy from the used SCS in an AWGN channel.

As expected, the measurement accuracy is significantly impacted by the number of available RS for measurements:

- Measurement bandwidth (for CSI-RS)

- Density (of CSI-RS) [3]

Conclusion

In Athens meeting RAN4 agreed a down selected set of assumptions for simulations [2] to facilitate determination of the CSI-RS measurement bandwidth as well as CSI-RS and SSB based L1-RSRP measurement accuracy. In this paper we have provided results based on the agreed assumption [1, 2].

References

- R4-1811426, Simulation assumption for evaluation of L1-RSRP measurement accuracy, Huawei, HiSilicon

- R4-1902565, Way forward on L1 RSRP accuracy, Ericsson, MediaTek, Nokia

- R4-1901780, Simulation results for L1-RSRP accuracy measurements, Nokia, Nokia Shanghai Bell

- R4-1903920, Updated simulation results for L1-RSRP accuracy measurements for FR2, Nokia, Nokia Shanghai Bell