3GPP TSG-RAN WG4 Meeting #90b                    	   R4-1904046

Xi’an, China, Apr 8th – 12th, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | draft | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS38.101-4: Correction to FR2 CSI test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-04-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | BWP and PTRS parameters for Rel-15 NR FR2 CSI test cases |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Modify the BWP setting: Use offsetToCarrier and RBstart - Modify the PTRS setting: Use epre-Ratio defined in 38.214 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-15 NR FR2 CSI test cases are not completed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Start of change>>

# 8	CSI reporting requirements (Radiated requirements)

## 8.1	General

This section includes radiated requirements for the reporting of channel state information (CSI).

### 8.1.1	Applicability of requirements

<TBA>

### 8.1.2	Common test parameters

Parameters specified in Table 8.1.2-1 are applied for all test cases in this section unless otherwise stated.

Table 8.1.2-1: Test parameters for CSI test cases

| Parameter |  |  |  | Unit | Value |
| --- | --- | --- | --- | --- | --- |
| PDSCH transmission scheme |  |  |  |  | Transmission scheme 1 |
| Duplex Mode |  |  |  |  | TDD |
| PTRS epre-Ratio |  |  |  |  | [0] |
| Active DL BWP index |  |  |  |  | 1 |
| Cyclic prefix |  |  |  |  | Normal |
| Common serving cell parameters | Physical Cell ID |  |  |  | 0 |
|  | SSB position in burst |  |  |  | First SSB in Slot #0 |
|  | SSB periodicity |  |  | ms | 20 |
| PDCCH configuration | Slots for PDCCH monitoring |  |  |  | Each slot |
|  | Symbols with PDCCH |  |  |  | 0,1 |
|  | Number of PDCCH candidates and aggregation levels |  |  |  | 1/[8] |
|  | DCI format |  |  |  | 1_1 |
|  | TCI state |  |  |  | TCI state #1 |
| Cross carrier scheduling |  |  |  |  | Not configured |
| PDSCH configuration | Mapping type |  |  |  | Type A |
|  | k0 |  |  |  | 0 |
|  | Starting symbol (S) |  |  |  | 2 |
|  | Length (L) |  |  |  | 12 |
|  | PDSCH aggregation factor |  |  |  | 1 |
|  | PRB bundling type |  |  |  | Static |
|  | PRB bundling size |  |  |  | 2 |
|  | Resource allocation type |  |  |  | 0 |
|  | VRB-to-PRB mapping type |  |  |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  |  |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  |  |  | Type 1 |
|  | Number of additional DMRS |  |  |  | 1 |
|  | DMRS ports indexes |  |  |  | {1000} for Rank1 {1000,1001} for Rank2 |
|  | Length |  |  |  | Single-symbol DM-RS |
|  | Number of PDSCH DMRS CDM group(s) without data |  |  |  | 2 |
| PTRS configuration | Frequency density (KPT-RS) |  |  |  | 2 |
|  | Time density (LPT-RS) |  |  |  | 1 |
| CSI-RS for tracking |  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | [0] |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | [4] |
|  |  | Number of CSI-RS ports (X) |  |  | 1 |
|  |  | CDM Type |  |  | No CDM |
|  |  | Density (ρ) |  |  | 3 |
|  |  | CSI-RS periodicity |  | slot | 120kHz SCS: 160 |
|  |  | CSI-RS offset |  | slot | 120 kHz SCS: 80 for CSI-RS resource 1 and 2 81 for CSI-RS resource 3 and 4 |
|  |  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = BWP size |
|  |  | QCL info |  |  | TCI state #0 |
| NZP CSI-RS for CSI acquisition |  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = BWP size |
|  |  | QCL info |  |  | TCI state #1 |
| ZP CSI-RS for CSI acquisition |  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = BWP size |
| CSI-RS for beam refinement |  | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resource 1,2 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 8 for CSI-RS resource 1 l0 = 9 for CSI-RS resource 2 |
|  |  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2 |
|  |  | CDM Type |  |  | ‘No CDM’ for CSI-RS resource 1,2 |
|  |  | Density (ρ) |  |  | 3 for CSI-RS resource 1,2 |
|  |  | CSI-RS periodicity |  | Slots | 120 kHz SCS: 160 for CSI-RS resource 1,2 |
|  |  | CSI-RS offset |  | Slots | 0 for CSI-RS resource 1,2 |
|  |  | Repetition |  |  | ON |
| TCI state #0 |  | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  |  | QCL Type |  | Type C |
|  |  | Type 2 QCL information | SSB index |  | SSB #0 |
|  |  |  | QCL Type |  | Type D |
| TCI state #1 |  | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from ‘CSI-RS for tracking’ configuration |
|  |  |  | QCL Type |  | Type A |
|  |  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 1 from ‘CSI-RS for tracking’ configuration |
|  |  |  | QCL Type |  | Type D |
| Number of HARQ Processes |  |  |  |  | 8 |
| HARQ ACK/NACK bundling |  |  |  |  | Multiplexed |
| Redundancy version coding sequence |  |  |  |  | {0,2,3,1} |
| K1 value (PDSCH-to-HARQ-timing-indicator) |  |  |  |  | For FR2.120-1: [3] if mod (i.5) = 0, [6] if mod(i,5) = 2 For FR2.120-2: [11] if mod(i,8) = 0, [7] if mod(i,8) = 4, [6] if mod(i,8) = 5, where i is slot index per radio fame with values 0-79. |
| Symbols for unused Res |  |  |  |  | OCNG as specified in A.5 |
| Note 1: PDSCH is not scheduled on slots containing CSI-RS or slots which are not full DL. Note 2: UE assumes that the TCI state for the PDSCH is identical to the TCI state applied for the PDCCH transmission. |  |  |  |  |  |

## 8.2	Reporting of Channel Quality Indicator (CQI)

### 8.2.1	1RX requirements

(Void)

### 8.2.2	2RX requirements

#### 8.2.2.1	FDD

(Void)

#### 8.2.2.2	TDD

##### 8.2.2.2.1	CQI reporting under AWGN conditions

<Editor’s note: The requirements were introduced based on current results from companies; these requirements can be revised based on more results from companies.>

The reporting accuracy of the channel quality indicator (CQI) under frequency non-selective conditions is determined by the reporting variance and the BLER performance using the transport format indicated by the reported CQI median. The purpose is to verify that the reported CQI values are in accordance with the CQI definition given in TS 38.214 [12]. To account for sensitivity of the input SNR the reporting definition is considered to be verified if the reporting accuracy is met for at least one of two SNR levels separated by an offset of [1] dB.

###### 8.2.2.2.1.1	Minimum requirement for periodic CQI reporting

For the parameters specified in Table 8.2.2.2.1.1-1, and using the downlink physical channels specified in Annex C.5.1, the minimum requirements are specified by the following:

a)	the reported CQI value shall be in the range of ±1 of the reported median more than [90%] of the time;

b)	if the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, the BLER using the transport format indicated by the (median CQI + 1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, the BLER using transport format indicated by (median CQI – 1) shall be less than or equal to 0.1.

Table 8.2.2.2.1.1-1 Test parameters

| Parameter |  |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  |  | MHz | 100 |  | 100 |  |
| Duplex Mode |  |  |  |  | TDD |  | TDD |  |
| TDD Slot Configuration |  |  |  |  | FR2.120-2 [Annex A.1.3] |  | FR2.120-2 [Annex A.1.3] |  |
| DL BWP configuration #1 |  |  | Ocarrier (offsetToCarrier) |  | 0 |  | 0 |  |
|  |  |  | RBstart |  | 0 |  | 0 |  |
|  |  |  | Number of contiguous PRB |  | 66 |  | 66 |  |
|  |  |  | Subcarrier spacing | kHz | 120 |  | 120 |  |
| SNRBB |  |  |  | dB | [8] | [9] | [14] | [15] |
| Propagation channel |  |  |  |  | AWGN |  | AWGN |  |
| Antenna configuration |  |  |  |  | 2×2 with static channel specified in [Annex B.1] |  | 2×2 with static channel specified in [Annex B.1] |  |
| Beamforming Model |  |  |  |  | As specified in Section [Annex TBD] |  | As specified in Section [Annex TBD] |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  |  | Periodic |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  |  |  | 4 |  | 4 |  |
|  | CDM Type |  |  |  | FD-CDM2 |  | FD-CDM2 |  |
|  | Density (ρ) |  |  |  | 1 |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | 8 |  | 8 |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  |  | 13 |  | 13 |  |
|  | CSI-RS periodicity and offset |  |  | slot | 8/1 |  | 8/1 |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  |  | Periodic |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  |  |  | 2 |  | 2 |  |
|  | CDM Type |  |  |  | fd-CDM2 |  | fd-CDM2 |  |
|  | Density (ρ) |  |  |  | 1 |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | 6 |  | 6 |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  |  | 13 |  | 13 |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  |  | slot | 8/1 |  | 8/1 |  |
| CSI-IM configuration | CSI-IM RE pattern |  |  |  | 1 |  | 1 |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  |  | (8, 13) |  | (8, 13) |  |
|  | CSI-IM timeConfig periodicity and offset |  |  | slot | 8/1 |  | 8/1 |  |
| ReportConfigType |  |  |  |  | Periodic |  | Periodic |  |
| CQI-table |  |  |  |  | Table 1 |  | Table 1 |  |
| reportQuantity |  |  |  |  | cri-RI-PMI-CQI |  | cri-RI-PMI-CQI |  |
| timeRestrictionForChannelMeasurements |  |  |  |  | Not configured |  | Not configured |  |
| timeRestrictionForInterferenceMeasurements |  |  |  |  | Not configured |  | Not configured |  |
| cqi-FormatIndicator |  |  |  |  | Wideband |  | Wideband |  |
| pmi-FormatIndicator |  |  |  |  | Wideband |  | Wideband |  |
| Sub-band Size |  |  |  | RB | [8] |  | [8] |  |
| csi-ReportingBand |  |  |  |  | [111111111] |  | [111111111] |  |
| CSI-Report periodicity and offset |  |  |  | slot | 8/1 |  | 8/1 |  |
| aperiodicTriggeringOffset |  |  |  |  | Not configured |  | Not configured |  |
| Codebook configuration |  | Codebook Type |  |  | typeI-SinglePanel |  | typeI-SinglePanel |  |
|  |  | Codebook Mode |  |  | 1 |  | 1 |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  |  | Not configured |  | Not configured |  |
|  |  | CodebookSubsetRestriction |  |  | [010000] |  | [010000] |  |
|  |  | RI Restriction |  |  | [N/A] |  | [N/A] |  |
| Physical channel for CSI report |  |  |  |  | [PUCCH] |  | [PUCCH] |  |
| CQI/RI/PMI delay |  |  |  | ms | [8.375] |  | [8.375] |  |
| Maximum number of HARQ transmission |  |  |  |  | 1 |  | 1 |  |
| Measurement channel |  |  |  |  | As specified in Table A.4-1, TBS.1-2 |  | As specified in Table A.4-1, TBS.1-2 |  |

##### 8.2.2.2.2	CQI reporting under fading conditions

###### 8.2.2.2.2.1	Minimum requirement for wideband CQI reporting

<Editor’s note: Open issues to be resolved:

-	SNR levels

-	Test parameters

-	Requirements values (BLER, α, γ)>

The purpose of the requirements is to verify that the UE is tracking the channel variations and selecting the largest transport format possible according to the prevailing channel state for the frequency non-selective scheduling.

The reporting accuracy of CQI under frequency non-selective fading conditions is determined by the reporting variance, the relative increase of the throughput obtained when the transport format is indicated by the reported CQI compared to the throughput obtained when a fixed transport format is configured according to the reported median CQI, and a minimum BLER using the transport formats indicated by the reported CQI. To account for sensitivity of the input SNR the CQI reporting under frequency non-selective fading conditions is considered to be verified if the reporting accuracy is met for at least one of two SNR levels separated by an offset of [1] dB.

For the parameters specified in Table 8.2.2.2.2.1-1 and using the downlink physical channels specified in Annex C.5.1, the minimum requirements are specified by the following:

a)	a CQI index not in the set {median CQI -1, median CQI, median CQI +1} shall be reported at least α % of the time, where α% is specified in Table 8.2.2.2.2.1-2;

b)	the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index and that obtained when transmitting a fixed transport format configured according to the wideband CQI median shall be ≥ γ, where γ is specified in Table 8.2.2.2.2.1-2;

c)	when transmitting the transport format indicated by each reported wideband CQI index, the average BLER for the indicated transport formats shall be greater or equal to [0.01].

Table 8.2.2.2.2.1-1 Test parameters

| Parameter |  |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  |  | MHz | 100 |  | 100 |  |
| Duplex Mode |  |  |  |  | TDD |  | TDD |  |
| TDD Slot Configuration |  |  |  |  | FR2.120-2 [Annex A.1.3] |  | FR2.120-2 [Annex A.1.3] |  |
| DL BWP configuration #1 |  |  | Ocarrier (offsetToCarrier) |  | 0 |  | 0 |  |
|  |  |  | RBstart |  | 0 |  | 0 |  |
|  |  |  | Number of contiguous PRB |  | 66 |  | 66 |  |
|  |  |  | Subcarrier spacing | kHz | 120 |  | 120 |  |
| SNRBB |  |  |  | dB | 6 | 7 | 12 | 13 |
| Propagation channel |  |  |  |  | [TDLA30-35] |  | [TDLA30-35] |  |
| Antenna configuration |  |  |  |  | 2×2 [ULA High] |  | 2×2 [ULA High] |  |
| Beamforming Model |  |  |  |  | As specified in Section [Annex TBD] |  | As specified in Section [Annex TBD] |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  |  | Aperiodic |  | Aperiodic |  |
|  | Number of CSI-RS ports (X) |  |  |  | 4 |  | 4 |  |
|  | CDM Type |  |  |  | FD-CDM2 |  | FD-CDM2 |  |
|  | Density (ρ) |  |  |  | 1 |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | 8 |  | 8 |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  |  | 13 |  | 13 |  |
|  | CSI-RS interval and offset |  |  | slot | [8/1] |  | [8/1] |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  |  | Aperiodic |  | Aperiodic |  |
|  | Number of CSI-RS ports (X) |  |  |  | 2 |  | 2 |  |
|  | CDM Type |  |  |  | fd-CDM2 |  | fd-CDM2 |  |
|  | Density (ρ) |  |  |  | 1 |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | 6 |  | 6 |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  |  | 13 |  | 13 |  |
|  | NZP CSI-RS-timeConfig interval and offset |  |  | slot | [8/1] |  | [8/1] |  |
| CSI-IM configuration | CSI-IM RE pattern |  |  |  | 1 |  | 1 |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  |  | (8, 13) |  | (8, 13) |  |
|  | CSI-IM timeConfig interval and offset |  |  | slot | [8/1] |  | [8/1] |  |
| ReportConfigType |  |  |  |  | Aperiodic |  | Aperiodic |  |
| CQI-table |  |  |  |  | Table 1 |  | Table 1 |  |
| reportQuantity |  |  |  |  | cri-RI-PMI-CQI |  | cri-RI-PMI-CQI |  |
| timeRestrictionForChannelMeasurements |  |  |  |  | Not configured |  | Not configured |  |
| timeRestrictionForInterferenceMeasurements |  |  |  |  | Not configured |  | Not configured |  |
| cqi-FormatIndicator |  |  |  |  | Wideband |  | Wideband |  |
| pmi-FormatIndicator |  |  |  |  | Wideband |  | Wideband |  |
| Sub-band Size |  |  |  | RB | [8] |  | [8] |  |
| csi-ReportingBand |  |  |  |  | [111111111] |  | [111111111] |  |
| CSI-Report periodicity and offset |  |  |  | slot | 8/1 |  | 8/1 |  |
| aperiodicTriggeringOffset |  |  |  |  | Not configured |  | Not configured |  |
| Codebook configuration |  | Codebook Type |  |  | typeI-SinglePanel |  | typeI-SinglePanel |  |
|  |  | Codebook Mode |  |  | 1 |  | 1 |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  |  | Not configured |  | Not configured |  |
|  |  | CodebookSubsetRestriction |  |  | [000001] |  | [000001] |  |
|  |  | RI Restriction |  |  | [N/A] |  | [N/A] |  |
| Physical channel for CSI report |  |  |  |  | [PUSCH] |  | [PUSCH] |  |
| CQI/RI/PMI delay |  |  |  | ms | [1.375] |  | [1.375] |  |
| Maximum number of HARQ transmission |  |  |  |  | 1 |  | 1 |  |
| Measurement channel |  |  |  |  | As specified in Table A.4-1, TBS.1-1 |  | As specified in Table A.4-1, TBS.1-1 |  |

Table 8.2.2.2.2.1-2 Minimum requirements

|  | Test 1 | Test 2 |
| --- | --- | --- |
|  [%] | [2] | [2] |
|  | [1.05] | [1.05] |

## 8.3	Reporting of Precoding Matrix Indicator (PMI)

The minimum performance requirements of PMI reporting are defined based on the precoding gain, expressed as the relative increase in throughput when the transmitter is configured according to the UE reports compared to the case when the transmitter is using random precoding, respectively. When the transmitter uses random precoding, for each PDSCH allocation a precoder is randomly generated and applied to the PDSCH. A fixed transport format (FRC) is configured for all requirements.

The requirements for transmission mode 1 with 2TX and higher layer parameter codebookType set to ‘typeI-SinglePanel’ are specified in terms of the ratio

In the definition of γ, for 2TX PMI requirements, is [90] % of the maximum throughput obtained at  using the precoders configured according to the UE reports, and is the throughput measured at with random precoding.

### 8.3.1	1RX requirements

(Void)

### 8.3.2	2RX requirements

#### 8.3.2.1	FDD

(Void)

#### 8.3.2.2	TDD

##### 8.3.2.2.1	Single PMI with 2TX TypeI-SinglePanel Codebook

For the parameters specified in Table 8.3.2.2.1-1, and using the downlink physical channels specified in Annex C.5.1, the minimum requirements are specified in Table 8.3.2.2.1-2.

Table 8.3.2.2.1-1: Test parameters (single layer)

| Parameter |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 100 | 100 |
| TDD DL-UL configuration |  |  | FR2.120-2 as specified in Annex A | FR2.120-2 as specified in Annex A |
| DL BWP configuration #1 | Ocarrier (offsetToCarrier) |  | 0 | 0 |
|  | RBstart |  | 0 | 0 |
|  | Number of contiguous PRB |  | 66 | 66 |
|  | Subcarrier spacing | kHz | 120 | 120 |
| Propagation channel |  |  | [TDLA30-35] |  |
| Antenna configuration |  |  | 2 x 2 [ULA Low] | 2 x 2 [ULA Low] |
| Beamforming Model |  |  | TBD |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 | 4 |
|  | CDM Type |  | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 4, (8,-) | Row 4, (8,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (13,-) | (13,-) |
|  | CSI-RS interval and offset | slot | 8/1 | 5/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 2 | 2 |
|  | CDM Type |  | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 3, (6,-) | Row 3, (6,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (13,-) | (13,-) |
|  | CSI-RS interval and offset | slot | 8/1 | 8/1 |
| CSI-IM configuration | CSI-IM RE pattern |  | Patten 0 | Patten 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (8,13) | (8,13) |
|  | CSI-IM timeConfig interval and offset | slot | 8/1 | 5/1 |
| ReportConfigType |  |  | Aperiodic | Aperiodic |
| CQI-table |  |  | Table 1 | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | Not configured | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured | Not configured |
| cqi-FormatIndicator |  |  | Wideband | Wideband |
| pmi-FormatIndicator |  |  | Wideband | Wideband |
| Sub-band Size |  | RB | [8] | [8] |
| csi-ReportingBand |  |  | [111111111] | [111111111] |
| CSI-Report interval and offset |  | slot | 8/1 | 5/1 |
| aperiodicTriggeringOffset |  |  | 0 | 0 |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel | typeI-SinglePanel |
|  | Codebook Mode |  | 1 | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | N/A | N/A |
|  | CodebookSubsetRestriction |  | 001111 | 001111 |
|  | RI Restriction |  | N/A | N/A |
| Physical channel for CSI report |  |  | PUSCH | PUSCH |
| CQI/RI/PMI delay |  | ms | 1.375 | 1.75 |
| Maximum number of HARQ transmission |  |  | 4 | 4 |
| Measurement channel |  |  | R.PDSCH.5-8.1 TDD | R.PDSCH.5-7.1 TDD |
| Note 1: For random precoder selection, the precoder shall be updated in each slot (0.125 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#[(n-4)], this reported PMI cannot be applied at the eNB downlink before slot#[(n+4)]. Note 3: Randomization of the principle beam direction shall be used as specified in TBD. |  |  |  |  |

Table 8.3.2.2.1-2: Minimum requirement

| Parameter | Test 1 | Test 2 |
| --- | --- | --- |
|  | [1.05] | [1.05] |

## 8.4	Reporting of Rank Indicator (RI)

The purpose of this test is to verify that the reported rank indicator accurately represents the channel rank. The accuracy of RI reporting is determined by the relative increase of the throughput obtained when transmitting based on the reported rank compared to the case for which a fixed rank is used for transmission.

### 8.4.1	1RX requirements

(Void)

### 8.4.2	2RX requirements

#### 8.4.2.1	FDD

(Void)

#### 8.4.2.2	TDD

The minimum performance requirement in Table 8.4.2.2-2 is defined as

a)	The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 1 shall be ≥ ;

b)	The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 2 shall be ≥ ;

For the parameters specified in Table 8.4.2.2-1, and using the downlink physical channels specified in Annex C.5.1, the minimum requirements are specified in Table 8.4.2.2-2.

Table 8.4.2.2-1: RI Test (TDD)

| Parameter |  |  |  | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  |  | MHz | 100 | 100 | 100 |
| Duplex Mode |  |  |  |  | TDD | TDD | TDD |
| TDD Slot Configuration |  |  |  |  | FR1.120-2 | FR1.120-2 | FR1.120-2 |
| DL BWP configuration #1 |  |  | Ocarrier (offsetToCarrier) |  | 0 | 0 | 0 |
|  |  |  | RBstart |  | 0 | 0 | 0 |
|  |  |  | Number of contiguous PRB |  | 66 | 66 | 66 |
|  |  |  | Subcarrier spacing | kHz | 120 | 120 | 120 |
| SNR |  |  |  | dB | [0] | [20] | [20] |
| Propagation channel |  |  |  |  | [TDLA30-35] | [TDLA30-35] | [TDLA30-35] |
| Antenna configuration |  |  |  |  | ULA Low 2x2 | ULA Low 2x2 | XP High 2x2 |
| Beamforming Model |  |  |  |  | As defined in Annex B.4.1 | As defined in Annex B.4.1 | As defined in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  |  | Aperiodic | Aperiodic | Aperiodic |
|  | Number of CSI-RS ports (X) |  |  |  | 4 | 4 | 4 |
|  | CDM Type |  |  |  | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  |  |  | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | Row 4, (8,-) | Row 4, (8,-) | Row 4, (8,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  |  | (13,-) | (13,-) | (13,-) |
|  | CSI-RS interval and offset |  |  | slot | 8/1 | 8/1 | 8/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  |  | Aperiodic | Aperiodic | Aperiodic |
|  | Number of CSI-RS ports (X) |  |  |  | 2 | 2 | 2 |
|  | CDM Type |  |  |  | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  |  |  | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | Row 3 (6,-) | Row 3 (6,-) | Row 3 (6,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  |  | (13,-) | (13,-) | (13,-) |
|  | NZP CSI-RS-timeConfig interval and offset |  |  | slot | 8/1 | 8/1 | 8/1 |
| CSI-IM configuration | CSI-IM RE pattern |  |  |  | Pattern 1 | Pattern 1 | Pattern 1 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  |  | (8,13) | (8,13) | (8,13) |
|  | CSI-IM timeConfig interval and offset |  |  | slot | 8/1 | 8/1 | 8/1 |
| ReportConfigType |  |  |  |  | Aperiodic | Aperiodic | Aperiodic |
| CQI-table |  |  |  |  | Table 1 | Table 1 | Table 1 |
| reportQuantity |  |  |  |  | cri-RI-PMI-CQI | cri-RI-PMI-CQI | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  |  |  | not configured | not configured | not configured |
| timeRestrictionForInterferenceMeasurements |  |  |  |  | not configured | not configured | not configured |
| cqi-FormatIndicator |  |  |  |  | Wideband | Wideband | Wideband |
| pmi-FormatIndicator |  |  |  |  | Wideband | Wideband | Wideband |
| Sub-band Size |  |  |  | RB | [8] | [8] | [8] |
| csi-ReportingBand |  |  |  |  | [111111111] | [111111111] | [111111111] |
| CSI-Report interval and offset |  |  |  | slot | 8/1 | 8/1 | 8/1 |
| aperiodicTriggeringOffset |  |  |  |  | 0 | 0 | 0 |
| Codebook configuration |  | Codebook Type |  |  | typeI-SinglePanel | typeI-SinglePanel | typeI-SinglePanel |
|  |  | Codebook Mode |  |  | 1 | 1 | 1 |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  |  | N/A | N/A | N/A |
|  |  | CodebookSubsetRestriction |  |  | [010000 for fixed rank 2, 010011 for following rank] | [000011 for fixed rank 1, 010011 for following rank] | [000011 for fixed rank 1, 010011 for following rank] |
|  |  | RI Restriction |  |  | N/A | N/A | N/A |
| Physical channel for CSI report |  |  |  |  | PUSCH | PUSCH | PUSCH |
| CQI/RI/PMI delay |  |  |  | ms | 1.375 | 1.375 | 1.375 |
| Maximum number of HARQ transmission |  |  |  |  | 1 | 1 | 1 |
| RI Configuration |  |  |  |  | Fixed RI = 2 and follow RI | Fixed RI = 1 and follow RI | Fixed RI = 1 and follow RI |

Table 8.4.2.2-2: Minimum requirement (TDD)

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
| 1 | N/A | [1.05] | [1.05] |
| 2 | [1.0] | N/A | N/A |

<< End of change>>