3GPP TSG-RAN WG4 Meeting #90bis	R4-1904150

Xi’an, China, 8 - 12 April, 2019

Title: 					Link level simulation results for SS-RSRP accuracy with cross correlation

Source: 	Ericsson

Agenda item:	6.11.2

Document for:	Discussion

# Introduction

In [1] there was discussion for SS-RSRP accuracy based on code pairs where there is a potential cross-correlation between SSS sequences due to the chosen PCI. In this paper, we provide our results based on link level studies of the same issue

# Discussion

Simulation assumptions are provided in tables 1 and 2. The simulation assumptions are a superset of those used in the study in [1]. Additionally, we have studied Es/Iot=-3dB since there is an accuracy breakpoint at -3dB, and we have studied two different coherent combining schemes. Differences from the earlier work are highlighted in table 1 and table 2.

Table 1: Simulation parameters for SS-RSRP measurement

| Parameters | Value |  |
| --- | --- | --- |
|  | Sub-6GHz | Above 6GHz |
| Carrier frequency | 4GHz | 30GHz |
| Subcarrier Spacing for SS block | 15KHz, 30KHz | 120KHz, 240KHz |
| Bandwidth of SS block | 20 RBs |  |
| Number of transmitted SS block within a SS burst set period (K) | K=1 |  |
| Reference signals | NR-SSS |  |
| Transmission period of SS block | 40ms; |  |
| Number of samples in L1 evaluation period | 5 samples |  |
| Transmit antennas | 1 |  |
| Receive antennas | 2 |  |
| Propagation conditions | AWGN |  |
| CP type | Normal CP |  |
| Coherent combining bandwidth | 2PRB, 4PRB |  |

Table 2: Cell-specific parameters for SS-RSRP measurement

| Parameters |  | Cell 1 (interfering cell) | Cell 2 (target cell) |
| --- | --- | --- | --- |
| RF Channel number |  | Channel 1 | Channel 1 |
| Cell ID |  | Case 1: Cell ID = 184 Case 2: Cell ID = 550 | Case 1: Cell ID = 0 Case 2: Cell ID = 0 |
| PBCH and DMRS power offset with respect to NR-PSS and NR-SSS |  | 0 | 0 |
| Data and control PSD relative to NR-PSS and NR-SSS |  | 0 | 0 |
| RB Utilization |  | 100% | 100% |
| Data Modulation |  | QPSK | QPSK |
| Frequency Offset relative to UE frequency reference |  | 0 | 0 |
| Relative Delay of 1st Path (synchronous) |  | CP/2 | 0 |
| Es/Ioc | Setting 1 | 10 dB | 4.5 dB |
|  | Setting 2 | 6 dB | 1 dB |
|  | Setting 3 | 3 dB | -1 dB |
|  | Setting 4 | 7 dB | 4.5 dB |
|  | Setting 5 | 3 dB | 1 dB |
|  | Setting 6 | 0dB | -1 dB |
| Es/Iot | Setting 1 | N/A | -6 dB |
|  | Setting 2 | N/A | -6 dB |
|  | Setting 3 | N/A | -6 dB |
|  | Setting 4 | N/A | -3 dB |
|  | Setting 5 | N/A | -3 dB |
|  | Setting 6 | N/A | -3 dB |

# Evaluation results

Since the results in AWGN are numerically very similar for different SCS, we tabulate results for 15kHz. Full results for all the simulated SCS are shown in annex A.

## Evaluation Results, 2PRB coherent combining

| Case | SINR | 5% | 10% | 50% | 90% | 95% |
| --- | --- | --- | --- | --- | --- | --- |
| #1 | Setting 1(Es/Iot=-6dB) | -2.06 | -2.00 | -1.79 | -1.58 | -1.52 |
|  | Setting 2(Es/Iot=-6dB) | -2.06 | -1.98 | -1.63 | -1.39 | -1.30 |
|  | Setting 3(Es/Iot=-6dB) | -1.96 | -1.87 | -1.49 | -1.11 | -1.04 |
|  | Setting 4(Es/Iot=-3dB) | -1.56 | -1.52 | -1.33 | -1.15 | -1.08 |
|  | Setting 5(Es/Iot=-3dB) | -1.59 | -1.50 | -1.25 | -0.97 | -0.92 |
|  | Setting 6(Es/Iot=-3dB) | -1.52 | -1.42 | -1.09 | -0.77 | -0.67 |
| #2 | Setting 1(Es/Iot=-6dB) | 0.84 | 0.88 | 1.04 | 1.20 | 1.24 |
|  | Setting 2(Es/Iot=-6dB) | 0.66 | 0.73 | 0.96 | 1.20 | 1.27 |
|  | Setting 3(Es/Iot=-6dB) | 0.43 | 0.53 | 0.85 | 1.14 | 1.24 |
|  | Setting 4(Es/Iot=-3dB) | 0.49 | 0.53 | 0.69 | 0.82 | 0.86 |
|  | Setting 5(Es/Iot=-3dB) | 0.36 | 0.41 | 0.62 | 0.84 | 0.91 |
|  | Setting 6(Es/Iot=-3dB) | 0.15 | 0.26 | 0.57 | 0.84 | 0.93 |

## Evaluation Results, Es/Iot=-6dB, 4PRB coherent combining

| Case | SINR | 5% | 10% | 50% | 90% | 95% |
| --- | --- | --- | --- | --- | --- | --- |
| #1 | Setting 1(Es/Iot=-6dB) | -1.34 | -1.27 | -1.07 | -0.86 | -0.81 |
|  | Setting 2(Es/Iot=-6dB) | -1.38 | -1.31 | -1.00 | -0.71 | -0.65 |
|  | Setting 3(Es/Iot=-6dB) | -1.45 | -1.32 | -0.96 | -0.64 | -0.57 |
|  | Setting 4(Es/Iot=-3dB) | -1.05 | -1.00 | -0.81 | -0.64 | -0.59 |
|  | Setting 5(Es/Iot=-3dB) | -1.11 | -1.06 | -0.77 | -0.50 | -0.40 |
|  | Setting 6(Es/Iot=-3dB) | -1.11 | -0.99 | -0.68 | -0.37 | -0.25 |
| #2 | Setting 1(Es/Iot=-6dB) | -1.32 | -1.30 | -1.12 | -0.91 | -0.87 |
|  | Setting 2(Es/Iot=-6dB) | -1.35 | -1.25 | -1.01 | -0.75 | -0.64 |
|  | Setting 3(Es/Iot=-6dB) | -1.37 | -1.28 | -0.89 | -0.54 | -0.43 |
|  | Setting 4(Es/Iot=-3dB) | -0.98 | -0.92 | -0.70 | -0.52 | -0.45 |
|  | Setting 5(Es/Iot=-3dB) | -1.05 | -0.96 | -0.66 | -0.38 | -0.34 |
|  | Setting 6(Es/Iot=-3dB) | -1.04 | -0.96 | -0.60 | -0.24 | -0.16 |

# Observations

In this section we provide our observations on the simulation results.

Baseline results with 2 PRB coherent averaging, PCI case 1 (interfering ID=184)

Observation 1: With 2 PRB coherent averaging and -6dB Es/Iot, we observe that when PCI case 1 is used, the measured cell has around -2dB bias at the 5th percentile. The bias is relatively independent of Es/Ioc at least at the 5th percentile for all the Es/Ioc studied (in all cases the interferer is dominant).

Observation 2: With 2 PRB coherent averaging and -3dB Es/Iot, we observe that when PCI case 1 is used, the measured cell has around -1.5dB bias at the 5th percentile. The bias is relatively independent of Es/Ioc at least at the 5th percentile for all the Es/Ioc studied (in all cases the interferer is dominant).

Baseline results with 2 PRB coherent averaging, PCI case 2 (interfering ID=550)

Observation 3: With 2 PRB coherent averaging and -6dB Es/Iot, we observe that when PCI case 2 is used, the measured cell has around  +1.25dB bias at the 95th percentile in the worst case. The bias improves as Es/Ioc reduces at least at the 95th percentile.

Observation 4: With 2 PRB coherent averaging and -3dB Es/Iot, we observe that when PCI case 2 is used, the measured cell has around +0.9dB bias in the worst case at the 95th percentile.

Observations 1 and 3 show the same general trend as the results provided in [1], although the results in this contribution show a reduced bias for both case 1 and case 2 compared with the results in [1].

Observation 5: SS-RSRP of ±2.5dB is met for all cases at Es/Iot=-6dB with 2PRB coherent averaging, although the margin for meeting the requirement is reduced for case 1 compared with case 2

Results with 4 PRB coherent averaging, PCI case 1 and 2

Observation 6: With 4PRB coherent averaging, there is little difference between any SS-RSRP CDF curve for case 1 or case 2

In a companion contribution we discuss proposals based on the observations.

# References

- R4-1901210, “Further discussion on SSB based measurement accuracy”, Huawei, Hisilicon

# Annex A, results

## -6dB Es/Iot, 2 PRB coherent combining

Figures 1a-1d show the results for 15kHz, 30kHz, 120 and 240kHz SCS respectively

Figure 1a: -6dB Es/Iot, 2PRB coherent combining, 15kHz SCS

Figure 1b: -6dB Es/Iot, 2PRB coherent combining, 30kHz SCS

Figure 1c: -6dB Es/Iot, 2PRB coherent combining, 120kHz SCS

Figure 1d: -6dB Es/Iot, 2PRB coherent combining, 240kHz SCS

## -3dB Es/Iot, 2 PRB coherent combining

Figures 2a-2d show the results for 15kHz, 30kHz, 120 and 240kHz SCS respectively

Figure 2a: -3dB Es/Iot, 2PRB coherent combining, 15kHz SCS

Figure 2b: -3dB Es/Iot, 2PRB coherent combining, 30kHz SCS

Figure 2c: -3dB Es/Iot, 2PRB coherent combining, 120kHz SCS

Figure 2d: -3dB Es/Iot, 2PRB coherent combining, 240kHz SCS

## -6dB Es/Iot, 4 PRB coherent combining

Figures 3a-3d show the results for 15kHz, 30kHz, 120 and 240kHz SCS respectively

Figure 3a: -6dB Es/Iot, 4PRB coherent combining, 15kHz SCS

Figure 3b: -6dB Es/Iot, 4PRB coherent combining, 30kHz SCS

Figure 3c: -6dB Es/Iot, 4PRB coherent combining, 120kHz SCS

Figure 3d: -6dB Es/Iot, 4PRB coherent combining, 240kHz SCS

## -3dB Es/Iot, 4 PRB coherent combining

Figures 4a-4d show the results for 15kHz, 30kHz, 120 and 240kHz SCS respectively

Figure 4a: -3dB Es/Iot, 4PRB coherent combining, 15kHz SCS

Figure 4b: -3dB Es/Iot, 4PRB coherent combining, 30kHz SCS

Figure 4c: -3dB Es/Iot, 4PRB coherent combining, 120kHz SCS

Figure 4d: -3dB Es/Iot, 4PRB coherent combining, 240kHz SCS