3GPP TSG-RAN WG4 Meeting #90bis		R4-192xxxx
Xi'an, China, 8 - 12 April 2019

Agenda Item:	10.2.3.1

Source:	Keysight

Title:	Impact of 2D vs. 3D channel models on spatial correlation

Document for:	Approval

# 1	Introduction

Geometric channel models are mandatory for NR MIMO OTA testing as they contain also angular and polarimetric information of the modelled radio channel. Clustered delay line (CDL) fading channel models specified in [1] are a good candidate for NR MIMO OTA. In this paper, we investigate 3D power distributions, i.e. of azimuth and elevation angles, in a few CDL models for FR1 UE testing.

There has been discussion on multi-probe anechoic chamber (MPAC) based OTA setups for FR1. One of the questions is if MPAC configurations for LTE UE testing can be used as such for FR1. The discussion whether a 2D or 3D probe configuration is needed in FR1 multi-probe anechoic chamber (MPAC) based OTA setup was initiated in [2]. The purpose of this paper is to continue the subject by investigating the impact of 2D vs. 3D models (and MPAC configurations) on the spatial correlation function at the DUT antenna aperture.

# 2	About channel models

## 2.1 Propagation models

We have chosen three non-line-of-sight (NLOS) CDL models from Section 7.7.1 of [1], namely CDL-A, B, and C. Model parameter, both delay and angular, are scaled to match parameterization of the baseline model. For the simulations of this paper we chose the target baseline scenario UMi NLOS and the target frequency 3.5GHz. The CDL model parameters were scaled with target spreads based on these assumptions. Like the delay spread scaling, the angular scaling is an important operation for adjusting CDL models to an environment & frequency combination.

The selected CDL channel models are inherently three dimensional, as any geometric channel models of [1]. Thus, a reduction of the models to 2D representation would be an additional simplification step to the original 3D models. The case was different when OTA fading testing with MPAC was specified for LTE. At that time the original channel models were 2D not 3D.

Line-of-sight (LOS) scenarios are not discussed in this paper. Not because they would not be important, but because they are simple and straightforward in the power angular distribution point of view. While the current priority is to focus on NLOS scenarios [3], LOS should be considered in the future.

## 2.2 BS antenna model

BS antenna elements are as described in [2] (originally specified in TR 38.803). Also, BS beams are defined in [2]. In the simulation the power angular spectrum (PAS) observable by the UE assumes the two best BS beams for the particular propagation channel are active. Define Pb1Ω,Pb2Ω, Pb3Ω,Pb4Ω are the power patterns of the strongest, second, third, and fourth strongest beams, respectively. The space angle is denoted by Ω and the beam patterns are specified in linear power units. Now the beam shapes of one, two, and four simultaneous beams in decibel units are

PoneΩ=10log10Pb1Ω dB,

PtwoΩ=10log10Pb1Ω+Pb2Ω dB,

PfourΩ=10log10Pb1Ω+Pb2Ω+Pb3Ω+Pb4Ω dB.

# 3	Simulation results

In this section we visualize full 3D and marginal 2D power angular spectra of three CDL models. We also show the spatial correlation functions resulting from 3D and 2D PASs. Two strongest BS beams (PtwoΩ) are considered in the simulations, i.e., the baseline for FR1 NR MIMO [3]. This means that the BS antenna array does not illuminate all clusters of the propagation model equally, but the beam patterns for two strongest beams are summed up and the sum pattern is spatially filtering the cluster powers (see 2.2).

The 3D PAS, i.e., the PAS as a function of azimuth and elevation angles of arrival, is illustrated in Figure 1 (left) for the CDL-A model. The corresponding 2D marginal PAS is in Figure 1 (right). The 2D PAS is obtained by integrating the power over elevation dimension, summing up in the vertical domain, such that all power is in the horizon and can be represented as a function of azimuth angle only. For CDL-B and C models the 3D and 2D PASs are depicted in Figure 4 and Figure 7, respectively. We can observe that with CDL-A the power is mainly condensed to a single cluster, while with CDL-B and C there are three or two significant clusters, respectively.

The two-dimensional spatial correlation function is illustrated in Figure 2 (left) for the 3D PAS of CDL-A model. The correlation function indicates fading correlation between origin of the observation point, e.g., centre of quiet zone, and points with vertical/horizontal displacement from the origin. The unit of displacement is wavelength . The depicted correlation is the absolute value of corresponding complex correlation coefficient. The figure indicates that the correlation decreases almost monotonically as the distance between observation points increases either in the horizontal or vertical direction. This is very well expected behaviour of the correlation function.

The corresponding two-dimensional spatial correlation function, but now of the 2D (marginal) CDL-A PAS is shown in Figure 2 (right). Now the correlation declines similarly to the 3D case on a horizontal displacement. However, the vertical displacement does not affect the correlation. Instead, the correlation remains unity with any vertical distances up to four wavelengths. The impact on MIMO performance is significant for this simplification. For example, if the DUT has an antenna separation of 30 cm at 7.125 GHz, i.e., a separation in wavelengths  7.1 (in case of an antenna separation of 15 cm, this is approximately 3.5 wavelengths). In the horizontal direction (“landscape mode”), the correlation is very low (<0.1) in both 3D and 2D cases. In the vertical direction (“portrait mode”), the correlation is low (0.25) at 7.1, (see 3D case), but in the 2D case it is still unity. If the fading correlation grows from 0.25 to close to unity, the MIMO performance will decline dramatically. Even if DUTs are tested with different orientations some UE models with a particular antenna placement within the form factor may be unfavoured by a 2D PAS only while others UE models may be lucky and not to suffer the lack of power spreading in elevation in the OTA setup. In the “white box” testing this can be considered, but in the “black box” testing not.

The mentioned correlation values can be read from the one-dimensional spatial correlation functions for horizontal and vertical displacements that are plotted for the 3D and 2D (marginal) PASs in Figure 3 (left) and (right), respectively. Two-dimensional spatial correlation function for CDL-B and C models are shown in Figure 5 and Figure 8, respectively. One dimensional spatial correlation functions for CDL-B and C models are shown in Figure 6 and Figure 9, respectively. The discrepancy of spatial correlation functions (on vertical displacement) of the 3D and 2D PAS is even higher on CDL-B and C models, as the spatial correlation decays faster due to more dispersed PAS. There the spatial correlation coefficients for 3D case are less than 0.1 for CDL-B and closer to 0 for CDL-C at 8 vertical distance in 3D case, but remain in unity in the 2D model case, i.e. the channel turns from almost independent fading to fully correlated.

### 3.1	CDL-A

Figure 1. Azimuth/elevation 3D PAS of CDL-A model with 2 strongest BS beams (left) and the corresponding 2D azimuthal marginal PAS (right).

Figure 2. Two-dimensional spatial correlation function in 3D PAS of CDL-A model with 2 strongest BS beams (left) and the corresponding 2D azimuthal marginal PAS (right).

Figure 3. One-dimensional spatial correlation function in 3D PAS of CDL-A model with 2 strongest BS beams (left) and the corresponding 2D azimuthal marginal PAS (right).

### 3.2	CDL-B

Figure 4. Azimuth/elevation 3D PAS of CDL-B model with 2 strongest BS beams (left) and the corresponding 2D azimuthal marginal PAS (right).

Figure 5. Two-dimensional spatial correlation function in 3D PAS of CDL-B model with 2 strongest BS beams (left) and the corresponding 2D azimuthal marginal PAS (right).

Figure 6. One-dimensional spatial correlation function in 3D PAS of CDL-B model with 2 strongest BS beams (left) and the corresponding 2D azimuthal marginal PAS (right).

### 3.3	CDL-C

Figure 7. Azimuth/elevation 3D PAS of CDL-C model with 2 strongest BS beams (left) and the corresponding 2D azimuthal marginal PAS (right).

Figure 8. Two-dimensional spatial correlation function in 3D PAS of CDL-C model with 2 strongest BS beams (left) and the corresponding 2D azimuthal marginal PAS (right).

Figure 9. One-dimensional spatial correlation function in 3D PAS of CDL-C model with 2 strongest BS beams (left) and the corresponding 2D azimuthal marginal PAS (right).

# 5	Summary

Observations:

- Channel models of TR 38.901 [1] are inherently three dimensional.

- With 3D models, the spatial correlation has a decreasing trend as the distance between observation points increases either in the horizontal or vertical direction.

- With 2D models the spatial correlation remains unity despite the spatial distance in the vertical direction.

- With 2D channel models, the MIMO performance may degrade significantly if DUT antennas separated by a vertical distance only

- Results in this contribution were presented for UMi only; for UMa, the spatial correlation decays even more rapidly due to wider elevation angle spread.

Based on the observations made in this paper, it is proposed to focus on the unmodified elevation angular parameters of 3D channel models in TR38.901 and to implement 3D probe configurations for the FR1 MPAC systems

Proposal: Focus on the 3D channel models in TR38.901 and implement 3D probe configurations for the FR1 MPAC systems used for NR MIMO OTA testing

# 6	References

TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz, V15.0.0 (2018-06)

R4-1814833, 2D vs. 3D MPAC Probe Configuration for FR1 CDL channel models, Keysight Technologies, 3GPP TSG-RAN WG4 Meeting #89, November 2018

R4-1901383, WF on NR MIMO OTA, CAICT, Spirent, Keysight, 3GPP TSG-RAN WG4 Meeting #90, March 2019

TR37.977, Verification of radiated multi-antenna reception performance of User Equipment (UE), V14.6.0 (2018-06).