3GPP TSG-RAN WG4 Meeting #90bis	R4-1904288

# Xi’an, China, 8th – 12th Apr, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev |  | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for 38.133 update SCell activation and deactivation (8.3) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-04-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Some of NR SCell activation and deactivation delay requirements are still not ready.(8.3) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Provide the requirements for SCell activation delay in FR2, Remove brakets in requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements are not complete in SCell activation and deactivation delay. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Start of change 1>>

## 8.3	SCell Activation and Deactivation Delay

### 8.3.1	Introduction

This section defines requirements for the delay within which the UE shall be able to activate a deactivated SCell and deactivate an activated SCell in EN-DC, or in standalone NR carrier aggregation, or in NE-DC, or in NR-DC.

The requirements shall apply for EN-DC, standalone NR carrier aggregation, NE-DC, and NR-DC.

### 8.3.2	SCell Activation Delay Requirement for Deactivated SCell

The requirements in this section shall apply for the UE configured with one downlink SCell in EN-DC, or in standalone NR carrier aggregation or in NE-DC or in NR-DC and when one SCell is being activated.

The delay within which the UE shall be able to activate the deactivated SCell depends upon the specified conditions.

Upon receiving SCell activation command in slot n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated no later than in slot n+ [THARQ + Tactivation_time + TCSI_Reporting], where:

THARQ is the timing between DL data transmission and acknowledgement as specified in [7].

Tactivation_time is the SCell activation delay. If the SCell is known and belongs to FR1, Tactivation_time is:

-	TSMTC_SCell + 5ms, if the SCell measurement cycle is equal to or smaller than 160ms.

-	TSMTC_MAX + TSMTC_SCell + 5ms, if the SCell measurement cycle is larger than 160ms.

If the SCell is unknown and belongs to FR1, Tactivation_time is:

-	2*TSMTC_MAX + 2*TSMTC_SCell + 5ms provided the SCell can be successfully detected on the first attempt.

If the SCell being activated belongs to FR2, Tactivation_time is:

-	TSMTC_SCell + 5ms if there is at least one active serving cell on that FR2 band, provided that the SSBs in the serving cell(s) and the SSBs in the SCell fulfil the condition defined in section 3.6.3.

-	if there is no active serving cell on that FR2 band provided that PCell or PSCell is FR1,

- if the SCell and gNB Tx beam are known,

- [TSMTC_SCell + 5ms], if the SCell measurement cycle is equal to or smaller than [160ms]

- [TSMTC_MAX + TSMTC_SCell + 5ms], if the SCell measurement cycle is larger than [160ms].

- if the SCell is known and gNB Tx beam is unknown,

- [2*TSMTC_SCell + 5ms], if the SCell measurement cycle is equal to or smaller than [160ms]

- [2*TSMTC_MAX + 2*TSMTC_SCell + 5ms], if the SCell measurement cycle is larger than [160ms].

- if the SCell is unknown,

- [8*TSMTC_MAX + 5*TSMTC_SCell + 5ms] provided the SCell can be successfully detected on the first attempt.

Where,

TSMTC_MAX:

-	In FR1, in case of intra-band SCell activation, TSMTC_MAX is the longer SMTC periodicity between active serving cells and SCell being activated provided the cell specific reference signals from the active serving cells and the SCells being activated or released are available in the same slot; in case of inter-band SCell activation, TSMTC_MAX is the SMTC periodicity of SCell being activated.

-	In FR2, TSMTC_MAX is the longer SMTC periodicity between active serving cells and SCell being activated provided that in Rel-15 only support FR2 intra-band CA.

-	TSMTC_MAX is bounded to a minimum value of 10ms.

TSMTC_SCell: SMTC periodicity of SCell being activated and the minimum value is 10ms.

TCSI_reporting is the delay including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources as specified in [2].

SCell in FR1 and FR2 is known if it has been meeting the following conditions:

-	During the period equal to max([5] measCycleSCell,  [5] DRX cycles) before the reception of the SCell activation command:

-	the UE has sent a valid measurement report for the SCell being activated and

-	the SSB measured remains detectable according to the cell identification conditions specified in section 9.2 and 9.3.

-	the SSB measured during the period equal to max([5] measCycleSCell, [5] DRX cycles) also remains detectable during the SCell activation delay according to the cell identification conditions specified in section 9.2 and 9.3.

Otherwise SCell in FR1 and FR2 is unknown.

Tx beam in FR2 is known if it has meeting the following conditions:

-	During the period equal to max([5] measCycleSCell,  [5] DRX cycles) for FR2 before the reception of the SCell activation command:

-	the UE has sent a valid measurement report for the SCell being activated and

-	the Tx beam remains the same according to the cell identification conditions specified in section 9.2 and 9.3,

-	The Tx beam also remains the same during the SCell activation delay according to the cell identification conditions specified in section 9.2 and 9.3.

Otherwise the Tx beam in FR2 is unknown.

If the UE has been provided with higher layer in TS 38.331 [2] signaling of smtc2 prior to the activation command, TSMTC_Scell follows smtc1 or smtc2 according to the physical cell ID of the target cell being activated. TSMTC_MAX follows smtc1 or smtc2 according to the physical cell IDs of the target cells being activated and the active serving cells.

In addition to CSI reporting defined above, UE shall also apply other actions related to the activation command specified in [2] for an SCell at the first opportunities for the corresponding actions once the SCell is activated.

The interruption for EN-DC on PSCell or any activated SCell in SCG specified in section 8.2 or the interruption on E-UTRA PCell or any activated E-UTRA SCell in MCG specified in section 7.32 of [15] shall not occur before slot n+1+THARQ and not occur after slot n+1+THARQ +3ms + TSMTC_MAX + TSMTC_duration.

The interruption on PCell or any activated SCell in MCG for NR standalone mode specified in section 8.2 shall not occur before slot n+1+THARQ and not occur after slot n+1+THARQ +3ms + TSMTC_MAX + TSMTC_duration.

The interruption on PCell or any activated SCell in MCG for NE-DC specified in section 8.2 and the interruption on E-UTRA PSCell or any activated E-UTRA SCell in SCG for NE-DC specified in section 7.36 of [15] shall not occur before slot n+1+[THARQ] and not occur after slot n+1+[THARQ +3ms + TSMTC_MAX + TSMTC_duration].

The interruption on PCell, PSCell or any activated SCell in MCG or SCG for NR-DC specified in section 8.2 shall not occur before slot n+1+[THARQ] and not occur after slot n+1+[THARQ +3ms + TSMTC_MAX + TSMTC_duration].

Starting from the slot specified in section 4.3 of [3] (timing for secondary Cell activation/deactivation) and until the UE has completed the SCell activation, the UE shall report out of range if the UE has available uplink resources to report CQI for the SCell.

### 8.3.3	SCell Deactivation Delay Requirement for Activated SCell

The requirements in this section shall apply for the UE configured with one downlink SCell in EN-DC, or in standalone NR carrier aggregation, or in NE-DC, or in NR-DC.

Upon receiving SCell deactivation command or upon expiry of the sCellDeactivationTimer in slot n, the UE shall accomplish the deactivation actions for the SCell being deactivated no later than in slot n+THARQ +3ms.

The interruption for EN-DC on PSCell or any activated SCell in SCG specified in section 8.2 or the interruption on E-UTRA PCell or any activated E-UTRA SCell in MCG specified in section 7.32 of [15] shall not occur before slot n+1+THARQ and not occur after slot n+1+THARQ +3ms.

The interruption on PCell or any activated SCell in MCG for NR standalone mode specified in section 8.2 shall not occur before slot n+1+[THARQ] and not occur after slot n+1+THARQ +3ms.

The interruption on PCell or any activated SCell in MCG for NE-DC specified in section 8.2 and the interruption on E-UTRA PSCell or any activated E-UTRA SCell in SCG for NE-DC specified in section -7.36 of [15] shall not occur before slot n+1+[THARQ] and not occur after slot n+1+[THARQ +3ms].

The interruption on PCell, PSCell or any activated SCell in MCG or SCG for NR standalone mode specified in section 8.2 shall not occur before slot n+1+[THARQ] and not occur after slot n+1+[THARQ +3ms].

<< End of change 1>>