3GPP TSG-RAN WG4 Meeting RAN4#90bis                                            R4-1904342

Xi’an, China, 8-12 April 2019

Agenda Item:   9.23

Source:        Ericsson

Title:          Regulatory situation and discussion on bandplan for 37-43.5 GHz range

Document for:  Discussion

# Introduction

In RAN#83 plenary in March 2019, a new WI has been approved on introduction of Band n259 [1]. According to the WID, work will start from RAN4#90bis (April 2019). The WI is scheduled to be completed by RAN4#93 (November 2019).

In this contribution, we present Regulatory situation of 37-43.5 GHz frequency range

# Background

WRC-19 agenda item 1.13, Resolution 238 (WRC-15) identifies 37-40.5 GHz, 42.5-43.5 GHz, which have allocations to the mobile service on a primary basis; and 40.5-42.5 GHz which may require additional allocations to the mobile service on a primary basis, among other as IMT candidate bands. Consequently, after WRC-15 there have been extensive discussions and studies on the suitability of these bands for IMT allocation in different regulatory organizations.

# Summary of discussions on 37-43.5 GHz frequency range in ITU [2]

3.1   APT, Asia-Pacific Telecommunity Members’ views

APT Members support identification of the frequency bands 37-40.5 GHz, 40.5-42.5 GHz and 42.5-43.5 GHz, or portions thereof, for IMT considering that protection of the incumbent services in these and adjacent frequency bands should be ensured.

APT Members recognize that different administrations would implement IMT in different portions of the 37-43.5 GHz frequency range for IMT, and a global identification for IMT in the 37-43.5 GHz band, or portions thereof, would allow each country/region to implement IMT in different portions of the band in accordance with their national/regional considerations, while still facilitating the benefits of economies of scale.

3.2   ASMG, Arab Spectrum Management Group Members’ views

Support IMT identification within the frequency bands:  40.5-42.5GHz and 42.5-43.5GHz

3.3   CEPT, Europe Members’ views

40.5 – 43.5 GHz

CEPT proposes an IMT identification for 40.5-43.5 GHz. This is a priority band for CEPT and already identified for future harmonization in Europe. CEPT considers that the bands 40.5-43.5 GHz has good potential for future harmonization in Europe.

37-40.5 GHz

Whilst CEPT will not propose identification and has no intention of using 37-40.5 GHz for IMT, CEPT will not oppose a global IMT identification for the full 37-43.5 GHz range.

3.4   CITEL Members’ views

Preliminary proposal to support IMT identification for the whole range, 37-43.5 GHz.

3.5   RCC, Regional Commonwealth in the field of Communications Members’ views

Position of the RCC Administrations on frequency bands 37.0−40.5 GHz, 40.5−42.5 GHz, included in Resolution 238 (WRC-15), is specified taking into account the need to protect both passive and active services.

Mobile Primary allocation

No Mobile allocation

# Compatibility studies with services in adjacent bands

In preparation for WRC-19, Task Group 5/1 was responsible for the development of draft Conference Preparation Meeting (CPM) text under WRC-19 Agenda item 1.13. In developing sharing studies and draft CPM text, Task Group 5/1 conducted and completed sharing and compatibility studies, taking into account the protection of services to which the band is allocated on a primary basis. The results of the studies can be found in CPM-19 document [3].

Table below is the information of the services allocated in the 36-47 GHz as adjacent bands that were studied on the sharing and compatibility with of the frequency range between 37-43.5 GHz by TG 5/1.

Allocation information in the 36-47GHz frequency range [4]

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 36-37 EARTH EXPLORATION-SATELLITE (passive) FIXED MOBILE SPACE RESEARCH (passive) 5.149 5.550A |  |  |
| 37-37.5 FIXED MOBILE except aeronautical mobile SPACE RESEARCH (space-to-Earth) 5.547 |  |  |
| 37.5-38 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE except aeronautical mobile SPACE RESEARCH (space-to-Earth) Earth exploration-satellite (space-to-Earth) 5.547 |  |  |
| 38-39.5 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE Earth exploration-satellite (space-to-Earth) 5.547 |  |  |
| 39.5-40 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE MOBILE-SATELLITE (space-to-Earth) Earth exploration-satellite (space-to-Earth) 5.547 |  |  |
| 40.5-41 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE BROADCASTING BROADCASTING-SATELLITE Mobile 5.547 | 40.5-41 FIXED FIXED-SATELLITE (space-to-Earth) 5.516B MOBILE BROADCASTING BROADCASTING-SATELLITE Mobile Mobile-satellite (space-to-Earth) 5.547 | 40.5-41 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE BROADCASTING BROADCASTING-SATELLITE Mobile 5.547 |
| 41-42.5 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE BROADCASTING BROADCASTING-SATELLITE Mobile 5.547 5.551F 5.551H 5.551I |  |  |
| 42.5-43.5 FIXED FIXED-SATELLITE (Earth-to-space) 5.552 MOBILE except aeronautical mobile RADIO ASTRONOMY 5.149 5.547 |  |  |
| 43.5-47 MOBILE 5.553 MOBILE-SATELLITE RADIONAVIGATION RADIONAVIGATION-SATELLITE 5.554 |  |  |

# 5	Frequency band definition

A proposed band plans for 37-43.5 GHz frequency range is shown in Figure below. Two bands are defined in this range, existing band n260 (37-40 GHz) and the new band n259 (39.5-43.5 GHz). They are noticeably 500 MHz overlapped that can be considered allowing economies of scale and allows a contiguous assignment of at least one NR carrier across the n259 and n260 upper and lower band edges. The width of overlap in multiples of 500 MHz would align with the block allocations used in the allocation table of the Radio Regulations (ITU-R) for the 40 GHz range.

Band definition in the frequency range 37- 43.5 GHz

Summary of the new NR band are provided in Table below:

New NR band in the frequency range 37- 43.5 GHz

| Band number | UL | DL | Duplex mode |
| --- | --- | --- | --- |
| n259 | 39.5 – 43.5 GHz | 39.5 - 43.5 GHz | TDD |

# Conclusion

From the WRC-19 preparations, it is evident that many administrations across all ITU regions are interested in the allocation of the whole or part of the 37-43.5 GHz frequency range to mobile services. Therefore, operating bands covering the entire 37-43.5 GHz is specified by 3GPP to accommodate operations in any part of this range. The range 37-40 GHz is already covered by the existing Band n260 (37-40 GHz) specified in Rel-15. By specifying the new band, n259 (39.5 – 43.5), in Rel-16 the whole range will be covered.

A partial overlap with Band n260 is beneficial for it allows a contiguous assignment of at least one NR carrier across the n259 and n260 upper and lower band edges. A width of overlap in multiples of 500 MHz would align with the block allocations used the allocation table of the Radio Regulations (ITU-R) for the 40 GHz range.

# Reference

[1] RP-190765, Introduction of Band n259; Ericsson, Huawei, HiSilicon

[2] https://www.itu.int/en/ITU-R/conferences/wrc/2019/Pages/reg-prep.aspx

[3] CPM19-2 report: https://www.itu.int/en/mediacentre/Pages/2019-CM02.aspx

[4] Radio Regulation 2016: https://www.itu.int/pub/R-REG-RR