3GPP TSG-RAN WG4 Meeting #90bis	R4-1904385

Xi’an, Shaanxi, China, 8 – 12 April 2019

Source:	Nokia, Nokia Shanghai Bell

Title:	Way forward on system simulation assumptions for Rel-16 HST

Agenda item:	7.15.3

Document for:	Approval

# 1	Introduction

This way forward proposes system simulation assumptions for performance evaluation of enhanced RRM mobility requirements under Rel-16 HST scenarios. The proposed simulation assumptions are based on the initial system simulation results in [1]. The purpose of system level simulation is used to validate whether the mobility performance could be guarantee with the enhance requirements in connected state.

# 2	Way forward

The simulation assumptions are listed in Table 1.

Table 1: Simulation parameters

| Parameters | Value | Comments |
| --- | --- | --- |
| Cell layout | 6 cells | 3 sites; 2 cells per site 2 wrap-around areas from the left and from the right |
| Network synchronization | Synchronized |  |
| Inter-site distance (Ds) | 1000 m |  |
| Minimum distance between eNB and railroad track (Dmin) | 50 m |  |
| Carrier frequency | 2.0 GHz |  |
| Antenna deployment | 11 degrees horizontal 6 degrees down-tilt |  |
| Antenna configuration | 3D antenna, 1x2 SIMO in DL Isotropic, 1x2 SIMO in UL |  |
| Receiver types | MRC in DL MRC in UL |  |
| Antenna gain | BS: 17dBi UE: 0dBi |  |
| Antenna height | BS: 35m UE: 1.5m |  |
| User speed | 500km/h (138.9m/s) |  |
| UE distribution | All UEs are generated consecutively in the left most point with the scenario in 1.44 seconds (500 km/h, 30 UE/s). | This is analogical of generating UEs in the train of 200m length. |
| Number of UEs | 63 |  |
| User mobility model | Constant speed, wrap around |  |
| Distance-dependent path loss | Simple logarithmic | 128.1+37.6log10(d) dB |
| Penetration loss | 20 dB |  |
| Slow fading/Shadowing | Not used |  |
| Fast fading | Not used |  |
| Traffic type | No data traffic in UL and DL | One packet is sent only in the beginning and in the end of simulations |
| Background load | Fixed as 0, 25%, 50%, 75%, 100% |  |
| Cell detection delay | PSS/SSS detection delay shall be within 5* DRX cycles when SINR>=-6dB |  |
| L1 measurement period | Measurement period shall be within 3* DRX cycles when SINR>=-6dB |  |
| T310 | 1s |  |
| T312 | Disabled |  |
| HO | A3-based |  |
| A3 event parameters | Hysteresis: 0 dB Threshold: 0, 3 dB Time To Trigger: 0 ms |  |
| HO Preparation delay | Constant delay of 50ms |  |
| L3 filtering | OFF |  |
| DRX | Long cycle values: OFF, 40ms, 80ms, 160ms, 320ms, 640ms | Other parameters: DRX.inactivityTimeInTTIs: 10 DRX.onDurationInTTIs: 5 |
| RACH parameters | Constant delay of 40ms |  |
| RRC measurement quantity | RSRP |  |
| RSRP Measurement Error Std. | 2dB | Random error of +-2dB with a normal distribution is added to the RSRP measurement of each cell. |
| Threshold Qout,SNR | -8 dB |  |
| Threshold Qin,SNR | -6 dB |  |
| RRC messages sent over the air | HO command, HO complete Measurement report Re-Establishment (request, response, complete) |  |
| RLM evaluation period | 10 DRX cycles | Per requirements in 36.133 |
| Simulation time | 10 min |  |

# References

- R4-1904383, Initial system simulation results for Rel-16 HST, Nokia, Nokia Shanghai Bell