#### 3GPP TSG RAN WG4 Meeting #90Bis				R4- 1904550

# Xi’an, China, April 8,  April 12, 2019

Agenda Item:	10.2.4

Source:	Spirent Communications

Title:	CDL Angle Modifications

Document for:	Approval

# 1	Introduction

RAN1 created channel models for MIMO link level testing in [1] presented as cluster delay line (CDL). However, all the CDLs in [1] have an unwanted behaviour. All CDLs seem to have two sets of three clusters, similar to expanded midpath clusters, but the CDLs define these as individual independent clusters. This creates a few undesired issues, like random power variations due to component summation with identical Doppler. This contribution presents proposals to correct that behaviour.

# 2	Details

[1] defines five CDL models, namely, CDL-A, CDL-B, and CDL-C represent NLOS scenarios, and CDL-D, CDL-E represent LOS scenarios. It is understood that each row in the CDLs defines a single cluster. In each CDL, there are two sets of three clusters that share the same AoD, AoA, ZoD, and ZoA (when the power of each set of three cluster is added, they end up being the two strongest NLOS paths in all CDLs.

For the reader’s convenience, CDL-A and CLD-C are presented next, where the issue is highlighted:

| Cluster # | Normalized delay |  | Power in [dB] |  | AOD in [°] |  | AOA in [°] |  | ZOD in [°] |  | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0.0000 |  | -13.4 |  | -178.1 |  | 51.3 |  | 50.2 |  | 125.4 |
| 2 | 0.3819 |  | 0 |  | -4.2 |  | -152.7 |  | 93.2 |  | 91.3 |
| 3 | 0.4025 |  | -2.2 |  | -4.2 |  | -152.7 |  | 93.2 |  | 91.3 |
| 4 | 0.5868 |  | -4 |  | -4.2 |  | -152.7 |  | 93.2 |  | 91.3 |
| 5 | 0.4610 |  | -6 |  | 90.2 |  | 76.6 |  | 122 |  | 94 |
| 6 | 0.5375 |  | -8.2 |  | 90.2 |  | 76.6 |  | 122 |  | 94 |
| 7 | 0.6708 |  | -9.9 |  | 90.2 |  | 76.6 |  | 122 |  | 94 |
| 8 | 0.5750 |  | -10.5 |  | 121.5 |  | -1.8 |  | 150.2 |  | 47.1 |
| 9 | 0.7618 |  | -7.5 |  | -81.7 |  | -41.9 |  | 55.2 |  | 56 |
| 10 | 1.5375 |  | -15.9 |  | 158.4 |  | 94.2 |  | 26.4 |  | 30.1 |
| 11 | 1.8978 |  | -6.6 |  | -83 |  | 51.9 |  | 126.4 |  | 58.8 |
| 12 | 2.2242 |  | -16.7 |  | 134.8 |  | -115.9 |  | 171.6 |  | 26 |
| 13 | 2.1718 |  | -12.4 |  | -153 |  | 26.6 |  | 151.4 |  | 49.2 |
| 14 | 2.4942 |  | -15.2 |  | -172 |  | 76.6 |  | 157.2 |  | 143.1 |
| 15 | 2.5119 |  | -10.8 |  | -129.9 |  | -7 |  | 47.2 |  | 117.4 |
| 16 | 3.0582 |  | -11.3 |  | -136 |  | -23 |  | 40.4 |  | 122.7 |
| 17 | 4.0810 |  | -12.7 |  | 165.4 |  | -47.2 |  | 43.3 |  | 123.2 |
| 18 | 4.4579 |  | -16.2 |  | 148.4 |  | 110.4 |  | 161.8 |  | 32.6 |
| 19 | 4.5695 |  | -18.3 |  | 132.7 |  | 144.5 |  | 10.8 |  | 27.2 |
| 20 | 4.7966 |  | -18.9 |  | -118.6 |  | 155.3 |  | 16.7 |  | 15.2 |
| 21 | 5.0066 |  | -16.6 |  | -154.1 |  | 102 |  | 171.7 |  | 146 |
| 22 | 5.3043 |  | -19.9 |  | 126.5 |  | -151.8 |  | 22.7 |  | 150.7 |
| 23 | 9.6586 |  | -29.7 |  | -56.2 |  | 55.2 |  | 144.9 |  | 156.1 |
| Per-Cluster Parameters |  |  |  |  |  |  |  |  |  |  |  |
| Parameter |  | cASD in [°] |  | cASA in [°] |  | cZSD in [°] |  | cZSA in [°] |  | XPR in [dB] |  |
| Value |  | 5 |  | 11 |  | 3 |  | 3 |  | 10 |  |

| Cluster # | Normalized delay |  | Power in [dB] |  | AOD in [°] |  | AOA in [°] |  | ZOD in [°] |  | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 |  | -4.4 |  | -46.6 |  | -101 |  | 97.2 |  | 87.6 |
| 2 | 0.2099 |  | -1.2 |  | -22.8 |  | 120 |  | 98.6 |  | 72.1 |
| 3 | 0.2219 |  | -3.5 |  | -22.8 |  | 120 |  | 98.6 |  | 72.1 |
| 4 | 0.2329 |  | -5.2 |  | -22.8 |  | 120 |  | 98.6 |  | 72.1 |
| 5 | 0.2176 |  | -2.5 |  | -40.7 |  | -127.5 |  | 100.6 |  | 70.1 |
| 6 | 0.6366 |  | 0 |  | 0.3 |  | 170.4 |  | 99.2 |  | 75.3 |
| 7 | 0.6448 |  | -2.2 |  | 0.3 |  | 170.4 |  | 99.2 |  | 75.3 |
| 8 | 0.6560 |  | -3.9 |  | 0.3 |  | 170.4 |  | 99.2 |  | 75.3 |
| 9 | 0.6584 |  | -7.4 |  | 73.1 |  | 55.4 |  | 105.2 |  | 67.4 |
| 10 | 0.7935 |  | -7.1 |  | -64.5 |  | 66.5 |  | 95.3 |  | 63.8 |
| 11 | 0.8213 |  | -10.7 |  | 80.2 |  | -48.1 |  | 106.1 |  | 71.4 |
| 12 | 0.9336 |  | -11.1 |  | -97.1 |  | 46.9 |  | 93.5 |  | 60.5 |
| 13 | 1.2285 |  | -5.1 |  | -55.3 |  | 68.1 |  | 103.7 |  | 90.6 |
| 14 | 1.3083 |  | -6.8 |  | -64.3 |  | -68.7 |  | 104.2 |  | 60.1 |
| 15 | 2.1704 |  | -8.7 |  | -78.5 |  | 81.5 |  | 93.0 |  | 61.0 |
| 16 | 2.7105 |  | -13.2 |  | 102.7 |  | 30.7 |  | 104.2 |  | 100.7 |
| 17 | 4.2589 |  | -13.9 |  | 99.2 |  | -16.4 |  | 94.9 |  | 62.3 |
| 18 | 4.6003 |  | -13.9 |  | 88.8 |  | 3.8 |  | 93.1 |  | 66.7 |
| 19 | 5.4902 |  | -15.8 |  | -101.9 |  | -13.7 |  | 92.2 |  | 52.9 |
| 20 | 5.6077 |  | -17.1 |  | 92.2 |  | 9.7 |  | 106.7 |  | 61.8 |
| 21 | 6.3065 |  | -16 |  | 93.3 |  | 5.6 |  | 93.0 |  | 51.9 |
| 22 | 6.6374 |  | -15.7 |  | 106.6 |  | 0.7 |  | 92.9 |  | 61.7 |
| 23 | 7.0427 |  | -21.6 |  | 119.5 |  | -21.9 |  | 105.2 |  | 58 |
| 24 | 8.6523 |  | -22.8 |  | -123.8 |  | 33.6 |  | 107.8 |  | 57 |
| Per-Cluster Parameters |  |  |  |  |  |  |  |  |  |  |  |
| Parameter |  | cASD in [°] |  | cASA in [°] |  | cZSD in [°] |  | cZSA in [°] |  | XPR in [dB] |  |
| Value |  | 2 |  | 15 |  | 3 |  | 7 |  | 7 |  |

Since the AoA and ZoA are identical for these cases, then the sinusoids form a coherent sum across the 3 taps due to identical cluster doppler (see equation 7.5-22 in [1] This is the generic equation for N-2 weakest clusters channel coefficient).  Thus, at a given frequency, this combining effect creates a power change within the distribution of rays, and this impacts the statistical behavior of the model.  It also creates sensitivity to phases, which will add more variation than expected.

This problem was first solved for single clusters in [2], where for the single cluster, all the AoAs are set to the same angle (for the single cluster it is 0°), and this creates an autocorrelation dependency on the initial phases of the rays (also called subpaths).

[1] allows an arbitrary rotation of angles to obtain desired cluster angular locations, but a simple dithering approached is preferred so the circular rms angle spread remains close to that of the original CDLs

Proposal 1: For the denormalized and angle-scaled CDLs, the values of angles must be dithered by 1°.

# 3	Conclusions

Proposal 1: For the denormalized and angle-scaled CDLs, the values of angles must be dithered by 1°.

# 4	References

[1] TR38.901, “Study on channel model for frequencies from 0.5 to 100 GHz,” 3GPP, March 2017

[2] MOSG131018, “Modification to the Single Cluster Model”, October 2013