3GPP TSG-RAN4 WG4 Meeting #90Bis	R4-1904747

# Xi'an, China,  8 - 12 April, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-2 | CR | - | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR: Updates to PUCCH format 3 and 4 radiated conformance testing in TS 38.141-2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-03-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Still TBD for some PUCCH 3 and 4 radiated conformance tests. New PUCCH format 3 and 4 conformance requirements are agreed to be defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the radiated conformance tests for PUCCH format 3 and 4. Added new PUCCH format 3 and 4 cases for FR2 with additional DMRS configured. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some radidated conformance tests for PUCCH format 3 and 4 will still be TBD or missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.4, 8.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### 8.3.4	Performance requirements for PUCCH format 3

#### 8.3.4.1	Definition and applicability

The performance is measured by the required SNR at UCI block error probability not exceeding 1%.

The UCI block error probability is defined as the conditional probability of incorrectly decoding the UCI information when the UCI information is sent. The UCI information does not contain CSI part 2.

The transient period as specified in TS 38.101-1[X] subclause 6.3.3.1 and TS 38.101-2[Y] subclause 6.3.3.1 is not taken into account for performance requirement testing, where the RB hopping is symmetric to the CC center, i.e. intra-slot frequency hopping is enabled.

Which specific test(s) are applicable to BS is based on the test applicability rules defined in subclause 8.1.2.2.

A test with or without additional DMRS configured is only applicable if the BS support it.

#### 8.3.4.2	Minimum requirement

For BS type 1-O, the minimum requirement is in TS 38.104 [2], subclause 11.3.1.5.

For BS type 2-O, the minimum requirement is in TS 38.104 [2], subclause 11.3.2.5.

#### 8.3.4.3	Test purpose

The test shall verify the receiver’s ability to detect UCI under multipath fading propagation conditions for a given SNR.

#### 8.3.4.4	Method of test

##### 8.3.4.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested for single carrier (SC): M; see subclause 4.9.1

Direction to be tested:

-	For BS type 1-O, receiver target reference direction (see D.31 in Table 4.6-1).

-	For BS type 2-O, OTA REFSENS receiver target reference direction (see D.54 in Table 4.6-1).

##### 8.3.4.4.2	Procedure

OTA test requires correct use of an appropriate test facility which has been calibrated and is capable of performing measurements within the measurement uncertainties in subclause 4.1.2.4.

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.3.

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Set the BS in the declared direction to be tested.

4)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex E.3. Each of the demodulation branch signals should be transmitted on each polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to TS 38.211 [20], and according to additional test parameters listed in Table 8.3.4.4.2-1.

Table 8.3.4.4.2-1: Test parameters

| Parameter | Test 1 | Test 2 |
| --- | --- | --- |
| Modulation | QPSK |  |
| startingPRB | 0 |  |
| intraSlotFrequencyHopping | enabled |  |
| secondHopPRB | The largest PRB index - nrofPRBs |  |
| pucch-GroupHopping | neither |  |
| hoppingId | 0 |  |
| nrofPRBs | 1 | 3 |
| nrofSymbols | 14 | 4 |
| the number of UCI bits | 16 | 16 |
| startingSymbolIndex | 0 | 0 |

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex J.

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the BS receiver is as specified in subclause 8.3.4.5.1 and 8.3.4.5.2 for BS type 1-O and BS type 2-O respectively, and the SNR at the BS receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in Table 8.3.4.4.2-2.

Table 8.3.4.4.2-2: AWGN power level at the BS input

| BS type | Subcarrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| BS type 1-O | 15 kHz | 5 | [-83.5] dBm - ΔOTAREFSENS/ 4.5MHz |
|  |  | 10 | [-80.3] dBm - ΔOTAREFSENS/ 9.36MHz |
|  |  | 20 | [-77.2] dBm - ΔOTAREFSENS/ 19.08MHz |
|  | 30 kHz | 10 | [-80.7] dBm - ΔOTAREFSENS/ 8.64MHz |
|  |  | 20 | [-77.4] dBm - ΔOTAREFSENS/ 18.36MHz |
|  |  | 40 | [-74.2] dBm - ΔOTAREFSENS/ 38.16MHz |
|  |  | 100 | [-70.1] dBm - ΔOTAREFSENS/ 98.28MHz |
| BS type 2-O | 60 kHz | 50 | TBD |
|  |  | 100 | TBD |
|  |  | 50 | TBD |
|  |  | 100 | TBD |
|  |  | 200 | TBD |

#### 8.3.4.5	Test requirement

##### 8.3.4.5.1	Test requirement for BS type 1-O

The fraction of incorrectly decoded UCI is shall be less than 1% for the SNR listed in table 8.3.4.5.1-1 and table 8.3.4.5.1-2.

Table 8.3.4.5.1-1: Required SNR for PUCCH format 3 with 15kHz SCS

| Test Number | Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and correlation matrix (Annex J) | Additional DM-RS configuration | Channel Bandwidth / SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 5 MHz | 10 MHz | 20 MHz |
| 1 | 1 | 2 | Normal | TDLC300-100 Low | No additional DM-RS | [0.7] | [1.7] | [0.9] |
|  |  |  |  |  | Additional DM-RS | [0.4] | [1.1] | [0.4] |
| 2 | 1 | 2 | Normal | TDLC300-100 Low | No additional DM-RS | TBD | [2.6] | [2.4] |

Table 8.3.4.5.1-2: Required SNR for PUCCH format 3 with 30kHz SCS

| Test Number | Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and correlation matrix (Annex J) | Additional DM-RS configuration | Channel Bandwidth / SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 10 MHz | 20 MHz | 40 MHz | 100 MHz |
| 1 | 1 | 2 | Normal | TDLC300-100 Low | No additional DM-RS | [1.6] | [1.3] | [1.2] | TBD |
|  |  |  |  |  | Additional DM-RS | [0.9] | [0.8] | [0.6] | [0.5] |
| 2 | 1 | 2 | Normal | TDLC300-100 Low | No additional DM-RS | [2.3] | [2.4] | [2.6] | TBD |

##### 8.3.4.5.2	Test requirement for BS type 2-O

The fraction of incorrectly decoded UCI is shall be less than 1% for the SNR listed in table 8.3.4.5.2-1 and table 8.3.4.5.2-2.

Table 8.3.4.5.2-1: Required SNR for PUCCH format 3 with 60kHz SCS

| Test Number | Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and correlation matrix (Annex J) | Additional DM-RS configuration | Channel Bandwidth / SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 50 MHz | 100 MHz |
| 1 | 1 | 2 | Normal | TDLA30-300 Low | No additional DM-RS | [2.4] | [1.3] |
|  |  |  |  |  | Additional DM-RS | [2.3] | [1.6] |
| 2 | 1 | 2 | Normal | TDLA30-300 Low | No additional DM-RS | [3.5] | [3.0] |

Table 8.3.4.5.2-2: Required SNR for PUCCH format 3 with 120kHz SCS

| Test Number | Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and correlation matrix (Annex J) | Additional DM-RS configuration | Channel Bandwidth / SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 50 MHz | 100 MHz | 200 MHz |
| 1 | 1 | 2 | Normal | TDLA30-300 Low | No additional DM-RS | [2.0] | [1.3] | [2.4] |
|  |  |  |  |  | Additional DM-RS | [2.0] | [2.0] | [1.7] |
| 2 | 1 | 2 | Normal | TDLA30-300 Low | No additional DM-RS | TBD | [3.5] | [2.0] |

### 8.3.5	Performance requirements for PUCCH format 4

#### 8.3.5.1	Definition and applicability

The performance is measured by the required SNR at UCI block error probability not exceeding 1%.

The UCI block error probability is defined as the conditional probability of incorrectly decoding the UCI information when the UCI information is sent. The UCI information does not contain CSI part 2.

The transient period as specified in TS 38.101-1[X] and TS 38.101-2[Y] subclause 6.3.3.1 is not taken into account for performance requirement testing, where the RB hopping is symmetric to the CC center, i.e. intra-slot frequency hopping is enabled.

Which specific test(s) are applicable to BS is based on the test applicability rules defined in subclause 8.1.2.2.

A test with or without additional DMRS configured is only applicable if the BS support it.

#### 8.3.5.2	Minimum requirement

For BS type 1-O, the minimum requirement is in TS 38.104 [2], subclause 11.3.1.6.

For BS type 2-O, the minimum requirement is in TS 38.104 [2], subclause 11.3.2.6.

#### 8.3.5.3	Test purpose

The test shall verify the receiver’s ability to detect UCI under multipath fading propagation conditions for a given SNR.

#### 8.3.5.4	Method of test

##### 8.3.5.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested for single carrier (SC): M; see subclause 4.9.1

Direction to be tested:

-	For BS type 1-O, receiver target reference direction (see D.31 in Table 4.6-1).

-	For BS type 2-O, OTA REFSENS receiver target reference direction (see D.54 in Table 4.6-1).

##### 8.3.5.4.2	Procedure

OTA test requires correct use of an appropriate test facility which has been calibrated and is capable of performing measurements within the measurement uncertainties in subclause 4.1.2.4.

1)	Place the BS with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.3.

2)	Align the manufacturer declared coordinate system orientation of the BS with the test system.

3)	Set the BS in the declared direction to be tested.

4)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex E.3. Each of the demodulation branch signals should be transmitted on each polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to TS 38.211 [20], and according to additional test parameters listed in Table 8.3.4.4.2-1.

Table 8.3.5.4.2-1: Test parameters

| Parameter | Value |
| --- | --- |
| Modulation | QPSK |
| startingPRB | 0 |
| intraSlotFrequencyHopping | enabled |
| secondHopPRB | The largest PRB index - nrofPRBs |
| pucch-GroupHopping | neither |
| hoppingId | 0 |
| nrofSymbols | 14 |
| the number of UCI bits | 22 |
| startingSymbolIndex | 0 |
| occ-Length | n2 |
| occ-Index | n0 |

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex J.

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the BS receiver is as specified in subclause 8.3.5.5.1 and 8.3.5.5.2 for BS type 1-O and BS type 2-O respectively, and that the SNR at the BS receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in Table 8.3.5.4.2-2.

Table 8.3.5.4.2-2: AWGN power level at the BS input

| BS type | Subcarrier spacing(SCS) (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| BS type 1-O | 15 kHz | 5 | [-83.5] dBm - ΔOTAREFSENS/ 4.5MHz |
|  |  | 10 | [-80.3] dBm - ΔOTAREFSENS/ 9.36MHz |
|  |  | 20 | [-77.2] dBm - ΔOTAREFSENS/ 19.08MHz |
|  | 30 kHz | 10 | [-80.7] dBm - ΔOTAREFSENS/ 8.64MHz |
|  |  | 20 | [-77.4] dBm - ΔOTAREFSENS/ 18.36MHz |
|  |  | 40 | [-74.2] dBm - ΔOTAREFSENS/ 38.16MHz |
|  |  | 100 | [-70.1] dBm - ΔOTAREFSENS/ 98.28MHz |
| BS type 2-O | 60 kHz | 50 | TBD |
|  |  | 100 | TBD |
|  |  | 50 | TBD |
|  |  | 100 | TBD |
|  |  | 200 | TBD |

#### 8.3.5.5	Test requirement

##### 8.3.5.5.1	Test requirement for BS type 1-O

The fraction of incorrectly decoded UCI is shall be less than 1% for the SNR listed in table 8.3.5.5.1-1 and table 8.3.5.5.1-2.

Table 8.3.5.5.1-1: Required SNR for PUCCH format 4 with 15kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and correlation matrix (Annex J) | Additional DM-RS configuration | Channel Bandwidth / SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 5 MHz | 10 MHz | 20 MHz |
| 1 | 2 | Normal | TDLC300-100 Low | No additional DM-RS | [2.4] | [3.1] | [2.8] |
|  |  |  |  | Additional DM-RS | [2.2] | [2.9] | [2.4] |

Table 8.3.5.5.1-2: Required SNR for PUCCH format 4 with 30kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and correlation matrix (Annex J) | Additional DM-RS configuration | Channel Bandwidth / SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 10 MHz | 20 MHz | 40 MHz | 100 MHz |
| 1 | 2 | Normal | TDLC300-100 Low | No additional DM-RS | [3.6] | [3.2] | [3.6] | [2.9] |
|  |  |  |  | Additional DM-RS | [3.3] | [2.9] | [3.6] | [2.4] |

##### 8.3.5.5.2	Test requirement for BS type 2-O

The fraction of incorrectly decoded UCI is shall be less than 1% for the SNR listed in table 8.3.5.5.2-1 and table 8.3.5.5.2-2.

Table 8.3.5.5.2-1: Required SNR for PUCCH format 4 with 60kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and correlation matrix (Annex J) | Additional DM-RS configuration | Channel Bandwidth / SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 50 MHz | 100 MHz |
| 1 | 2 | Normal | TDLA30-300 Low | No additional DM-RS | [3.6] | [3.3] |
|  |  |  |  | Additional DM-RS | [4.8] | [3.7] |

Table 8.3.5.5.2-2: Required SNR for PUCCH format 4 with 120kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic Prefix | Propagation conditions and correlation matrix (Annex J) | Additional DM-RS configuration | Channel Bandwidth / SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 50 MHz | 100 MHz | 200MHz |
| 1 | 2 | Normal | TDLA30-300 Low | No additional DM-RS | [3.4] | [3.4] | TBD |
|  |  |  |  | Additional DM-RS | TBD | [4.5] | [4.6] |