3GPP TSG-WG RAN4 Meeting #90bis	R4-1904881

# Xian, China, 8th – 12th April, 2019                                                            (Rev. of R4-1903081)

Source: 	Nokia, Nokia Shanghai Bell

Title:	TP for TR 37.716-31-11 DC_2A-30A-66A_n5A, DC_2A-2A-30A-66A_n5A and DC_2A-30A-66A-66A_n5A

Agenda Item: 			9.5.2 [DC_R16_3BLTE_1BNR_4DL2UL-Core]

Document for:	Approval

# 1	Introduction

This contribution is a TP for TR 37.716-31-11 to introduce the option for DC_2A-30A-66A_n5A, DC_2A-2A-30A-66A_n5A and DC_2A-30A-66A-66A_n5A

*********************** Start of the TP ***************************************************

## 5.X	DC_2A-30A-66A_n5A, DC_2A-2A-30A-66A_n5A and DC_2A-30A-66A-66A_n5A

### 5.1.X.1	Operating bands for DC

Table 5.1.X.1-1: Band combinations EN-DC (four bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-30-66_n5 | CA_2-30-66 | n5 | No |

### 5.1.X.2	Configuration for DC

Table 6.1.X.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_2A-30A-66A_n5A | DC_2A_n5A DC_30A_n5A DC_66A_n5A | CA_2A-30A-66A | n5A |
| DC_2A-2A-30A-66A_n5A | DC_2A_n5A DC_30A_n5A DC_66A_n5A | CA_2A-2A-30A-66A | n5A |
| DC_2A-30A-66A-66A_n5A | DC_2A_n5A DC_30A_n5A DC_66A_n5A | CA_2A-30A-66A-66A | n5A |

### 5.1.X.3	Coexistence studies

Based on co-existence studies of DC_66A_n5A in TR 37.863-01-01, the 2nd and 5th IMD of 66+n5 may fall into the band n5 downlink and Pcell REFSENS excpetions can be represented by the REFSENS expcetion already specified for DC_66A-n5A.

Based on co-existence studies of DC_30A_n5A in TR 37.863-01-01, 5th IMD product of 30+n5 may fall into band 66 downlink.

Co-existence studies for DC_2A_n5A is also available in TR 37.863-01-01. For this 4th IMD product of 2+n5 may fall into band 66 downlink.

### 5.1.X.4	∆TIB and ∆RIB values

The TIB,c and RIB,c values are derived from CA_2-5-30-66 in TS 36.101.

Table 5.X.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2A-30A-66A_n5A, DC_2A-2A-30A-66A_n5A, DC_2A-30A-66A-66A_n5A | 2 | 0.5 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |
|  | n5 | 0.3 |

Table 5.X.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2A-30A-66A_n5A, DC_2A-2A-30A-66A_n5A, DC_2A-30A-66A-66A_n5A | 2 | 0.4 |
|  | 30 | 0.5 |
|  | 66 | 0.4 |
|  | n5 | 0 |

### 5.1.X.5	Refsens requirements

No further refsens requirements are needed as this is covered by lower order combinations.

**************************** End of changes ********************************************