# 3GPP TSG-RAN WG4 Meeting #90bis			  												R4-1905194

# Xian, China, 8 – 12 April, 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-2 | CR | CRNum | rev | 1 | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to 38.141-2 with updates of Annex F |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  |  | Date: |  | 2019-04-11 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Based on the discussion during RAN4#90bis meeting and WF on Annex F for test specifications, lots of corrections are needed to update the LTE content for NR This draft CR further updates details in Annex F based on endorsed draft CR [1] during RAN4#90 meeting in Athens. [1] R4-1901340 Draft CR to 38.141-2 with corrections and additions to Annex F |  |  |  |  |  |  |  |  |
| Summary of change: |  | - “Subframe(s)” changed to “slot(s)” - sTTI removed from all sections. - F.2.1 deleted sentence “frame structure 1 with” - F.2.3 editorial restructure of text - F.2.5 Missing text when copied from LTE added “The result of the frequency…” - F.2.5 Updated figures that include point for RE TX power measurement - F.3.1 Editorial corrections - F.3.2 remove primary synchronization signals and update the window position for EVM measurement; - F.3.3 update the formula for RETP, remove RSTP and corrections on the OSTP; - F.7. remove the primary synchronization singals and remove the exceptions for DC carrier - F.7.1 remove the items for sTTI and replace the subframe with slot and put the REs numbers in the square bracket. - F.8 “TDD special fields…” deleted |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification would not include require details to perform tests. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex F |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | N | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

Annex F (normative):
Global In-channel Tx tests

# F.1	General

Editor’s note: Placeholder for the remaining elements of the “Global in-channel Tx test”, e.g. reuse of the E-UTRA annex for frequency error measurement description which is performed together with the EVM.

The Global in-channel Tx test enables the measurement of all relevant parameters that describe the in-channel quality of the output signal of the TX under test in a single measurement process.

The parameters describing the in-channel quality of a transmitter, however, are not necessarily independent. The algorithm chosen for description inside this annex places particular emphasis on the exclusion of all interdependencies among the parameters.

# F.2.1	Basic principles

The process is based on the comparison of the actual output signal of the TX under test, received by an ideal receiver, with a reference signal, that is generated by the measuring equipment and represents an ideal error free received signal. All signals are represented as equivalent (generally complex) baseband signals.

The description below uses numbers and illustrations as examples. These numbers are taken from  normal CP length and a transmission bandwidth configuration of [NRB = 100]. The application of the text below, however, is not restricted to this parameterset.

## F.2.2 Output signal of the TX under test

The output signal of the TX under test is acquired by the measuring equipment and stored for further processsing. It is sampled at a sampling rate of [30.72] Msps and it is named z(ν). In the time domain it comprises at least 1 frame: z(ν). It is modelled as a signal with the following parameters: demodulated data content, carrier frequency, amplitude and phase for each subcarrier.

## F.2.3 Reference signal

Two types of reference signal are defined:

- The reference signal i1(ν) is constructed by the measuring equipment according to the relevant TX specifications, using the following parameters:

  - demodulated data content,

  - nominal carrier frequency,

  - nominal amplitude and phase for each subcarrier.

It is represented as a sequence of samples at a sampling rate of [30.72] Msps in the time domain. The structure of the signal is described in the test models.

- The reference signal i2(ν) is constructed by the measuring equipment according to the relevant TX specifications, using the following parameters:

  - restricted data content:

    - nominal Reference Symbols and the Primary Synchronisation Channel (all other modulation symbols are set to 0 V),

    - nominal carrier frequency,

    - nominal amplitude and phase for each applicable subcarrier,

    - nominal timing.

It is represented as a sequence of samples at a sampling rate of [30.72] Msps in the time domain.

## F.2.4 Measurement results

The measurement results, achieved by the global in channel TX test are the following:

-	Carrier Frequency error

-	EVM (Error Vector Magnitude)

-	Resource Element TX power

-	OFDM Symbol TX power (OSTP)

Other side results are: residual amplitude- and phase response of the TX chain after equalisation.

## F.2.5 Measurement points

The Resource element TX power is measured after the FFT as described in figures F.2.5-1 and F.2.5-2. The EVM shall be measured at the point after the FFT and a zero-forcing (ZF) equalizer in the receiver, as depicted in for FR1 in figure F.2.5-1 and for FR2 in figure F.2.5-2. The result of the frequency synchronisation is the frequency offset. It is performed in the pre- and/or post-FFT domain. For FR2 The FFT window of [2048] samples out of [2194] samples (data +CP) in the time domain is selected in the box CP removal.

Figure F.2.5-1: Reference point for FR1 EVM measurement

Figure F.2.5-2: Reference point for FR2 EVM measurement

# F.3	Void

F.3.1 Pre FFT minimization process

Sample Timing, Carrier Frequency in z(ν) are varied in order to minimise the difference between z(ν) and i1 (ν) , after the amplitude ratio of z(ν) and i1 (ν) has been scaled. Best fit (minimum difference) is achieved when the RMS difference value between z(ν) and i(ν) is an absolute minimum.

The carrier frequency variation  is  the measurement result: Carrier Frequency Error.

From the acquired samples one carrier frequency error can be derived.

Note 1:	The minimization process, to derive the RF error can be supported by Post FFT operations. However the minimisation process defined in the pre FFT domain comprises all acquired samples (i.e. it does not exclude the samples inbetween the FFT widths and it does not exclude the bandwidth outside the transmission bandwidth configuration.

Note 2:	The algorithm would allow to derive Carrier Frequency error and Sample Frequency error of the TX under test separately. However there are no requirements for Sample Frequeny error. Hence the algorithm models the RF and the sample frequency commonly (not independently). It returns one error and does not distinguish between both.

After this process the samples z(ν) are called z0(ν).

## F.3.2 Timing of the FFT window

The FFT window length is [2048] samples per OFDM symbol. [140] FFTs ([286720] samples) cover less than the acquired number of samples ([307200] samples in 10 slots) The position in time for FFT must be determined.

In an ideal signal, the FFT may start at any instant within the cyclic prefix without causing an error. The TX filter, however, reduces the window. The EVM requirements shall be met within a window W<CP. There are three different instants for FFT:

Centre of the reduced window, called , ΔC –W/2 and ΔC +W/2,

The BS shall transmit a signal according to the Test models, intended for EVM. The reference signal shall be used to find the centre of the FFT window.

The timing of the measured signal is determined in the pre FFT domain as follows, using z0(ν) and i2(ν) :

1.	The measured signal is delay spread by the TX filter. Hence the distinct boarders between the OFDM symbols and between Data and CP are also spread and the timing is not obvious.

2.	In the Reference Signal i2(ν) the timing is known.

Correlation between (1.) and (2.) will result in a correlation peak. The meaning of the correlation peak is approx. the “impulse response” of the TX filter.

3.	The meaning of “impulse response” assumes that the autocorrelation of the reference signal i2(ν) is a Dirac peak and that the correlation between the reference signal i2(ν)  and the data in the measured signal is 0. The correlation peak, (the highest, or in case of more than one highest, the earliest) indicates the timing in the measured signal.

The number of samples, used for FFT is reduced compared to z0(ν). This subset of  samples is called z’(ν).

From the acquired samples one timing can be derived.

The timing of the centre  with respect to the different CP length in a slot is as follows: (normal CP length)

[ is on Tf =72 within  the CP of length 144  (in OFDM symbol 1 to 6)]

[ is on Tf=88 (=160-72)  within  the CP of length 160 (in OFDM symbol 0)]

## F.3.3	Resource Element TX power

Perform FFT (z’(ν))  with the FFT window timing

The result is called Z’(t,f). The RE TX power is then defined as:

It is an average power and accumulates the powers of the reference symbols within a slot divided by n, the number of  reference symbols within a slote.

From RETP the OFDM Symbol TX power (OSTP) is derived as follows:

It accumulates all sub carrier powers of the 4th OFDM symbol. The 4th (out of 14 OFDM symbols) within a slot (in case of  normal CP length)) contains exclusively PDSCH.

From the acquired samples 10 values for each RSTP and OSTP can be derived

# F.4	Void

# F.5	Void

# F.6	Void

# F.7	Post FFT equalisation

Perform [140] FFTs on z’(ν), one for each OFDM symbol comprising the full frame with the FFT window timing. (in case of normal CP length) The result is an array of samples, 140 in the time axis t times [2048] in the frequency axis f.

The equalizer coefficients and  are determined as follows:

1.	Calculate the complex ratios (amplitude and phase) of the post-FFT acquired signal  and the post-FFT Ideal signal , for each reference symbol, over [10 slots]. This process creates a set of complex ratios:

Where the post-FFT Ideal signal  is constructed by the measuring equipment according to the relevant TX specifications, using the following parameters:

- For FR1: restricted content: i.e. nominal Reference Symbols (all other modulation symbols are set to 0 V), nominal carrier frequency,  nominal amplitude and phase for each applicable subcarrier, nominal timing.

- For FR2: restricted content: i.e. nominal Demodulation Reference.

2.	Perform time averaging at each reference signal subcarrier of the complex ratios, the time-averaging length is [10 slots]. Prior to the averaging of the phases  an unwrap operation must be performed according to the following definition: The unwrap operation corrects the radian phase angles of  by adding multiples of 2*PI when absolute phase jumps between consecutive time instances ti are greater than or equal to the jump tolerance of PI radians. This process creates an average amplitude and phase for each reference signal subcarrier (i.e. every second subcarrier).

Where N is the number of reference symbol time-domain locations ti from Z’(f,t) for each reference signal subcarrier .

3.	The equalizer coefficients for amplitude and phase  and  at the reference signal subcarriers are obtained by computing the moving average in the frequency domain of the time-averaged reference signal subcarriers, i.e. every second subcarrier. The moving average window size is 19. For reference subcarriers at or near the edge of the channel the window size is reduced accordingly as per figure F.7-1.

4.	Perform linear interpolation from the equalizer coefficients  and  to compute coefficients ,  for each subcarrier.

Figure F.7-1: Reference subcarrier smoothing in the frequency domain

a)	In case of FR2 EVM, to account for the common phase error (CPE) experienced in millimetre wave frequencies, φ(f), in the estimated coefficients contain phase rotation due to the CPE, θ, in addition to the phase of the equalizer coefficient φf, that is

φf=φf+θ(t)

For OFDM symbols where PT-RS does not exist, θ(t) can be estimated by performing linear interpolation from neighboring symbols where PT-RS is present.

In order to separate component of the CPE, θ, contained in, φ(f), estimation and compensation of the CPE needs to follow. θ(t) is the common phase error (CPE), that rotates all the subcarriers of the OFDM symbol at time t.

Estimate of the CPE, θ(t), at OFDM symbol time, t, can then be obtained from using the PT-RS employing the expression

θ(t)=argf∈fptrsZ't,fIptrst,f afe-jφf

In the above equation, fptrs is the set of subcarriers where PT-RS are mapped, t∈tptrs where  tptrs is the set of OFDM symbols where PT-RS are mapped while Z't,f and Iptrst,f are is the post-FFT acquired signal and the ideal PT-RS signal respectively. That is, estimate of the CPE at a given OFDM symbol is obtained from frequency correlation of the complex ratios at the PT-RS positions with the conjugate of the estimated equalizer complex coefficients. The estimated CPE can be subtracted from φf to remove influence of the CPE, and obtain estimate of the complex coefficient’s phase

φf=φf-θ(t)

## F.7.1 EVM

For EVM create two sets of Z’eq(f,t)., according to the timing ”ΔC –W/2 and ΔC +W/2”, using the equalizer coefficients from F.3.4.

The equivalent ideal samples are calculated form  i1(ν) (clause F.2.3) and are called I(f,t).

The EVM is the difference between the ideal waveform and the measured and equalized waveform.

,

where

is the set of symbols with the considered modulation scheme being active within the slot,.

is the set of subcarriers within the  resource blocks with the considered modulation scheme being active in symbol t,

is the ideal signal reconstructed by the measurement equipment in accordance with relevant Test models,

is the equalized signal under test.

Note1:	Although the basic unit of measurement is one slot, the equalizer is calculated over the entire [10 slots] measurement period to reduce the impact of noise in the reference symbols.

Note 2:	Applicability of EVM calculation:

One EVM value is associated to 12 subcarriers times 1 slot = 168 resource elements.

But only a reduced number of REs in this slot contribute to EVM. Those are the PDSCH REs, containing the considered modulation scheme. Only those slots are evaluated with respect to EVM, which contain the maximum number of PDSCH REs. (EVM-relevant location in the time/frequency grid ) The others are not evaluated.

In specific:

-	For all other Bandwidths:

-	Only the slots containing 150 PDSCH REs are used for EVM. Only those 150 REs contribute to EVM

-	All slots, which contain less than 150 PDSCH REs, are not evaluated with respect to EVM.

This restriction serves to avoid weighted averaging in F.8.

# F.8	Averaged EVM

EVM is averaged over all allocated downlink resource blocks with the considered modulation scheme in the frequency domain, and a minimum of 10 downlink slots:

For FDD the averaging in the time domain equals the 10] subframeslots duration of the 10 slots measurement period from the equalizer estimation step.

For TDD the averaging in the time domain can be calculated from slots of different frames and should have a minimum of 10 slots averaging length. TDD special fields (i.e. GP) are not included in the averaging.

Where Ni is the number of resource blocks with the considered modulation scheme in slots i and Ndl is the number of allocated downlink slots in one frame.

The EVM requirements shall be tested against the maximum of the RMS average at the window W extremities of the EVM measurements:

Thus   is calculated using in the expressions above and is calculated using  in the  calculation.

Thus we get:

The averaged EVM with the minimum averaging length of at least 10 slots is then achieved by further averaging of the  results

,

## F.8.1 Averaged EVM (TDD)

For TDD the averaging in the time domain can be calculated from slots of different frames and should have a minimum of 10 slots averaging length. TDD special fields are not included in the averaging.

frame is derived by: Square the EVM results in a frame. Relevant for EVM are subframesslots in a frame, which are active in the DL, Ndl. Within these slots, those RBs are relevant, that carry the maximum number of PDSCH REs (same as FDD). Sum the squares, divide the sum by the number of EVM relevant locations, square-root the quotient. (RMS)

The EVMframe is calculated, using the maximum of  frame at the window W extremities. Thus   is calculated using  and is calculated using . (l and h, low and high. Where l is the timing ΔC –W/2 and and high is the timing ΔC +W/2)

In order to unite at least 10 slots, consider the minimum integer number of radio frames, containing at least 10 EVM relevant slots or 20 l. Unite by RMS.

,

The result, , is compared against the limit.

<End of modified section>