3GPP TSG RAN WG4 Meeting #90bis	R4-1902811

Xian, China, 8th– 12thh April, 2019

3GPP TSG-RAN WG2 #105	 R2-1902732

Athens, Greece, February 25th-March 1st, 2019

Title:	LS on capability of measurement gap patterns

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN2

To:	RAN4

Cc:

Contact Person:

#### Name:	Chun-Fan (Felix), Tsai

E-mail Address:	Chun-Fan.Tsai (at) mediaTek.com

1. Overall Description

RAN2 would like to inform RAN4 that RAN2 has discussed the capability signalling of measurement gap patterns.

RAN2 agreed that current RRC signaling for gap pattern capabilities as following:

- Gap patterns 0 and 1 are mandatory for all cases.

- In LTE SA

  - LTE RRC signalling shortMeasurementGap is used to signal whether UE supports gap patterns 2 and 3.

  - LTE RRC signalling measGapPatterns (8 bits) is used to signal whether UE supports gap patterns 4 to 11.

- In EN-DC

  - LTE RRC signalling shortMeasurementGap is used to signal whether UE supports gap patterns 2 and 3 (for per-UE cap and FR1 gap of per-FR gap).

  - LTE RRC signalling measGapPatterns is used to signal whether UE supports gap patterns 4 to 11 (for per-UE cap and FR1 gap of per-FR gap).

  - NR RRC signalling supportedGapPattern (22 bits) is used to signal whether UE supports gap patterns 2 to 23 (for FR2 gap of per-FR gap).

- In NR SA

  - NR RRC signalling supportedGapPattern (22 bits) is used to signal whether UE supports gap patterns 2 to 23.

- If the UE supports an optional gap pattern (#2 to #23), it supports using the gap pattern according to the corresponding applicability defined in RAN4 specifications.

RAN2 would like to confirm with RAN4 that the above agreements are consistent with TS 36.133 and TS 38.133.

2. Actions:

ACTION: 	RAN2 respectfully asks RAN4 to take above RAN2 agreements into consideration and check if they are consistent with TS 36.133 and TS 38.133.

3. Date of Next RAN2 Meetings:

RAN2#105Bis, Xi'an, 8-12 Apr 2018

RAN2#106, Reno, 13-17 May 2019