3GPP TSG-RAN WG4 Meeting #90bis	R4-1902875

# Xi’an, China, April 8 – April 12, 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | CRNum | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to optimize EN-DC configuration definition in TS 38.101-3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  |  | Date: |  | 2019-04-08 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | No additional information is provided with E-UTRA Band and NR Band columns. In RAN4#90 was agreed that at least two right columns are deleted from the configuration table (E-UTRA Band and NR Band) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed E-UTRA Band and NR Band from subsection 5.2B Operating bands for DC |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unnecessary information remains in the subsection for operating band for DC |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2B |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

< start of changes >

## 5.2B Operating bands for DC

### 5.2B.1	General

The operating bands are specified for operation with EN-DC or NGEN-DC, NR-DC configured. The EN-DC or NGEN-DC band combinations include at least one E-UTRA operating band.

For EN-DC configurations indicated by column “Single Uplink allowed” (e.g., problematic band combinations as defined in TS38.306) in tables in this section the UE may indicate capability of not supporting simultaneous dual and triple uplink operation due to possible intermodulation interference to its own primary downlink channel bandwidth if the intermodulation order is 2 or if the intermodulation order is 3 for the combinations when both operating bands are between 450 MHz – 960 MHz or between 1427 MHz – 2690 MHz. In case for the EN-DC configurations listed in tables in this section for which the intermodulation products caused by the dual and triple uplink operation fall into the receive band but do not interfere with the own primary downlink channel bandwidth as defined in Annex-I the UE is mandated to operate in dual and triple uplink mode. Single Uplink is also allowed for certain band combinations where intermodulation or reverse intermodulation products could create difficulty for meeting emission requirements.

For EN-DC combinations of order 3 or higher, “Single Uplink allowed” UL configurations captured in Table 5.2B.2.1-1, Table 5.2B.3.1-1, Table 5.2B.4.1-1 apply.

### 5.2B.2	Intra-band contiguous EN-DC

#### 5.2B.2.1	EN-DC

Table 5.2B.2.1-1: Band combinations for intra-band contiguous EN-DC

| EN-DC band |  |  | Single UL allowed |
| --- | --- | --- | --- |
| DC_(n)71 |  |  | No3 |
| DC_(n)414 |  |  | Yes1 |
| NOTE 1: Single UL allowed due to potential emission issues, not self-interference. NOTE 2: The minimum requirements apply for 15 kHz subcarrier spacing on the SCG. NOTE 3: For UE(s) supporting dynamic power sharing it is mandatory to do dual simultaneous UL. For UE(s) not supporting dynamic power sharing single UL is allowed. NOTE 4: The minimum requirements only apply for non-simultaneous Tx/Rx between all carriers. |  |  |  |

### 5.2B.3	Intra-band non-contiguous EN-DC

#### 5.2B.3.1	EN-DC

Table 5.2B.3.1-1: Band combinations for intra-band non-contiguous EN-DC

| EN-DC Band Uplink Combination |  |  | Single UL allowed |
| --- | --- | --- | --- |
| DC_3_n3 |  |  | Yes1 |
| DC_41_n412 |  |  | Yes |
| NOTE 1: Only single switched UL is supported in Rel.15 NOTE 2: The minimum requirements only apply for non-simultaneous Tx/Rx between all carriers. |  |  |  |

#### 5.2B.3.2	Void

### 5.2B.4	Inter-band EN-DC within FR1

#### 5.2B.4.1	EN-DC (two bands)

Table 5.2B.4.1-1: Band combinations for inter-band EN-DC within FR1 (two bands)

| EN-DC band |  |  | Single UL allowed |
| --- | --- | --- | --- |
| DC_1_n28 |  |  | No |
| DC_1_n40 |  |  | No |
| DC_1_n51 |  |  | No |
| DC_1_n773 |  |  | DC_1_n77 |
| DC_1_n783 |  |  | No |
| DC_1_n793 |  |  | No |
| DC_2_n5 |  |  | No |
| DC_2_n66 |  |  | DC_2_n66 |
| DC_2_n71 |  |  | No |
| DC_2_n78 |  |  | DC_2_n78 |
| DC_3_n7 |  |  | No |
| DC_3_n28 |  |  | No |
| DC_3_n40 |  |  | No |
| DC_3_n51 |  |  | No |
| DC_3_n773 |  |  | DC_3_n77 |
| DC_3_n783 |  |  | DC_3_n78 |
| DC_3_n793 |  |  | No |
| DC_5_n40 |  |  | No |
| DC_5_n66 |  |  | DC_5_n66 |
| DC_5_n783 |  |  | No |
| DC_7_n28 |  |  | No |
| DC_7_n51 |  |  | No |
| DC_7_n783 |  |  | No |
| DC_7-7_n783 |  |  | No |
| DC_8_n40 |  |  | No |
| DC_8_n773 |  |  | No |
| DC_8_n783 |  |  | No |
| DC_8_n793 |  |  | No |
| DC_11_n773 |  |  | No |
| DC_11_n783 |  |  | No |
| DC_11_n793 |  |  | No |
| DC_12_n5 |  |  | No |
| DC_12_n66 |  |  | No |
| DC_18_n773 |  |  | No |
| DC_18_n783 |  |  | No |
| DC_18_n793 |  |  | No |
| DC_19_n773 |  |  | No |
| DC_19_n783 |  |  | No |
| DC_19_n793 |  |  | No |
| DC_20_n8 |  |  | DC_20_n8 |
| DC_20_n284 |  |  | No |
| DC_20_n51 |  |  | No |
| DC_20_n77 |  |  | No |
| DC_20_n783 |  |  | No |
| DC_21_n773 |  |  | No |
| DC_21_n783 |  |  | No |
| DC_21_n793 |  |  | No |
| DC_25_n41 |  |  | No |
| DC_26_n413 |  |  | No |
| DC_26_n773 |  |  | No |
| DC_26_n783 |  |  | No |
| DC_26_n793 |  |  | No |
| DC_28_n51 |  |  | No |
| DC_28_n773 |  |  | No |
| DC_28_n783 |  |  | No |
| DC_28_n793 |  |  | No |
| DC_30_n5 |  |  | No |
| DC_30_n66 |  |  | No |
| DC_38_n78 |  |  | No |
| DC_39_n781,3 |  |  | No |
| DC_39_n793 |  |  | No |
| DC_40_n77 |  |  | No |
| DC_41_n77 |  |  | No |
| DC_41_n78 |  |  | No |
| DC_41_n792,3 |  |  | No |
| DC_42_n51 |  |  | No |
| DC_42_n775 |  |  | N/A |
| DC_42_n785 |  |  | N/A |
| DC_42_n795 |  |  | N/A |
| DC_66_n71 |  |  | No |
| DC_66_n5 |  |  | DC_66_n5 |
| DC_66_n78 |  |  | No |
| NOTE 1: The frequency range above 3600 MHz for Band n78 is not used in this combination. NOTE 2: The frequency range below 2506 MHz for Band 41 is not used in this combination. NOTE 3: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability. NOTE 4: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. NOTE 5: The combination is not used alone as fall back mode of other band combinations in which UL in Band 42 is not used. |  |  |  |

#### 5.2B.4.2	EN-DC (three bands)

Table 5.2B.4.2-1: Band combinations for inter-band EN-DC within FR1 (three bands)

| EN-DC Band |  |  |  |
| --- | --- | --- | --- |
| DC_1-3_n28 |  |  |  |
| DC_1-3_n772 |  |  |  |
| DC_1-3_n782 |  |  |  |
| DC_1-3_n792 |  |  |  |
| DC_1-5_n782 |  |  |  |
| DC_1-7_n282 |  |  |  |
| DC_1-7_n78 2 |  |  |  |
| DC_1-7-7_n782 |  |  |  |
| DC_1-8_n782 |  |  |  |
| DC_1-18_n772 |  |  |  |
| DC_1-18_n782 |  |  |  |
| DC_1-18_n79 |  |  |  |
| DC_1-19_n772 |  |  |  |
| DC_1-19_n782 |  |  |  |
| DC_1-19_n792 |  |  |  |
| DC_1-20_n283 |  |  |  |
| DC_1-20_n782 |  |  |  |
| DC_1-21_n772 |  |  |  |
| DC_1-21_n782 |  |  |  |
| DC_1-21_n792 |  |  |  |
| DC_1-28_n772 |  |  |  |
| DC_1-28_n782 |  |  |  |
| DC_1-28_n79 |  |  |  |
| DC_1_n28-n782 |  |  |  |
| DC_1_n77-n79 |  |  |  |
| DC_1_n78-n79 |  |  |  |
| DC_1-41_n77 |  |  |  |
| DC_1-41_n78 |  |  |  |
| DC_1-41_n79 |  |  |  |
| DC_1-42_n77 |  |  |  |
| DC_1-42_n78 |  |  |  |
| DC_1-42_n79 |  |  |  |
| DC_1_SUL_n78-n842 |  |  |  |
| DC_2-5_n66 |  |  |  |
| DC_2-12_n66 |  |  |  |
| DC_2-30_n66 |  |  |  |
| DC_2-(n)71 |  |  |  |
| DC_2-66_n71 |  |  |  |
| DC_3_n3-n77 |  |  |  |
| DC_3_n3-n78 |  |  |  |
| DC_3-5_n782 |  |  |  |
| DC_3-7_n28 |  |  |  |
| DC_3-7_n782 |  |  |  |
| DC_3-7-7_n782 |  |  |  |
| DC_3-8_n78 |  |  |  |
| DC_3-19_n772 |  |  |  |
| DC_3-19_n782 |  |  |  |
| DC_3-19_n792 |  |  |  |
| DC_3-20_n282,3 |  |  |  |
| DC_3-20_n782 |  |  |  |
| DC_3-21_n772 |  |  |  |
| DC_3-21_n782 |  |  |  |
| DC_3-21_n792 |  |  |  |
| DC_3-28_n782 |  |  |  |
| DC_3_n28-n782 |  |  |  |
| DC_3-28_n79 |  |  |  |
| DC_3-38_n78 |  |  |  |
| DC_3-41_n78 |  |  |  |
| DC_3-42_n77 |  |  |  |
| DC_3-42_n78 |  |  |  |
| DC_3-42_n79 |  |  |  |
| DC_3_n77-n79 |  |  |  |
| DC_3_n78-n79 |  |  |  |
| DC_3_SUL_n78-n802 |  |  |  |
| DC_3_SUL_n78-n822 |  |  |  |
| DC_3_SUL_n79-n802 |  |  |  |
| DC_5-7-7_n78 |  |  |  |
| DC_5-7_n78 |  |  |  |
| DC_5-30_n66 |  |  |  |
| DC_7-20_n283 |  |  |  |
| DC_7-20_n782 |  |  |  |
| DC_7-28_n782 |  |  |  |
| DC_7_n28-n782 |  |  |  |
| DC_7-46_n78 |  |  |  |
| DC_8_SUL_n78-n812 |  |  |  |
| DC_8_SUL_n79-n812 |  |  |  |
| DC_12-30_n66 |  |  |  |
| DC_18-28_n772 |  |  |  |
| DC_18-28_n782 |  |  |  |
| DC_18-28_n792 |  |  |  |
| DC_19-21_n772 |  |  |  |
| DC_19-21_n782 |  |  |  |
| DC_19-21_n792 |  |  |  |
| DC_19-42_n77 |  |  |  |
| DC_19-42_n78 |  |  |  |
| DC_19-42_n79 |  |  |  |
| DC_19_n77-n79 |  |  |  |
| DC_19_n78-n79 |  |  |  |
| DC_20_n8-n75 |  |  |  |
| DC_20_n28-n753 |  |  |  |
| DC_20_n28-n782,3 |  |  |  |
| DC_20_n75-n782 |  |  |  |
| DC_20_n76-n782 |  |  |  |
| DC_20_SUL_n78-n822 |  |  |  |
| DC_20_SUL_n78-n832 |  |  |  |
| DC_21-42_n77 |  |  |  |
| DC_21-42_n78 |  |  |  |
| DC_21-42_n79 |  |  |  |
| DC_21_n77-n79 |  |  |  |
| DC_21_n78-n79 |  |  |  |
| DC_28-42_n77 |  |  |  |
| DC_28-42_n78 |  |  |  |
| DC_28-42_n79 |  |  |  |
| DC_41-42_n77 |  |  |  |
| DC_41-42_n78 |  |  |  |
| DC_41-42_n79 |  |  |  |
| DC_28_SUL_n78-n832 |  |  |  |
| DC_66_(n)71 |  |  |  |
| DC_66_SUL_n78-n862 |  |  |  |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier can be up to 140us and placed in SUL resources. NOTE 2: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 3: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. |  |  |  |

#### 5.2B.4.3	EN-DC (four bands)

Table 5.2B.4.3-1: Band combinations for inter-band EN-DC within FR1 (four bands)

| EN-DC Band |  |  |  |
| --- | --- | --- | --- |
| DC_1-3-5_n781 |  |  |  |
| DC_1-3-7_n28 |  |  |  |
| DC_1-3-7-7_n781 |  |  |  |
| DC_1-3-7_n781 |  |  |  |
| DC_1-3-8_n781 |  |  |  |
| DC_1-3-28_n771 |  |  |  |
| DC_1-3-28_n781 |  |  |  |
| DC_1-3_n28-n781 |  |  |  |
| DC_1-3-28_n791 |  |  |  |
| DC_1-3-19_n771 |  |  |  |
| DC_1-3-19_n781 |  |  |  |
| DC_1-3-19_n791 |  |  |  |
| DC_1-3-20_n282 |  |  |  |
| DC_1-3-20_n781 |  |  |  |
| DC_1-3-21_n771 |  |  |  |
| DC_1-3-21_n781 |  |  |  |
| DC_1-3-21_n791 |  |  |  |
| DC_1-3-42_n77 |  |  |  |
| DC_1-3-42_n78 |  |  |  |
| DC_1-3-42_n79 |  |  |  |
| DC_1-5-7_n78 |  |  |  |
| DC_1-5-7-7_n78 |  |  |  |
| DC_1-7-20_n282 |  |  |  |
| DC_1-7-20_n781 |  |  |  |
| DC_1-7_n28-n781 |  |  |  |
| DC_1-18-28_n77 |  |  |  |
| DC_1-18-28_n78 |  |  |  |
| DC_1-18-28_n791 |  |  |  |
| DC_1-19-21_n77 |  |  |  |
| DC_1-19-21_n78 |  |  |  |
| DC_1-19-21_n79 |  |  |  |
| DC_1-19-42_n77 |  |  |  |
| DC_1-19-42_n78 |  |  |  |
| DC_1-19-42_n79 |  |  |  |
| DC_1-20_n28-n781,2 |  |  |  |
| DC_1-21-28_n771 |  |  |  |
| DC_1-21-28_n781 |  |  |  |
| DC_1-21-28_n791 |  |  |  |
| DC_1-21-42_n77 |  |  |  |
| DC_1-21-42_n78 |  |  |  |
| DC_1-21-42_n79 |  |  |  |
| DC_1-28-42_n77 |  |  |  |
| DC_1-28-42_n78 |  |  |  |
| DC_1-28-42_n79 |  |  |  |
| DC_1-41-42_n77 |  |  |  |
| DC_1-41-42_n78 |  |  |  |
| DC_1-41-42-n79 |  |  |  |
| DC_2-66-(n)71 |  |  |  |
| DC_3-5-7_n78 |  |  |  |
| DC_3-5-7-7_n78 |  |  |  |
| DC_3-7-20_n282 |  |  |  |
| DC_3-7-20_n781 |  |  |  |
| DC_3-7-28_n781 |  |  |  |
| DC_3-7_n28-n781 |  |  |  |
| DC_3-19-21_n771 |  |  |  |
| DC_3-19-21_n781 |  |  |  |
| DC_3-19-21_n791 |  |  |  |
| DC_3-19-42_n77 |  |  |  |
| DC_3-19-42_n78 |  |  |  |
| DC_3-19-42_n791 |  |  |  |
| DC_3-20_n28-n781,2 |  |  |  |
| DC_3-21-42_n77 |  |  |  |
| DC_3-21-42_n78 |  |  |  |
| DC_3-21-42_n79 |  |  |  |
| DC_3-28-42_n77 |  |  |  |
| DC_3-28-42_n78 |  |  |  |
| DC_3-28-42_n79 |  |  |  |
| DC_7-20_n28-n781,2 |  |  |  |
| DC_19-21-42_n77 |  |  |  |
| DC_19-21-42_n78 |  |  |  |
| DC_19-21-42_n79 |  |  |  |
| DC_21-28-42_n77 |  |  |  |
| DC_21-28-42_n78 |  |  |  |
| DC_21-28-42_n79 |  |  |  |
| NOTE 1: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 2: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. |  |  |  |

#### 5.2B.4.4	EN-DC (five bands)

Table 5.2B.4.4-1: Band combinations inter-band EN-DC within FR1 (five bands)

| EN-DC Band |  |  |  |
| --- | --- | --- | --- |
| DC_1-3-5-7_n78 |  |  |  |
| DC_1-3-5-7-7_n78 |  |  |  |
| DC_1-3-7-20_n282 |  |  |  |
| DC_1-3-7-20_n781 |  |  |  |
| DC_1-3-7_n28-n781 |  |  |  |
| DC_1-3-19-21_n771 |  |  |  |
| DC_1-3-19-21_n781 |  |  |  |
| DC_1-3-19-21_n791 |  |  |  |
| DC_1-3-19-42_n77 |  |  |  |
| DC_1-3-19-42_n78 |  |  |  |
| DC_1-3-19-42_n79 |  |  |  |
| DC_1-3-20_n28-n781,2 |  |  |  |
| DC_1-3-21-42_n77 |  |  |  |
| DC_1-3-21-42_n78 |  |  |  |
| DC_1-3-21-42_n79 |  |  |  |
| DC_1-7-20_n28-n781,2 |  |  |  |
| DC_1-19-21-42_n77 |  |  |  |
| DC_1-19-21-42_n78 |  |  |  |
| DC_1-19-21-42_n79 |  |  |  |
| DC_1-3-5-7_n78 |  |  |  |
| DC_1-3-28-42_n77 |  |  |  |
| DC_1-3-28-42_n78 |  |  |  |
| DC_1-3-28-42_n79 |  |  |  |
| DC_1-21-28-42_n77 |  |  |  |
| DC_1-21-28-42_n78 |  |  |  |
| DC_1-21-28-42_n79 |  |  |  |
| DC_3-7-20_n28-n781,2 |  |  |  |
| NOTE 1: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 2: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL |  |  |  |

#### 5.2B.4.5	EN-DC (six bands)

Table 5.2B.4.5-1: Band combinations for inter-band EN-DC within FR1 (six bands)

| EN-DC Band |  |  |  |
| --- | --- | --- | --- |
| DC_1-3-7-20_n28-n781,2 |  |  |  |
| NOTE 1: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 2: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL |  |  |  |

### 5.2B.5	Inter-band EN-DC including FR2

#### 5.2B.5.1	EN-DC (two bands)

Table 5.2B.5.1-1: Band combinations for inter-band EN-DC including FR2 (two bands)

| EN-DC Band |  |  |  |
| --- | --- | --- | --- |
| DC_1_n257 |  |  |  |
| DC_2-2_n257 |  |  |  |
| DC_2_n257 |  |  |  |
| DC_2_n260 |  |  |  |
| DC_2_n260 |  |  |  |
| DC_2-2_n260 |  |  |  |
| DC_3_n257 |  |  |  |
| DC_3_n258 |  |  |  |
| DC_5_n257 |  |  |  |
| DC_5-5_n257 |  |  |  |
| DC_5-5_n260 |  |  |  |
| DC_5_n260 |  |  |  |
| DC_5_n261 |  |  |  |
| DC_7-7_n257 |  |  |  |
| DC_7_n257 |  |  |  |
| DC_7_n258 |  |  |  |
| DC_8_n257 |  |  |  |
| DC_8_n258 |  |  |  |
| DC_11_n257 |  |  |  |
| DC_12_n260 |  |  |  |
| DC_13_n257 |  |  |  |
| DC_13_n260 |  |  |  |
| DC_18_n257 |  |  |  |
| DC_19_n257 |  |  |  |
| DC_20_n258 |  |  |  |
| DC_21_n257 |  |  |  |
| DC_26_n257 |  |  |  |
| DC_28_n257 |  |  |  |
| DC_28_n258 |  |  |  |
| DC_30_n260 |  |  |  |
| DC_39_n258 |  |  |  |
| DC_41_n257 |  |  |  |
| DC_41_n258 |  |  |  |
| DC_42_n257 |  |  |  |
| DC_48-48_n257 |  |  |  |
| DC_48_n257 |  |  |  |
| DC_48-48_n260 |  |  |  |
| DC_48_n260 |  |  |  |
| DC_66-66_n257 |  |  |  |
| DC_66_n257 |  |  |  |
| DC_66-66_n260 |  |  |  |
| DC_66_n260 |  |  |  |
| DC_66_n261 |  |  |  |
| NOTE 1: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability for all of the above combinations |  |  |  |

#### 5.2B.5.2	EN-DC (three bands)

Table 5.2B.5.2-1: Band combinations inter-band EN-DC including FR2 (three bands)

| EN-DC Band |  |  |  |
| --- | --- | --- | --- |
| DC_1-3_n2571 |  |  |  |
| DC_1-5_n2571 |  |  |  |
| DC_1-7_n2571 |  |  |  |
| DC_1-7-7_n2571 |  |  |  |
| DC_1-8_n257 |  |  |  |
| DC_1-18_n2571 |  |  |  |
| DC_1-19_n2571 |  |  |  |
| DC_1-21_n2571 |  |  |  |
| DC_1-28_n2571 |  |  |  |
| DC_1-41_n257 |  |  |  |
| DC_1-42_n257 |  |  |  |
| DC_2-5_n2571 |  |  |  |
| DC_2-5_n260 |  |  |  |
| DC_2-12_n260 |  |  |  |
| DC_2-13_n2571 |  |  |  |
| DC_2-13_n2601 |  |  |  |
| DC_2-30_n260 |  |  |  |
| DC_2-66_n2571 |  |  |  |
| DC_2-66_n260 |  |  |  |
| DC_3-5_n2571 |  |  |  |
| DC_3-7_n2571 |  |  |  |
| DC_3-7-7_n2571 |  |  |  |
| DC_3-19_n2571 |  |  |  |
| DC_3-21_n2571 |  |  |  |
| DC_3-28_n2571 |  |  |  |
| DC_3-41_n257 |  |  |  |
| DC_3-42_n2571 |  |  |  |
| DC_5-7-7_n2571 |  |  |  |
| DC_5-7_n2571 |  |  |  |
| DC_5-30_n260 |  |  |  |
| DC_5-66_n260 |  |  |  |
| DC_12-30_n260 |  |  |  |
| DC_12-66_n260 |  |  |  |
| DC_13-66_n2571 |  |  |  |
| DC_13-66_n2601 |  |  |  |
| DC_18-28_n2571 |  |  |  |
| DC_19-21_n2571 |  |  |  |
| DC_19-42_n2571 |  |  |  |
| DC_21-42_n2571 |  |  |  |
| DC_21-28_n2571 |  |  |  |
| DC_28-42_n2571 |  |  |  |
| DC_30-66_n260 |  |  |  |
| DC_41-42_n257 |  |  |  |
| NOTE 1: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability |  |  |  |

#### 5.2B.5.3	EN-DC (four bands)

Table 5.2B.5.3-1: Band combinations inter-band EN-DC including FR2 (four bands)

| EN-DC Band |  |  |  |
| --- | --- | --- | --- |
| DC_1-3-5_n2571 |  |  |  |
| DC_1-3-7_n2571 |  |  |  |
| DC_1-3-7-7_n257 |  |  |  |
| DC_1-3-19_n2571 |  |  |  |
| DC_1-3-21_n2571 |  |  |  |
| DC_1-3-28_n2571 |  |  |  |
| DC_1-3-42_n257 |  |  |  |
| DC_1-5-7_n2571 |  |  |  |
| DC_1-5-7-7_n257 |  |  |  |
| DC_1-18-28_n2571 |  |  |  |
| DC_1-19-21_n257 |  |  |  |
| DC_1-19-42_n257 |  |  |  |
| DC_1-21-28_n2571 |  |  |  |
| DC_1-21-42_n257 |  |  |  |
| DC_1-28-42_n257 |  |  |  |
| DC_1-41-42_n257 |  |  |  |
| DC_3-5-7-7_n257 |  |  |  |
| DC_3-5-7_n2571 |  |  |  |
| DC_3-19-21_n2571 |  |  |  |
| DC_3-19-42_n257 |  |  |  |
| DC_3-21-42_n257 |  |  |  |
| DC_3-28-42_n257 |  |  |  |
| DC_19-21-42_n2571 |  |  |  |
| DC_21-28-42_n2571 |  |  |  |
| NOTE 1: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability |  |  |  |

#### 5.2B.5.4	EN-DC (five bands)

Table 5.2B.5.4-1: Band combinations inter-band EN-DC including FR2 (five bands)

| EN-DC Band |  |  |  |
| --- | --- | --- | --- |
| DC_1-3-5-7_n2571 |  |  |  |
| DC_1-3-5-7-7_n2571 |  |  |  |
| DC_1-3-19-21_n2571 |  |  |  |
| DC_1-3-19-42_n257 |  |  |  |
| DC_1-3-21-42_n257 |  |  |  |
| DC_1-3-28-42_n257 |  |  |  |
| DC_1-19-21-42_n257 |  |  |  |
| DC_1-21-28-42_n257 |  |  |  |
| NOTE 1: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability |  |  |  |

### 5.2B.6	Inter-band EN-DC including both FR1 and FR2

#### 5.2B.6.1	Void

#### 5.2B.6.2	EN-DC (three bands)

Table 5.2B.6.2-1: Band combinations inter-band EN-DC including both FR1 and FR2 (three bands)

| EN-DC Band |  |  |  |
| --- | --- | --- | --- |
| DC_1_n77-n257 |  |  |  |
| DC_1_n78-n257 |  |  |  |
| DC_1_n79-n257 |  |  |  |
| DC_3_n77-n257 |  |  |  |
| DC_3_n78-n257 |  |  |  |
| DC_3_n79-n257 |  |  |  |
| DC_5_n78-n2571 |  |  |  |
| DC_7-7_n78-n257 |  |  |  |
| DC_7_n78-n257 |  |  |  |
| DC_19_n77-n257 |  |  |  |
| DC_19_n78-n257 |  |  |  |
| DC_19_n79-n257 |  |  |  |
| DC_21_n77-n257 |  |  |  |
| DC_21_n78-n257 |  |  |  |
| DC_21_n79-n257 |  |  |  |
| NOTE 1: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability |  |  |  |

#### 5.2B.6.3	EN-DC (four bands)

Table 5.2B.6.3-1: Band combinations inter-band EN-DC including both FR1 and FR2 (four bands)

| EN-DC Band |  |  |  |
| --- | --- | --- | --- |
| DC_1-3_n78-n257 |  |  |  |
| DC_1-5_n78-n257 |  |  |  |
| DC_1-7-7_n78-n257 |  |  |  |
| DC_1-7_n78-n257 |  |  |  |
| DC_3-5_n78-n257 |  |  |  |
| DC_3-7-7_n78-n257 |  |  |  |
| DC_3-7_n78-n257 |  |  |  |
| DC_5-7-7_n78-n257 |  |  |  |
| DC_5-7_n78-n257 |  |  |  |

#### 5.2B.6.4	EN-DC (five bands)

Table 5.2B.6.4-1: Band combinations inter-band EN-DC including both FR1 and FR2 (five bands)

| EN-DC Band |  |  |
| --- | --- | --- |
| DC_1-3-5_n78-n257 |  |  |
| DC_1-3-7-7_n78-n257 |  |  |
| DC_1-3-7_n78-n257 |  |  |
| DC_1-5-7-7_n78-n257 |  |  |
| DC_1-5-7_n78-n257 |  |  |
| DC_3-5-7-7_n78-n257 |  |  |
| DC_3-5-7_n78-n257 |  |  |

#### 5.2B.6.5	EN-DC (six bands)

Table 5.2B.6.5-1: Band combinations inter-band EN-DC including both FR1 and FR2 (six bands)

| EN-DC Band |  |  |
| --- | --- | --- |
| DC_1-3-5-7_n78-n257 |  |  |
| NOTE 1: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability |  |  |

### 5.2B.7	Inter-band NR-DC between FR1 and FR2

#### 5.2B.7.1	NR-DC (two bands)

Table 5.2B.7.1-1: Band combinations inter-band NR-DC between FR1 and FR2 (two bands)

| NR-DC Band |  |
| --- | --- |
| DC_n77-n257 |  |
| DC_n78-n257 |  |
| DC_n79-n257 |  |

< End of changes >