3GPP TSG-RAN4 Meeting #90-Bis	R4-1902929

# Xi'an, China, 8 - 12 Apr 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | CRNum | rev | - | Current version: | V15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.101-2 PC2/PC4 MPR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT_Core |  |  |  |  |  | Date: |  |  | 2019-04-8 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | PC2/PC4 MPR should follow PC3 MPR. For small RB allocations, the limiting factor is the 2nd harmonic. MPRnarrow should be defined for both inner and outer small RB allocations. |  |  |  |  |  |  |  |  |
| Summary of change: |  | PC2/PC4 MPR is updated to follow PC3 MPR. For PC2/PC4 MPR, MPRnarrow = 2.5 dB, when the allocated RB size is less than or equal to 1.44 MHz and MPRnarrow = 1.0 dB, when the allocated RB size is greater than 1.44 MHz and less than or equal to 3.6 MHz. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PC2/PC4 devices may not meet RF requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2.2, 6.2.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-2 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

< start of changes >

#### 6.2.2.2	UE maximum output power reduction for power class 2

For power class 2, MPR for contiguous allocations is defined as:

MPR = max(MPRWT, MPRnarrow)

Where,

-	MPRnarrow = 2.5 dB, when the allocated RB size is less than or equal to 1.44MHz and MPRnarrow = 1.0 dB, when the allocated RB size is greater than 1.44 MHz and less than or equal to 3.6 MHz.

MPRWT is the maximum power reduction due to modulation orders, transmission bandwidth configurations listed in table 5.3.2-1, and waveform types. MPRWT is defined in Table 6.2.2.2-1 and Table 6.2.2.2-2.

Table 6.2.2.2-1 MPRWT for power class 2, BWchannel ≤ 200 MHz

|  |  | MPRWT, BWchannel ≤ 200 MHz |  |
| --- | --- | --- | --- |
|  |  | RBstart ≥ Ceil(1/3 NRB) AND RBend ≤ Ceil(2/3 NRB) | RBstart ≥ Ceil(1/3 NRB) AND RBend ≤ Ceil(2/3 NRB) |
| DFT-s-OFDM | Pi/2 BPSK | 0.0 | 2.0 |
|  | QPSK | 0.0 | 2.0 |
|  | 16QAM | 3.0 | 3.5 |
|  | 64QAM | 5.0 | 5.5 |
| CP-OFDM | QPSK | 3.5 | 4.0 |
|  | 16QAM | 5.0 | 5.0 |
|  | 64QAM | 7.5 | 7.5 |

Table 6.2.2.2-2 MPRWT for power class 2, BWchannel = 400 MHz

|  |  | MPRWT, BWchannel = 400 MHz |  |
| --- | --- | --- | --- |
|  |  | RBstart ≥ Ceil(1/4 NRB) AND RBend ≤ Ceil(3/4 NRB) AND LCRB≤Ceil(1/4 NRB) | RBstart < Ceil(1/4 NRB) OR RBend > Ceil(3/4 NRB) OR LCRB>Ceil(1/4 NRB) |
| DFT-s-OFDM | Pi/2 BPSK | 0.0 | 3.0 |
|  | QPSK | 0.0 | 3.0 |
|  | 16QAM | 4.5 | 4.5 |
|  | 64QAM | 6.5 | 6.5 |
| CP-OFDM | QPSK | 5.0 | 5.0 |
|  | 16QAM | 6.5 | 6.5 |
|  | 64QAM | 9.0 | 9.0 |

< end of changes

< start of changes >

#### 6.2.2.4	UE maximum output power reduction for power class 4

For power class 4, MPR for contiguous allocations is defined as:

MPR = max(MPRWT, MPRnarrow)

Where,

MPRnarrow = 2.5 dB, when the allocated RB size is less than or equal to 1.44 MHz and MPRnarrow = 1.0 dB, when the allocated RB size is greater than 1.44 MHz and less than or equal to 3.6 MHz.

MPRWT is the maximum power reduction due to modulation orders, transmission bandwidth configurations listed in Table 5.3.2-1, and waveform types. MPRWT is defined in Table 6.2.2.4-1 and Table 6.2.2.4-2.

Table 6.2.2.4-1 MPRWT for power class 4, BWchannel ≤ 200 MHz

|  |  | MPRWT, BWchannel ≤ 200 MHz |  |
| --- | --- | --- | --- |
|  |  | RBstart ≥ Ceil(1/3 NRB) AND RBend ≤ Ceil(2/3 NRB) | RBstart ≥ Ceil(1/3 NRB) AND RBend ≤ Ceil(2/3 NRB) |
| DFT-s-OFDM | Pi/2 BPSK | 0.0 | 2.0 |
|  | QPSK | 0.0 | 2.0 |
|  | 16QAM | 3.0 | 3.5 |
|  | 64QAM | 5.0 | 5.5 |
| CP-OFDM | QPSK | 3.5 | 4.0 |
|  | 16QAM | 5.0 | 5.0 |
|  | 64QAM | 7.5 | 7.5 |

Table 6.2.2.4-2 MPRWT for power class 4, BWchannel = 400 MHz

|  |  | MPRWT, BWchannel = 400 MHz |  |
| --- | --- | --- | --- |
|  |  | RBstart ≥ Ceil(1/4 NRB) AND RBend ≤ Ceil(3/4 NRB) AND LCRB≤Ceil(1/4 NRB) | RBstart < Ceil(1/4 NRB) OR RBend > Ceil(3/4 NRB) OR LCRB>Ceil(1/4 NRB) |
| DFT-s-OFDM | Pi/2 BPSK | 0.0 | 3.0 |
|  | QPSK | 0.0 | 3.0 |
|  | 16QAM | 4.5 | 4.5 |
|  | 64QAM | 6.5 | 6.5 |
| CP-OFDM | QPSK | 5.0 | 5.0 |
|  | 16QAM | 6.5 | 6.5 |
|  | 64QAM | 9.0 | 9.0 |

< end of changes