3GPP TSG-RAN WG4 Meeting #90bis 	R4-1902946

Xi’an, China, 8 - 12 April, 2019

Agenda item:       6.12.1.3.1

Source:                Intel Corporation

Title:	                     NR PDCCH Demodulation Requirements

Document for:     Discussion

# Introduction

In RAN4#90 NR PDCCH UE demodulation requirements were discussed and way forward [1] was agreed. The agreements from the way forward are summarized below

- Test cases with 4Rx and AL=16 - Introducing FR1 15kHz test with 4Rx AL16 (test 14)- TDL-A ULA Medium A - MIMO correlation for FR1 30kHz test with 4Rx AL 16 (15b) - TDL-A, ULA Medium A - REG bundle size for test cases with 4Rx and interleaved mapping - using bundle size =2 (test 6a,7a adopted) - RAN4 will finalize the Rel-15 PDCCH demodulation requirements for test cases listed in pages 4 and 5 in RAN4#90bis - Requirements will not be updated after RAN4#90bis unless the technical issues remain

# Simulation Results

For the test cases agreed in [1], we provide alignment and impairments results for requirements definition for NR PDCCH demodulation.

## Alignment results with 2Rx

In this section we provide results for alignment as SNR at 1% Pm-dsg with 2Rx.

Table 1: PDCCH Alignment Results with 2Rx

| Test No. | SCS (KHz) | Format | CORESET RB | Payload | CORESET time duration | AL | CCE-to-REG Mapping | REG bundle size | CORESET-interleaver-size | Prop condition | Ant config | SNR @ 1% Pm-dsg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 15 | 1_0 | 24 | 39 | 2 | 2 | Non-interleaved | 6 | NA | TDLA30; 10Hz | 1x2 Low | 6.04 |
| 2 | 15 | 1_0 | 24 | 39 | 2 | 4 | Non-interleaved | 6 | NA | TDLC300; 100Hz | 2x2 Low | -0.72 |
| 3 | 15 | 1_1 | 24 | 52 | 2 | 2 | Non-interleaved | 6 | NA | TDLC300; 100Hz | 1x2 Low | 5.92 |
| 4 | 15 | 1_1 | 48 | 52 | 2 | 4 | Non-interleaved | 6 | NA | TDLA30; 10Hz | 1x2 Low | 3.66 |
| 5 | 15 | 1_1 | 48 | 52 | 2 | 8 | Non-interleaved | 6 | NA | TDLC300; 100Hz | 2x2 Low | -3.99 |
| 6 | 30 | 1_0 | 102 | 41 | 1 | 2 | Interleaved | 2 | 3 | TDLA30; 10Hz | 1x2 Low | 5.28 |
| 7 | 30 | 1_1 | 102 | 53 | 1 | 4 | Interleaved | 2 | 3 | TDLC300; 100Hz | 1x2 Low | 1.62 |
| 8 | 30 | 1_1 | 90 | 53 | 1 | 8 | Interleaved | 6 | 3 | TDLC300; 100Hz | 2x2 Low | -3.74 |
| 9 | 120 | 1_0 | 60 | 40 | 1 | 2 | Interleaved | 2 | 3 | TDL-A, 30ns, 75Hz | 1x2 Low | 4.26 |
| 10 | 120 | 1_1 | 60 | 56 | 1 | 4 | Interleaved | 6 | 2 | TDL-A, 30ns, 300Hz | 1x2 Low | 0.15 |
| 11 | 120 | 1_1 | 60 | 56 | 1 | 8 | Interleaved | 2 | 3 | TDL-A, 30ns, 75Hz | 2x2 Low | -1.90 |
| 12 | 15 | 1_0 | 48 | 39 | 1 | 4 | Non-interleaved | 6 | NA | TDLA30; 10Hz | 1x2 Low | 2.39 |
| 13 | 15 | 1_1 | 48 | 52 | 1 | 8 | Non-interleaved | 6 | NA | TDLA30; 10Hz | 2x2 Low | -2.32 |
| 14 | 15 | 1_0 | 48 | 39 | 2 | 16 | Non-interleaved | 6 | NA | TDLA30; 10Hz | 1x2 Low | -3.91 |
| 15 | 30 | 1_0 | 48 | 41 | 2 | 16 | Non-interleaved | 6 | NA | TDLC300; 100Hz | 1x2 Low | -6.63 |
| 16 | 120 | 1_0 | 60 | 40 | 2 | 16 | Interleaved | 2 | 3 | TDL-A, 30ns, 75Hz | 2x2 Low | -5.01 |

## Alignment results with 4Rx

In this section we provide results for alignment as SNR at 1% Pm-dsg with 4Rx.

Table 2: PDCCH Alignment Results with 4Rx

| Test No. | SCS (KHz) | Format | CORESET RB | Payload | CORESET time duration | AL | CCE-to-REG Mapping | REG bundle size | CORESET-interleaver-size | Prop condition | Ant config | SNR @ 1% Pm-dsg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 15 | 1_0 | 24 | 39 | 2 | 2 | Non-interleaved | 6 | NA | TDLA30; 10Hz | 1x4 Low | 1.02 |
| 2 | 15 | 1_0 | 24 | 39 | 2 | 4 | Non-interleaved | 6 | NA | TDLC300; 100Hz | 2x4 Low | -4.02 |
| 3 | 15 | 1_1 | 24 | 51 | 2 | 2 | Non-interleaved | 6 | NA | TDLC300; 100Hz | 1x4 Low | 1.25 |
| 4 | 15 | 1_1 | 48 | 51 | 2 | 4 | Non-interleaved | 6 | NA | TDLA30; 10Hz | 1x4 Low | -1.01 |
| 5 | 15 | 1_1 | 48 | 51 | 2 | 8 | Non-interleaved | 6 | NA | TDLC300; 100Hz | 2x4 Low | -6.50 |
| 6 | 30 | 1_0 | 102 | 41 | 1 | 2 | Interleaved | 2 | 3 | TDLA30; 10Hz | 1x4 Low | 0.46 |
| 7 | 30 | 1_1 | 102 | 53 | 1 | 4 | Interleaved | 2 | 3 | TDLC300; 100Hz | 1x4 Low | -2.39 |
| 8 | 30 | 1_1 | 90 | 53 | 1 | 8 | Interleaved | 6 | 3 | TDLC300; 100Hz | 2x4 Low | -5.72 |
| 12 | 15 | 1_0 | 48 | 39 | 1 | 4 | Non-interleaved | 6 | NA | TDLA30; 10Hz | 1x4 Low | -1.54 |
| 13 | 15 | 1_1 | 48 | 51 | 1 | 4 | Non-interleaved | 6 | NA | TDLA30; 10Hz | 2x4 Low | -2.22 |
| 14 | 15 | 1_0 | 48 | 39 | 2 | 16 | Non-interleaved | 6 | NA | TDLA30; 10Hz | 1x4 Med-A | -4.50 |
| 15 | 30 | 1_0 | 48 | 41 | 2 | 16 | Non-interleaved | 6 | NA | TDLA30; 10Hz | 1x4 Med-A | -4.77 |

## Impairment Results

For the agreed PDCCH test cases we provide the results with impairment margin added in the table below.

Table 3: Impairment results for PDCCH test cases

| Test No. | Ant config | SNR @ 1% Pm-dsg | Ant config | SNR @ 1% Pm-dsg |
| --- | --- | --- | --- | --- |
| 1 | 1x2 Low | 7.54 | 1x4 Low | 2.52 |
| 2 | 2x2 Low | 0.78 | 2x4 Low | -2.52 |
| 3 | 1x2 Low | 7.42 | 1x4 Low | 2.75 |
| 4 | 1x2 Low | 5.16 | 1x4 Low | 0.49 |
| 5 | 2x2 Low | -2.49 | 2x4 Low | -5.00 |
| 6 | 1x2 Low | 6.78 | 1x4 Low | 1.96 |
| 7 | 1x2 Low | 3.12 | 1x4 Low | -0.89 |
| 8 | 2x2 Low | -2.24 | 2x4 Low | -4.22 |
| 9 | 1x2 Low | 5.76 | - | - |
| 10 | 1x2 Low | 1.65 | - | - |
| 11 | 2x2 Low | -0.40 | - | - |
| 12 | 1x2 Low | 3.89 | 1x4 Low | -0.04 |
| 13 | 2x2 Low | -0.82 | 2x4 Low | -0.72 |
| 14 | 1x2 Low | -2.41 | 1x4 Med-A | -3.00 |
| 15 | 1x2 Low | -5.13 | 1x4 Med-A | -3.27 |
| 16 | 2x2 Low | -3.51 | - | - |

# PDCCH Demodulation Requirements

From the results collected in RAN4#90, for the test cases with span in results within 2.5 dB it was agreed to introduce tentative requirements in 38.101-4. For test cases with large span the requirements are TBD. In order to define NR PDCCH demodulation requirements for Rel-15, in case the span in results is > 2.5dB in RAN4#90bis, we propose the following options:

Option1: Increase the margin added to derive SNR requirement from 0.5 dB to 1 dB

Option2: Eliminate outliers from results for SNR requirement derivation in order to bring the span in results within 2.5 dB.

Based on the results submitted in RAN4#90bis we recommend to use either approach and finalize requirements for NR PDCCH.

Proposal #1: For test cases with span > 2.5dB – Option1: Increase margin to derive SNR requirement (or) Option2: eliminate outlier results to derive SNR requirements

# Conclusion

In this paper we present simulation results and our views on defining NR PDCCH demodulation requirements and have the following proposals.

Proposal #1: For test cases with span > 2.5dB – Option1: Increase margin to derive SNR requirement (or) Option2: eliminate outlier results to derive SNR requirements

# Reference

- R4-192406, “Way forward on NR PDCCH demodulation requirements”, Ericsson