3GPP TSG-RAN WG4 Meeting #90bis		R4-1902948

Xi’an, China, 8 - 12 April, 2019

Agenda item:	7.15.4

Source:	Intel Corporation

Title: 	LTE HST performance evaluation – Bidirectional HST SFN scenario

Document for	Discussion

# Introduction

The work item (WI) “LTE high speed” was approved in RAN #80 meeting [1]. One of the objectives of this WI is to identify downlink demodulation requirements under the identified high speed scenarios. In the previous meeting several options for candidate scenarios were identified in a WF [2]:

- HST-SFN scenario to be specified - Option 1: Bidirectional HST SFN with limited max Doppler shift - Option 2: Unidirectional HST SFN - Option 3: Bidirectional HST SFN - If single-tap HST and multipath scenario are considered or not - Option 1: Specify HST, multipath and HST SFN scenario - Option 2: Specify only HST SFN scenario

Companies were encouraged to provide performance evaluations and analysis of the candidate scenarios, in order to down select options. In this contribution we provide simulation results and our views for the bidirectional “4-path” HST SFN scenario.

# Bidirectional “4-path” HST SFN scenario

## Channel model

Figure 1. Bidirectional channel model

Fig.1 shows an example of a “4-path” HST SFN channel model. Given different RRH to RRH distances (Ds) and RRH to railway track distances (Dmin), deployment scenarios for open space and tunnel are considered in [3,4]. The main parameters of deployment are summarized in Table 1.

Table 1. Parameters for bidirectional deployment

| Scenario/Parameters | Ds | Dmin |
| --- | --- | --- |
| Scenario a - Open Space | 1000 m | 50 m |
| Scenario b - Tunnel | 500 m | 5 m |

## Simulation assumptions

For performance alignment at the first stage it is most reasonable to reuse the simulation assumptions which were used in Rel. 14 HST WI [3]. The key parameters are summarized in Table 2.

Table 2. Simulation assumptions

| Parameter | Value |
| --- | --- |
| Bandwidth | 10 MHz |
| Duplex mode | FDD |
| MCS | MCS#17 |
| Antenna configuration | 2x2 |
| Transmission mode | TM3 |

The simulation assumptions in Table 2 were used to define the requirements for the Rel. 14 scenarios with 350 km/h velocity and with 875 Hz max Doppler shift. For Rel. 16 up to 500km/h velocity is considered. Depending on different frequency bands, maximum Doppler shift are different correspondingly. For the analysis we consider the range from 850 to 1550 Hz Doppler shift.

## Simulation results

### Open space vs Tunnel environment

In the figure 2 we provide the normalized throughput simulation results to compare the open space (scenario a) and tunnel environments (scenario b). Evaluations were done for 950 and 1250 Hz Doppler shifts.

| 950 Hz Max Doppler shift |  |
| --- | --- |
| 1250 Hz Max Doppler shift |  |
| Figure 2. Open space vs Tunnel performance comparison |  |

Observation #1: For the bidirectional “4-tap” HST SFN channel model

- For both open space and tunnel with max Doppler shift 950 Hz, UE with practical channel estimation can reach max throughput value with some performance degradation compared to perfect channel estimation

- For both open space and tunnel scenarios with max Doppler shift 1250 Hz, UE with practical channel estimation cannot achieve max throughput value

- UE Demodulation performance in the tunnel Scenario b is better than in the open space scenario.

Proposal #1:		Further deprioritize bidirectional “4-path” HST SFN tunnel scenario

### Max supported Doppler shift

In Figure 3 we provide the simulation results for the Scenario a (open space) for different maximum Doppler shift values. For simulations we use MCS 17 and 16 in order to ensure that the max throughput is comparable with the one achieved in the Rel. 14 HST WI [3]. In Figure 4 we summarize the results and show the maximum achievable normalized throughput values for different Doppler shift scenarios.

| Figure 3. Normalized throughput for different Doppler shift |  |
| --- | --- |

Figure 4. Maximum normalized throughput vs Doppler shift

Observation #2: For the bidirectional “4-tap” HST SFN channel model

- Doppler shift up to 950 Hz does not has any impact on max configured throughput for MCS 16-17

- Doppler shift from 950 to 1150 Hz has a negligible impact on max configured throughput for MCS 16-17

- Doppler shift above 1250 Hz has a significant impact on max configured throughput for MCS 16-17

# Conclusion

In this contribution we provided simulation results to show the throughput performance under the bidirectional “4-path” HST SFN channel for open space and tunnel environment. Following observations and proposals were made:

Observation #1: For the bidirectional “4-tap” HST SFN channel model

- For both open space and tunnel with max Doppler shift 950 Hz, UE with practical channel estimation can reach max throughput value with some performance degradation compared to perfect channel estimation

- For both open space and tunnel scenarios with max Doppler shift 1250 Hz, UE with practical channel estimation cannot achieve max throughput value

- UE Demodulation performance in the tunnel Scenario b is better than in the open space scenario.

Proposal #1:		Further deprioritize bidirectional “4-path” HST SFN tunnel scenario

We also analyzed the Doppler shift range from 850 to 1550 Hz with Rel. 14 simulation assumptions, and following observations were made:

Observation #2: For the bidirectional “4-tap” HST SFN channel model

- Doppler shift up to 950 Hz does not has any impact on max provided throughput for MCS 16-17

- Doppler shift from 950 to 1150 Hz has a negligible impact on max provided throughput for MCS 16-17

- Doppler shift above 1250 Hz has a significant impact on max provided throughput for MCS 16-17

# References

- RP-181482 “New Work Item on Further performance enhancement for LTE in high speed scenario”, RAN #80, June 2018

- R4-1902521 “Way forward on UE demodulation enhancement for HST”, NTT DOCOMO, INC, RAN4 #90, February 2019

- TR 36.878 v13.0.0 “Study on performance enhancements for high speed scenario”

- R4-1900805 “High speed scenarios and parameters for 500km/h velocity”, NTT DOCOMO, INC.