3GPP TSG RAN WG4 Meeting #90bis	R4-1902967

Xi'an, China, 8 - 12 April 2019

Agenda item:	6.3.1

Source:	SoftBank Corp.

Title:	On UE CBW determination in RRC_idle state

Document for:	Approval

1	Introduction

The issue of UE CBW determination has been handled in system parameter session and in RAN#82, a new signalling was defined for RRC_connected state to indicate UE CBW. This paper is to discuss an outstanding issue relevant to UE CBW determination.

2	Consideration

In RAN1/RAN2 spec, there was no IE to indicate BS/UE CBW (RAN4 term, {5, 10, 15,..., 400[MHz]}) that forms an essential part of RF requirements. The carrier bandwidth in RAN1/RAN2 (NSize,μgrid ) has RB granularity per SCS and since the carrier bandwidth per SCS can be configured as a portion of BS CBW, we cannot always assume a direct relation to BS/UE CBW as shown in Figure 1. Our understanding is that a vast variety of possible configurations (including all the example configurations shown below, even if some don't look rational) are allowed with the width of each carrier bandwidth in RB granularity.

Figure 1: Examples of carrier bandwidth configuration

As a solution, RAN#82 agreed the introduction of new IEs "uplink/downlinkChannelBW-PerSCS-List" as UE-specific information [1]. This IE can indicate, per SCS, the starting position and width of usable PRBs relative to "common resource blocks" that has "Point A" as the common originating point to all the SCSs supported.

Since a BS does not indicate UE/BS CBW via MIB/SIB as above, a UE is impossible to know an actual CBW to follow without the newly added information. So the new information would be needed for all the UEs to enforce relevant requirements.

2.1  Handling of random access Msg1 and 3

From UE RF standpoint, UE Tx will also occur while a UE stays in RRC_idle, at least Msg1(preamble) and Msg3 (RRC_message as the first scheduled transmission) in contention-based random access procedures. Apparently these transmissions are also subject to requirements/regulations then they have to follow SEM and ACLR (MPR) or regional protection requirements (A-MPR if so required) determined through a relevant UE CBW. Unsettled UE CBW during RRC_idle could be a risk to violate these requirements through improper MPR/A-MPR to be applied.

[Proposal-1] We need to make clear how UE CBW is to be enforced for Tx in RRC_idle state.

3	Conclusion

This paper addresses remaining topics on UE CBW determination. CBW determination is still needed also in RRC_idle for RACH information transfer from a UE at least.

Reference

RP-182896	   UE specific channel bandwidth signaling (Company CR for TS38.331), Qualcomm