3GPP TSG-RAN WG4 Meeting #90bis	R4-1903014

# Xi'an, China,  8 Apr - 12 Apr 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.817-02 | CR | <CR#> | rev | <Rev#> | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TR 38.817-02 Addition of FR2 TX ON OFF transient background |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 10-04-2019 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The method of translating FR2 TRP OFF requirement to the relative EIRP OFF has been decided but the background information is not updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The relationship between TRP OFF requirement and the relative EIRP measurement is documented. Other background information and some typos are also corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The background information of translating TRP OFF requirement to EIRP OFF is not clearly elaborated and confusion may be introduced in the test. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<START OF  CHANGE>

9.5	OTA Transmit ON/OFF power

9.5.1	OTA Transmitter OFF power

9.5.1.1	General

OTA transmitter OFF power is defined as the mean power measured over 70/N s filtered with a square filter of bandwidth equal to the Base Station RF transmission Bandwidth (s) centred on the assigned channel frequency of the Base Station RF Bandwidth (s) during the transmitter OFF period. N = SCS/15, where SCS is Sub Carrier Spacing in kHz. For BS type 1-O, the transmitter OFF power is defined as the output power at the co-location reference antenna. For BS type 2-O the transmitter OFF power is defined as the TRP. For BS capable of multi-band operation, the requirement is agreed to be only applicable during the transmitter OFF period in all supported operating bands, same as is currently applied for E-UTRA BS.

9.5.1.2	OTA Transmitter OFF power for NR BS 1-O

The transmitter OFF power spectral density for NR BS 1-O is a co-location requirement. The requirement is defined at the co-location reference antenna conductive output side.

The requirements for transmitter OFF power spectral density threshold at the co-location reference antenna is calculated based on the following formula - 85dBm/MHz – 30dB + X dB, where the scaling factor X is equal 9dB. This leads to a transmitter OFF power spectral density threshold of -106 dBm/MHz which is adopted as the minimum requirement in the technical specifications for NR BS type 1-O.

9.5.1.3	OTA Transmitter OFF power for NR BS Type 2-O

For BS Type 1-O three cases are analyzed:

-	Own receiver – this is possible but, it is unlikely a TDD system will have separate Tx and Rx antennas so will be identified by the RX sensitivity requirement even for a conducted system.

-	Co-located receivers – the requirement is measured for the Tx channel – however the same noise can be expected across the whole operational band. Other co-located receivers using other channels in the band are hence protected.

-	Same geographical area receivers – most co-location requirements in the same geographical area assume a reasonable separation between BS’s, hence the isolation is assumed to be greater.

In the case of inter-site interference, the TDD system must be dimensioned with a Guard Period which corresponds to the cell size and its corresponding propagation delay. If we assume that the cell size corresponds to an ISD of d as illustrated in Figure 9.5.1.3-1, we can then calculate the OFF power interference at the neighboring site that can be tolerated as follows.

d

Figure 9.5.1.3-1: The inter site interference propagation case

Assume the Total Radiated Power Off Power, TRPOff, from an interfering site traverse a path loss  LP to reach the victim receiver. The power at the victim receiver array is then:

TRPOff - LP

The off power into each victim transceiver equipped with an element receive gain of  GERX , becomes:

TRPOff - LP+ GERX

The total off power fed into the whole array of N transceivers, then becomes:

TRPOff - LP+ GERX+10logN   (1)

If we assume the margin for a given noise increase in the system is M dB and the applicable bandwidth of B Hz, we get the total noise in the system as:

-174 + 10log(B) + NF - M    (2)

From equations  (1) and (2) we get

TRPOff=-174 + 10log(B) + NF - M - 10logN+LP - GERX

The path loss LP can be estimated using the 3GPP model for NLOS case:

Where:

-	d is in km, and f in GHz

LP=31.3 - (15.07 - 1480hb2)log(hb)+(43.42-3.1log(hb))log(d)+ 20log(f)- 0.6(hm-1.5)

By assuming the following parameters for the path loss LP:

-	B = 1 MHz

-	NF = 10 dB

-	M = 6 dB, for 1 dB noise rise

-	N = 128 transceivers

-	GETX= GERX= 6 dB

-	f = 28 GHz

-	d = 100 m

-	hb= 25 m

-	hm= 1.5 m

We get a value for LP = 117 dB. However, when considering interference, the NLOS case is not the limiting factor but rather the LOS case. In 3GPP we have 2 LOS models LOS1 and LOS2. The path loss LP for each of these LOS models can be expressed as follows:

LP = 28 + 22log(d) + 20log(f), at LOS1

LP = 28 + 22log(d) + 20log(f)– 9log(dbrk2+(hb-hm)2) , at LOS2

LOS1 model is used for the Urban environment which is characterized by shorter distance.

We get LP = 101 dB and the corresponding off power  TRPOff = -36 dBm/MHz.

This value of -36dBm/MHz can then be a requirement for the transmitter off power for BS type 2-O..

This value can be measured either as TRP or at the output of the colocation reference antenna.

In case of transmit power off measurement as a TRP, the transient time is measured as a relative EIRP measurement.

For the colocation interference scenario corresponds to when the base stations of two operators mutually interfere with each other simultaneously. A typical configuration is shown in Figure 9.5.1.3-2 where both antennas are mounted on the same vertical plane. The most difficult case is when both operators coexist in the same band.

The TDD network is synchronized to within a TSync, Cell Phase Sync requirement and the Guard Period TGUARD is dimensioned to cope with the cell size and the transient times:

TGUARD ≥ 2* TSync + 2*Tprop_cell edge +max ((TBS on off), (TUE off on)) + max ((TBS off on), (TUE on off))

The synchronization assumption, the guard period and the assumptions that UL and DL TDD configurations are coordinated so that only the TDD OFF power needs to be considered.

d

Figure 9.5.1.3-2: Colocation interference scenario

This colocation scenario is governed by the OTA reference sensitivity. Equipment not conforming to reference sensitivity requirements would self desense their own transceiver array and its neighboring sectors, even in its own channel. In this colocation case we would have some additional protection, since this is a neighbor carrier, even if it is in the same band.

For the scenario of sector site example, different sectors are installed within one site. This scenario is also governed by the OTA reference sensitivity. Equipment not conforming to reference sensitivity requirements would self desense their own transceiver array and its neighboring.

In summary, the transmit OFF power spectral density for NR BS 2-O can be considered as TRP with a value less than -36 dBm/MHz adopted as the requirement.

After further evaluation, this value can not be measured as TRP in the far field or with CATR test method. Instead, a method of translating the TRP requirement to EIRP level is introduced when performing conformance test. The relationship between TRP and EIRP in the OFF state can be determined as follows.

The TRP OFF level TRPoff  can be evaluated in terms of OFF EIRP, EIRPoff in dBm with the following equation:

EIRPoff=TRPoff+Doff

Doff stands for the antenna directivity in the OFF state in dBi and directivity can be considered as antenna array gain. Since  TRPoff = - 36 dBm/MHz is defined as the requirement, ERIPoff level can be clearly calculated with  Doff. In the transient process, the directivity is assumed as the same for both ON and OFF state, therefore, Doff can be calculated with full antenna gain from the ON state:

Doff=Don

where  Don  stands for directivity in the ON state in dBi.  Don can be calculated with the test results of ON state TRP, TRPon and ON state EIRP,  EIRPon:

Don=EIRPon-TRPon

The corresponding EIRPoff translated from  TRPoff = - 36 dBm/MHz  is therefore defined as:

EIRPoff=TRPoff+Don=TRPoff+EIRPon-TRPon

For example, if the antenna gain in ON state is 27dBi,  EIRPoff  relative to  TRPoff  can be calculated as:

EIRPoff=TRPoff+Don=-36+27=-9dBm/MHz

9.5.2	OTA Transmitter transient period

For OTA, the transmitter transient period is defined the same as for the conducted case. That is, it is defined as the time period during which the transmitter is changing from the OFF period to the ON period or vice versa. The transmitter transient period is illustrated in Figure 9.5.2-1.

Figure 9.5.2-1: Illustration of the relations of transmitter ON period, transmitter OFF period and transmitter transient period.

In E-UTRA BS, the transmitter transient period for both OFF to ON and ON to OFF are specified to be shorter than 17us.

For NR, since larger sub-carrier spacing are introduced, such as 30kHz, 60kHz and 120kHz, the length of symbol is reduced significantly. It means if the same transient period is kept and the same number of symbol is configured for gap period (GP), the cell coverage will be reduced and BS to BS distance without interference will also be reduced. This may bring the risk of introducing BS to BS interference in the network. Balance needs to be considered between transient period requirements and BS to BS interference in the network. So NR BS transient periods need to be optimized.

Based on the above argument, it is agreed to adopt shorter transient periods for NR BS than the values specified for E-UTRA BS by considering the capabilities of current hardware. A value of 10 µs is agreed and accepted for the transmitter transient periods for FR1 as specified in Table 9.5.3.2-1 of 3GPP TS 38.104 [3].

For FR2, a value of 3 µs is agreed for the transmitter transient period as specified in Table 9.5.3.3-1 of 3GPP TS 38.104 [3].

<END OF CHANGE>