3GPP TSG-RAN WG4 Meeting #90-bis                                 R4-1903023

Xi’an, China, 8 – 12 April, 2019

Agenda Item:		9.20.2

Source: 		CMCC

Title: 	TP for TR 37.825 for PC2 EN-DC_41A-n41A

Document for:		Approval

- Introduction

New WID on Power Class 2 UE for EN-DC (1 LTE band +1 NR band) were approved in RAN#82 meeting [1]. This TP summarizes the Band combination specific RF requirements for PC2 EN-DC_41A-n41A.

- Reference

[1]  RP-182877 New WID on Power Class 2 UE for EN-DC (1 LTE band +1 NR band). CMCC

- Text Proposal

<Start of Text Proposal for TR 37.825>

  - Operating bands for PC2 EN_DC

# Table 2.1-1: DC band combination of LTE 1DL/1UL + one NR band

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_41A-n41A 1 | 41 | n41 | Yes |
| NOTE 1: Applicable for UE supporting intra-band EN-DC without simultaneous Rx/Tx. |  |  |  |

  - Channel bandwidths per operating band for PC2 EN_DC

# Table 2.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + one NR band

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |  | Maximum aggregated bandwidth [MHz] |
| DC_41A-n41A | 41 | 15 |  |  |  | Yes |  |  |  |  |  |  |  | 120 |
|  | n41 | 15 |  |  |  |  |  | Yes | Yes |  |  |  |  |  |
|  |  | 30 |  |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  | 60 |  |  |  |  |  | Yes | Yes | Yes | Yes | Yes |  |  |

  - Co-existence studies for PC2 EN_DC

Table 2.3-1 lists Band 41 + Band n41 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

# Table 2.3-1: Band 41 and Band n41 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2496 | 2690 | 2496 | 2690 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 4992 | 5380 | 4992 | 5380 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7488 | 8070 | 7488 | 8070 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 194 | 194 | 4992 | 5380 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2302 | 2884 | 2302 | 2884 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7488 | 8070 | 7488 | 8070 |
| 3rd order IMD products (max BW) | ¦fx_low – max BW fy¦ | ¦fx_high + max BW fy¦ | ¦fy_low – max BW fx¦ | ¦fy_high + max BW fx¦ |
| IMD frequency limits (MHz) | 2396 | 2790 | 2476 | 2710 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 388 | 388 | 9984 | 10760 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4798 | 5574 | 4798 | 5574 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9984 | 10760 | 9984 | 10760 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8264 | 7294 | 8264 | 7294 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 12480 | 13450 | 12480 | 13450 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3078 | 2108 | 3078 | 2108 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 12480 | 13450 | 12480 | 13450 |

Based on Table 2.3-1;

2nd order harmonics may fall into Rx frequencies of bands 46 and n79

2nd order IMD may fall into Rx frequencies of bands 46 and n79

3rd order IMD may fall into Rx frequencies of band 7, 30, 38, 40. 41 and 69

4th order IMD may fall into Rx frequencies of band 46 and 79

5th order IMD may fall into Rx frequencies of band 1, 4, 7, 10, 23, 30, 38, 40, 41, 65. 66 and 69

When 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table2.3-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

# Table 2.3-2: 2UL Band 41 +Band n41 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | YesNote 1 | US/Europe | IMD3 |
|  | 2400 | - | 2494 | YesNote 1 | Asia | IMD3 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 2nd Harmonic, IMD2, IMD4 |
|  | 5150 | - | 5350 | Yes | Europe | 2nd Harmonic, IMD2, IMD4 |
|  | 5470 | - | 5725 | No |  | IMD4 |
|  | 5150 | - | 5825 | Yes | Asia | 2nd Harmonic, IMD2, IMD4 |
| Note: depends on the B41 and n41 allocation |  |  |  |  |  |  |

Table 2.3-3 lists the protected bands required for the dual connectivity configuration.

# Table 2.3-3: Protected bands for the dual connectivity configuration

| EN-DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| … |  |  |  |  |  |  |  |
| DC_41_n41 | E-UTRA Band 1, 2, 3, 4, 5, 8, 10, 12, 13 , 14, 17, 24, 25, 26, 27, 28, 29, 34, 39, 42, 44, 45, 48, 50, 51, 66, 70, 71, 73, 74 NR Band n77, n78 and n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 30, | FDL_low | - | FDL_high | [-40] | 1 |  |
| … |  |  |  |  |  |  |  |

  - ∆TIB and ∆RIB values for PC2 EN_DC

For PC2 EN-DC_41A-n41A, the TIB,c and RIB values are given in Table 2.4-1 Table 2.4-2

# Table 2.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_41A-n41A | 41 | 0 |
|  | n41 | 0 |

# Table 2.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_41A-n41A | 41 | 0 |
|  | n41 | 0 |

  - MSD for PC2 EN-DC

Since Band 41 and n41 are both TDD bands, no MSD is required for the dual connectivity configuration.

  - SEM/A-SEM for PC2 EN-DC

SEM/A-SEM can reuse the band combination specific RF requirements of PC2 EN-DC_41A-n41A in TS 38.101-3.

  - MPR/A-MPR for PC2 EN-DC

MPR/A-MPR can reuse the band combination specific RF requirements of PC2 EN-DC_41A-n41A in TS 38.101-3.

<Start of Text Proposal for TR 37.825>