3GPP TSG-RAN WG4 Meeting #90Bis                 	R4-1903208

Xi’an, China, 8th – 12th April 2019

Source:	Nokia, Nokia Shanghai Bell

Title:	On NR UCI over PUSCH demodulation requirements

Agenda item:	6.12.2.2

Document for:	Discussion

Introduction

In RAN4#90 the NR BS group agreed on the way forward for UCI over PUSCH demodulation requirements [1], which comprehensively defines the simulations assumptions and indicates the following open issues:

- Test metric: - Option1: 1%BLER of UCI with CSI - Option2: 0.1% BLER of UCI with CSI part1, 1% BLER of UCI with CSI part2 - Other options is not precluded. - Company is encouraged to provide results for both option 1 and option 2 including BLER curve in next meeting, decision will be made based on the submitted results. - Number of payload: - 7 bits(part1=5bits, part2=2bits) - One more payload size need to be added - Option 1: [40] bits (part1 = 20 bits, part2 = 20 bits) - Other options are not precluded

In this contribution, we provide our views on these open issues and make few general observations and proposals concerning previously overlooked questions, such as payload content and RV value.

Simulation insights and discussion

In this section we present an analyse our complete BLER vs. SNR simulations. All simulations in this section are based on the assumptions tabularized in the next section. The results for the shared result collection template are presented in the next section.

HARQ settings

It was agreed in [1] not to allow for HARQ retransmissions. However, the RV of the initial transmission was not explicitly set in the simulation setup.

Capture in the UCI over PUSCH simulation setup that RV value 0 is to be used for the initial transmission.

Payload content

First, we tested if the actual payload content of CSI part 1 (CSI-1) has an impact on the observed BLER.

In Figure 1, we see the BLER for both CSI-1 and CSI-2, for the case of CSI-1=5bit and CSI-2=2bit (5, 2 configuration), given CSI-1 payloads of [01010], [00000], and [011111].

Figure 1: CSI part 1 payload comparison (5, 2)

There is no apparent impact of the chosen payload content.
We remark that the first bit of CSI-1 needs to be 0 (i.e., RI=1) for CSI-2 to be of length 2 in a practical implementation of our defined test system (only 1 transmission antenna, i.e., single layer maximum configuration). 
For test system implementation, it is preferable to have a fixed and known payload content, to avoid side-link feedback requirements between TE and BS under test, since no CRC is available. Based on these observations we make our first proposal:

Select the fixed payload content pattern of [01010] for CSI part 1 in the (5, 2) configuration. CSI part 2 should be fixed as well.

For the case of CSI-1=20bit and CSI-2=20bit (20, 20 configuration), there is no practically valid way to set the content of the payload. Hence the payload content should be set at random for each multiplexing realization. This technique was used in Figure 2.

Figure 2: CSI part 1 and 2 random payload (20, 20)

Select the random payload content CSI part 1 and 2 in the (20, 20) configuration, for each multiplexing realization and rely on CRC for detecting decoding errors.

  - Performance metric

When analysing the BLER vs. SNR curves, one notices that the BLERs for CSI-1 and CSI-2 differ for the non-symmetric case of (5, 2) (see Figure 3) and are the same for the symmetric case of (20,20) (see Figure 4).

The symmetric case is explained by the simulation setup (βOffset equal for part 1 and part 2) and the equal number of information bits, which are protected by the same polar code.

For the non-symmetric case, we need to consider that we have and unequal number of information bits, and channel coding with different efficiencies (5 bits are protected via Reed-Muller codes, while 2 bits are protected by repetition coding and usage of the outermost constellation points).

Figure 3: BLER vs. SNR curves for (5, 2)

Figure 4: BLER vs. SNR curves for (20, 20)

The BLER calculated on the complete payload is seen to be a non-linear function of both the CSI-1 BLER and CSI-2 BLER. However, the overall absolute difference between BLERfull_payload and max(BLER1, BLER2) is rather small.

RAN4 to consider the performance metric 1% BLER of UCI with CSI part1, 1% BLER of UCI with CSI part2.

During the email discussions that took place in the period leading up to RAN4#90b, it became evident that the calculation of BLERfull_payload, BLER1, and BLER2, needs to be define more rigorously. Following the convergence of the email discussion we propose the following:

The calculation of BLERfull_payload should count any packet as erroneous that has one or more bits of the packet payload decoded erroneously. The packet payload is the concatenation of CSI-1 and CSI-2.

The calculation of BLER1 should count any packet as erroneous that has one or more bits of the packet payload decoded erroneously. The packet payload is CSI-1.
BLER2 should be treated independently.
The calculation of BLER2 should count any packet as erroneous that has one or more bits of the packet payload decoded erroneously. The packet payload is CSI-2.

BLERfull_payload cannot be mathematically calculated from BLER1 and BLER2, since the overlapping case of CSI-1 and CSI-2 being wrong in the same TTI is not captured.

Simulation results delivery

The following results were obtained using the initial simulation assumptions, as captured in [1].

Table 1: Simulation assumption for UCI over PUSCH results

| Parameter |  | Value |  |
| --- | --- | --- | --- |
|  |  | FR1 | FR2 |
| General System | Transform precoding | Disabled | Disabled |
|  | Number of Tx | 1 | 1 |
|  | Number of Rx | 2 | 2 |
|  | Number of layers | 1 | 1 |
|  | [Uplink-downlink allocation for TDD] | 30 kHz SCS: 7D1S2U, S=6D:4G:4U | 120kHz SCS: 3D1S1U, S=10D:2G:2U |
|  | Code block group based PUSCH transmission | Disabled | Disabled |
|  | MCS index | 16 | 16 |
|  | Propagation Condition | MCS 16: TDL-C 300ns, 100 Hz | MCS 16: TDL-A 30ns, 300Hz |
| HARQ | Number of HARQ transmissions | 1 | 1 |
|  | RV value | 0 | 0 |
| DMRS | DMRS configuration type | 1 | 1 |
|  | Maximum number of OFDM symbols for front loaded DMRS | 1 | 1 |
|  | Number of additional DMRS symbols | 1 | 0 |
|  | Number of DMRS CDM group(s) without data | 2 | 2 |
|  | EPRE ratio of PUSCH to DMRS | -3 dB | -3 dB |
|  | DMRS port | 0 | 0 |
|  | DMRS sequence generation | NID=0, nSCID =0 | NID=0, nSCID =0 |
| Time domain resource | PUSCH mapping type | A, B | B |
|  | PUSCH starting symbol index | 0 | 0 |
|  | PUSCH symbol length | 14 | 14 |
| Frequency domain resource | Carrier frequency (GHz) | 4 | 30 |
|  | SCS and BW | 30kHz: 10 MHz | 120kHz: 50 MHz |
|  | RB assignment | Full applicable test bandwidth | Full applicable test bandwidth |
|  | Frequency hopping | Disabled | Disabled |
| PTRS configuration | Frequency density (KPT-RS) | no PTRS | 2 |
|  | Time density (LPT-RS) | no PTRS | 1 |
| UCI | UL-SCH | With | With |
|  | Payload (bits) | Case 1: [csi part1= 5, csi_part2 =2] Case 2 [csi_part1=[20],csi_part2=[20]] | Case 1: [csi_part1 = 5 ,csi_part2 = 2] Case 2: [csi_part1=[20], csi_part2=[20]] |
|  | uci-on-pusch-scaling | 1 | 1 |
|  | I^CSI-1 and I^CSI-2 | [6.25] for CSI | [6.25] for CSI |
| Test metric |  | [SNR@ 1% BLER] [0.1% BLER of UCI with CSI part1, 1% BLER of UCI with CSI part2 ] | [SNR@ 1% BLER] [0.1% BLER of UCI with CSI part1, 1% BLER of UCI with CSI part2 ] |

| UCI on PUSCH 1T2R |  |  |  |  | UCI | CSI part1 | CSI part2 | Test metric | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Ideal | Impair-ment |
|  | FR1 Mappying type A | 30kHz, 10MHz | MCS16 | TDLC300-100 | 7 | 5 | 2 | 1% of BLER CSI | 0,3 | 2,8 |
|  |  |  |  |  |  |  |  | 0.1% of BLER CSI part1 | 2,7 | 5,2 |
|  |  |  |  |  |  |  |  | 1% of BLER CSI part2 | -0,85 | 1,65 |
|  |  |  |  |  | 40 | 20 | 20 | 1% of BLER CSI |  |  |
|  |  |  |  |  |  |  |  | 0.1% of BLER CSI part1 | 2,5 | 5 |
|  |  |  |  |  |  |  |  | 1% of BLER CSI part2 | 0,45 | 2,95 |
|  | FR1 Mappying type B | 30kHz, 10MHz | MCS16 | TDLC300-100 | 7 | 5 | 2 | 1% of BLER CSI | 0,9 | 3,4 |
|  |  |  |  |  |  |  |  | 0.1% of BLER CSI part1 | 3,4 | 5,9 |
|  |  |  |  |  |  |  |  | 1% of BLER CSI part2 | -0,15 | 2,35 |
|  |  |  |  |  | 40 | 20 | 20 | 1% of BLER CSI |  |  |
|  |  |  |  |  |  |  |  | 0.1% of BLER CSI part1 | 2,3 | 4,8 |
|  |  |  |  |  |  |  |  | 1% of BLER CSI part2 | 0,45 | 2,95 |
|  | FR2 with PTRS under type B | 120kHz, 50MHz | MCS 16 | TDLA30-300 | 7 | 5 | 2 | 1% of BLER CSI | 5,55 | 8,65 |
|  |  |  |  |  |  |  |  | 0.1% of BLER CSI part1 | 8,05 | 11,15 |
|  |  |  |  |  |  |  |  | 1% of BLER CSI part2 | 4,3 | 7,4 |
|  |  |  |  | TDLA30-300 | 40 | 20 | 20 | 1% of BLER CSI |  |  |
|  |  |  |  |  |  |  |  | 0.1% of BLER CSI part1 | 6,55 | 9,65 |
|  |  |  |  |  |  |  |  | 1% of BLER CSI part2 | 0,42 | 3,52 |

Conclusion

In this contribution we have discussed the open issues configuration issues of UCI multiplexing and made few general observations and proposals concerning previously overlooked questions, such as payload content and RV value.
In particular, we have made the following proposals and observations:

- Capture in the UCI over PUSCH simulation setup that RV value 0 is to be used for the initial transmission.

- Select the fixed payload content pattern of [01010] for CSI part 1 in the (5, 2) configuration. CSI part 2 should be fixed as well.

- Select the random payload content CSI part 1 and 2 in the (20, 20) configuration, for each multiplexing realization and rely on CRC for detecting decoding errors.

- RAN4 to consider the performance metric 1% BLER of UCI with CSI part1, 1% BLER of UCI with CSI part2.

- The calculation of BLERfull_payload should count any packet as erroneous that has one or more bits of the packet payload decoded erroneously. The packet payload is the concatenation of CSI-1 and CSI-2.

- The calculation of BLER1 should count any packet as erroneous that has one or more bits of the packet payload decoded erroneously. The packet payload is CSI-1.
BLER2 should be treated independently.
The calculation of BLER2 should count any packet as erroneous that has one or more bits of the packet payload decoded erroneously. The packet payload is CSI-2.

- BLERfull_payload cannot be mathematically calculated from BLER1 and BLER2, since the overlapping case of CSI-1 and CSI-2 being wrong in the same TTI is not captured.

- References

- R4-1902439, WF on performance requirements of UCI Multiplexing on PUSCH, Samsung, RAN4#90.