3GPP TSG-RAN WG4 Meeting #90b                    	   R4-1903267

Xi’an, China, Apr 8th – 12th, 2019

Agenda item:            6.12.1.2.1

Source:	MediaTek Inc.

Title:	NR PDSCH HST simulation result

Document for:          Discussion

# 1	Introduction

In this paper, we present the simulation results of PDSCH HST channel and the proposal for the HST test case setting.

# 2	Discussion

Based on the simulation assumption in [1], we provide the simulation result of HST test cases. Table A.1 and Table A.2 (in Appendix) contain detailed parameter setting in the simulation.

Table 2.1 and Table 2.2 are the 70% Throughput SNR(dB) for FDD cases and  Table 2.3 and Table 2.4 are the 70% Throughput SNR(dB) for TDD cases

Table 2.1 70% Tput SNR for FDD cases (without SSB tracking)

| MCS | FDD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 300km/h (fd = 750 Hz) |  |  |  | 350km/h (fd = 875 Hz) |  |  |  |
|  | TRS 10ms |  | TRS 20ms |  | TRS 10ms |  | TRS 20ms |  |
|  | DMRS add1 | DMRS add2 | DMRS add1 | DMRS add2 | DMRS add1 | DMRS add2 | DMRS add1 | DMRS add2 |
| 4 | -6.25 | -6.45 | -6.3 | -6.5 | -6.25 | -6.45 | -6.3 | -6.5 |
| 13 | 0.2 | 0.2 | 0.15 | 0.15 | 0.2 | 0.2 | 0.15 | 0.15 |

Table 2.2 70% Tput SNR for FDD cases (with SSB tracking)

| MCS | FDD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 300km/h (fd = 750 Hz) |  |  |  | 350km/h (fd = 875 Hz) |  |  |  |
|  | TRS 10ms |  | TRS 20ms |  | TRS 10ms |  | TRS 20ms |  |
|  | DMRS add1 | DMRS add2 | DMRS add1 | DMRS add2 | DMRS add1 | DMRS add2 | DMRS add1 | DMRS add2 |
| 4 | -6.25 | -6.45 | -6.3 | -6.5 | -6.25 | -6.45 | -6.3 | -6.5 |
| 13 | 0.2 | 0.2 | 0.15 | 0.15 | 0.2 | 0.2 | 0.15 | 0.15 |

Table 2.3 70% Tput SNR for TDD cases (without SSB tracking)

| MCS | TDD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 300km/h (fd = 750 Hz) |  |  |  | 350km/h (fd = 1167 Hz) |  |  |  |
|  | TRS 10ms |  | TRS 20ms |  | TRS 10ms |  | TRS 20ms |  |
|  | DMRS add1 | DMRS add2 | DMRS add1 | DMRS add2 | DMRS add1 | DMRS add2 | DMRS add1 | DMRS add2 |
| 4 | -6.15 | -6.5 | -6.05 | -6.5 | -6.15 | -6.5 | -6.15 | -6.5 |
| 13 | 0.2 | 0.3 | 0.2 | 0.3 | 0.25 | 0.3 | 0.25 | 0.3 |

Table 2.4 Tput SNR for TDD cases (with SSB tracking)

| MCS | TDD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 300km/h (fd = 750 Hz) |  |  |  | 350km/h (fd = 1167 Hz) |  |  |  |
|  | TRS 10ms |  | TRS 20ms |  | TRS 10ms |  | TRS 20ms |  |
|  | DMRS add1 | DMRS add2 | DMRS add1 | DMRS add2 | DMRS add1 | DMRS add2 | DMRS add1 | DMRS add2 |
| 4 | -6.15 | -6.5 | -6.15 | -6.5 | -6.15 | -6.5 | -6.15 | -6.5 |
| 13 | 0.2 | 0.3 | 0.2 | 0.3 | 0.25 | 0.3 | 0.25 | 0.3 |

2.1 PDSCH DMRS symbol number

For MCS4 (QPSK), 2 additional DMRS has 0.2~0.4dB SNR gain compared to 1 additional DMRS due to time domain combining gain, which is more significant in low SNR region. For MCS13 (16QAM), the performance is almost the same between the 2 DMRS settings since channel variation dominates the performance in this SNR region and 1 additional DMRS is enough to handle the simple HST channel (1-tap). Under MCS13 setting, 1 additional DMRS can obtain higher throughput (~5% better) due to lower DMRS overhead.

Figure 2.1.1 HST performance with different DMRS number

| FDD, QPSK, Trs10ms, 300km/h | FDD, 16QAM, Trs10ms, 300km/h |
| --- | --- |

Based on the simulation results, there is no much difference between the 2 DMRS settings. However, we think 1-tap HST model is too simple and the results cannot justify the typical DMRS setting for HST scenario. Therefore, we leverage the 4-tap HST channel model from [2] to provide more practical analysis for PDSCH DMRS symbol number requirement. Figure 2.1 is the reference 4-tap HST model

Figure 2.1.2 4-tap HST model

Based on the 4-tap HST model, we can select some locations to have more sophisticated performance evaluation. We provide Tput performance evaluation at point A, C and F under 2.6GHz and mobility 500km/h (Maximum Doppler shift = 1204Hz). More simulation setting is in Table A.3 (in Appendix). Table 2.1.1 is the Tput performance for 3 PDSCH DMRS settings

- At the lowest Doppler shift position A, 1 additional DMRS outperforms 2 additional DMRS but the gain is less than 5%.

- At the highest Doppler shift position F, 1 additional DMRS underperforms 2 additional DMRS, and the loss is larger than 25% (especially in SNR operating range 20~25dB, loss > 70%)

Table 2..1.1 Tput for 4-tap HST channel

|  |  | Throughput (Mbps) |  |  |
| --- | --- | --- | --- | --- |
| Position | SNR | 1 additional DMRS | 2 additional DMRS | 3 additional DMRS |
| A | 0 | 28.08 | 25.85 | 22.70 |
|  | 10 | 93.24 | 91.66 | 81.45 |
|  | 20 | 145.38 | 144.08 | 115.71 |
|  | 25 | 161.77 | 168.29 | 118.30 |
| C | 0 | 18.24 | 16.15 | 14.45 |
|  | 10 | 70.98 | 74.59 | 68.94 |
|  | 20 | 83.14 | 111.35 | 101.11 |
|  | 25 | 83.14 | 111.59 | 105.46 |
| F | 0 | 8.19 | 11.26 | 7.92 |
|  | 10 | 10.82 | 23.92 | 19.37 |
|  | 20 | 9.19 | 30.05 | 22.17 |
|  | 25 | 9.22 | 28.48 | 21.95 |

Based on the result, it is obvious 1 additional PDSCH DMRS is not enough to handle the high Doppler spread and 2 additional PDSCH DMRS is required for 4-tap HST channel. Combining the performance results of 1-tap HST channel and 4-tap HST channel, 2 additional DMRS should be more practical for HST scenario

Proposal 1: Use 2 additional PDSCH DMRS symbols for HST test cases

2.2 TRS configuration

According to the simulation result, the performance of 10ms and 20ms TRS are almost the same since the Doppler spread is not significant and the channel model is simple. Considering even higher carrier frequency and more complicated channel mode, 10ms TRS should be more robust

Proposal 2: Use 10ms TRS for HST test cases

Figure 2.1.3 HST performance with different TRS period

| FDD, QPSK, 1 additional DMRS, 300km/h | FDD, 16QAM, 1 additional DMRS, 300km/h |
| --- | --- |

2.3 UE tracking

The performance difference between UE tracking with and without SSB is not significant. To simplify the assumption, we can select the simple one for the performance requirement

Proposal 3: Assume UE only uses TRS on tracking for the HST performance requirement

Figure 2.1.4 HST performance with different UE tracking RS

| FDD, QPSK, 2 additional DMRS, Trs20ms, 350km/h | FDD, 16QAM, 2 additional DMRS, Trs20ms, 350km/h |
| --- | --- |

According to the 1-tap HST performance analysis under various aspects, it is obvious that 1-tap HST channel is too simple to provide the typical system parameters. To guarantee UE performance in HST scenario, we should include more practical HST channel model for the requirement. To reduce the development effort, we can leverage LTE HST-SFN channel (4-tap) in [2]

Proposal 4: Introduce LTE HST-SFN channel for NR HST performance requirement

# 3	Conclusions

In the contribution, we provide our simulation results of PDSCH HST tests and we have the following proposals

Proposal 1: Use 2 additional PDSCH DMRS symbols for HST test cases

Proposal 2: Use 10ms TRS for HST test cases

Proposal 3: Assume UE only uses TRS on tracking for the HST performance requirement

Proposal 4: Introduce LTE HST-SFN channel for NR HST performance requirement

# 4	Reference

# [1] R4-1902403, “Way forward on NR UE PDSCH demodulation requirements and General aspects”, Intel.

# [2] 3GPP TS 36.101 V15.5.0

Appendix

Table A.1 PDSCH HST test case channel model parameter

| Parameter | Option 1 | Option 2 |
| --- | --- | --- |
| Ds | 300 m | 300 m |
| Dmin | 2 m | 2 m |
| v | 350 km/h | 300 km/h |
| fd | 875 Hz for 15 kHz SCS test and 1167 Hz for 30 kHz SCS test | 750 Hz for 15 kHz SCS and 30 kHz SCS tests |

Table A.2 PDSCH HST test case parameter

| Parameter | Setting |
| --- | --- |
| Channel bandwidth and SCS | FDD: 10 MHz, SCS 15 kHz TDD: 40 MHz, SCS 30 kHz (7D1S2U, S = 6D:4S:4U) |
| PDSCH mapping | Type A, Start symbol 2, Duration 12 |
| DMRS configuration | Option 1: Type 1, Single symbol, 1 additional DMRS Option 2: Type 1, Single symbol, 2 additional DMRS |
| TRS configuration | Option 1: 10ms, 2 slots, offset 1 Option 2: 20ms, 2 slots, offset 10 |
| FRC | Option 1: QPSK Rank 1 (MCS 4) Option 2: 16QAM Rank 1 (MCS 13) |
| Baseline receiver assumption for frequency offset tracking | Option1: TRS based on tracking Option2: SSB+TRS based on tracking (SSB with 20ms periodicity) |

Table A.3 The setting of four tap HST model

| Parameter |  |
| --- | --- |
| Ds | 750 m |
| Dmin | 120 m |
| v | 500 km/h |
| fd | 1204 Hz for 30 kHz SCS |
| Channel bandwidth and SCS | FDD: 20 MHz, SCS 30 kHz |
| PDSCH mapping | Type A, Start symbol 2, Duration 12 |
| DMRS configuration | Option 1: Type 1, Single symbol, 1 additional DMRS Option 2: Type 1, Single symbol, 2 additional DMRS |
| TRS configuration | 20ms, 2 slots, offset 1 |
| FRC | Choose MCS/Rank with max Tput for each point Rank ∈ {1, 2, 4}, MCS ∈ {0~27} |