3GPP TSG-RAN WG4 Meeting #90Bis	R4-1903348

# Xi’an, China, 8th – 12st April 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 6363 | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction CR to TS36.133 on capabilities for support of reporting criteria requirement for EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2019-03-31 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Correction is needed for capabilities for support of event triggering and reporting criteria requirement for EN-DC capable UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following changes have been provided to Table 8.2.2-1 in this CR: - In Table 8.2.2-1, the description note for “inter-RAT NR carrier frequency” has been updated: (a) total 10 reporting criteria for the category of “inter-RAT NR carrier frequency” is supported, while no need to discriminate per RAT (similar to other NR requriement); (b) removal of the restriction of the applicability rule , i.e., only applicable for mesurements on any of the NR carrer frequencies other than carrier frequency of the NR PSCell and NR SCell; (c) square bracket removal. - In Table 8.2.2-1’s Note 1 for intra-frequency measurement, no need to mention NR PSCell, since NR carriers to be measured by EN-DC MN is considered as the category of “inter-RAT NR” measurement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EN-DC capable UE’s capabilities for support of event triggering and reporting criteria requirement contains error. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### >>>>>Start of Change 1<<<<<

## 8.2	Capabilities for Support of Event Triggering and Reporting Criteria

### 8.2.1	Introduction

This clause contains requirements on UE capabilities for support of event triggering and reporting criteria. As long as the measurement configuration does not exceed the requirements stated in clause 8.2.2, the UE shall meet the performance requirements defined in clause 9.

The UE can be requested to make measurements under different measurement identities defined in TS 36.331 [2]. Each measurement identity corresponds to either event based reporting, periodic reporting, logged measurement reporting [2] or no reporting. In case of event based reporting, each measurement identity is associated with an event. In case of periodic reporting, a measurement identity is associated with one periodic reporting criterion. In case of logged measurement reporting, a measurement identity is associated with one logged measurement reporting criterion. In case of no reporting, a measurement identity is associated with one no reporting criterion.

The purpose of this clause is to set some limits on the number of different event, periodic, logged measurement and no reporting criteria the UE may be requested to track in parallel.

### 8.2.2	Requirements

In this clause a reporting criterion corresponds to either one event (in the case of event based reporting), or one periodic reporting criterion (in case of periodic reporting), or one logged measurement reporting criterion (in case of logged measurement reporting), or one no reporting criterion (in case of no reporting). For event based reporting, each instance of event, with the same or different event identities, is counted as separate reporting criterion in table 8.2.2-1.

The UE shall be able to support in parallel per category up to Ecat reporting criteria according to table 8.2.2-1. For the measurement categories belonging to measurements on: E-UTRA intra-frequency cells, E-UTRA inter-frequency cells, and inter-RAT per supported RAT(i.e. without counting other categories that the UE shall always support in parallel), the UE need not support more than the total number of reporting criteria as follows:

-	26 reporting criteria in total if the UE is not configured with any SCell or PSCell carrier frequency,

-	35 reporting criteria in total if the UE is configured with one SCell carrier frequency,

-	44 reporting criteria in total if the UE is configured with two SCell carrier frequencies,

-	53 reporting criteria in total if the UE is configured with three SCell carrier frequencies,

-	62 reporting criteria in total if the UE is configured with four SCell carrier frequencies,

-	71 reporting criteria in total if the UE is configured with five SCell carrier frequencies,-	80 reporting criteria in total if the UE is configured with six SCell carrier frequencies,

-	35 reporting criteria in total if the UE is configured with one PSCell carrier frequency, and

-	44 reporting criteria in total if the UE is configured with one PSCell carrier frequency and one SCell carrier frequency.

Editor’s note: the total reporting criteria above are to be updated if all UEs will have to support RS-SINR measurements; the total reporting criteria are to be verified when the UE capabilities related to frame structure 3 are decided.

A UE supporting increased number of carriers to monitor beyond 3 carriers shall be able to support up to 20 reporting criteria for inter-frequency measurement category according to table 8.2.2-1. Additionally such UE shall be able to support in parallel per category up to Ecat reporting criteria according to table 8.2.2-1. For the measurement categories belonging to measurements on: E-UTRA intra-frequency cells, E-UTRA inter-frequency cells, and inter-RAT per supported RAT, the UE need not support more than the total number of reporting criteria as follows:

-	39 reporting criteria in total if the UE is not configured with any SCell carrier frequency,

-	48 reporting criteria in total if the UE is configured with one SCell carrier frequency,

-	57 reporting criteria in total if the UE is configured with two SCell carrier frequencies,

-	48 reporting criteria in total if the UE is configured with one PSCell carrier frequency,

-	57 reporting criteria in total if the UE is configured with one PSCell carrier frequency and one SCell carrier frequencies,

-	66 reporting criteria in total if the UE is configured with three SCell carrier frequencies, and

-	75 reporting criteria in total if the UE is configured with four SCell carrier frequencies.

-	84 reporting criteria in total if the UE is configured with five SCell carrier frequencies

-	93 reporting criteria in total if the UE is configured with six SCell carrier frequencies

Editor’s note: the total reporting criteria above are to be updated if all UEs will have to support RS-SINR measurements; the total reporting criteria are to be verified when the UE capabilities related to frame structure 3 are decided.

The UE capable of supporting EN-DC operation with NR PSCell and one or more NR carrier frequencies in total shall be able to support in parallel per category up to Ecat reporting criteria according to table 8.2.2-1. For the measurement categories belonging to measurements on: E-UTRA intra-frequency cells, E-UTRA inter-frequency cells, inter-RAT per supported RAT, and NR cells on serving and non-serving carrier frequencies (i.e. without counting other categories that the UE shall always support in parallel), the UE need not support more than the number of reporting criteria, excluding reporting criteria specified in TS 38.133 [50] that are applicable for the UE configured with EN-DC operation, as follows:

-	[36] reporting criteria if the UE is not configured with any SCell or PSCell carrier frequency or NR SCell or NR PSCell,

-	[36] reporting criteria if the UE is not configured with any SCell or NR SCell but configured with one NR PSCell carrier frequency.

Editor’s note: the above list is to be updated for the agreed CA combinations with NR PSCell.

Table 8.2.2-1: Requirements for reporting criteria per measurement category

| Measurement category | Ecat | Note |  |
| --- | --- | --- | --- |
| Intra-frequency Note 1 | 10 | Events for any one or a combination of intra-frequency RSRP, RSRQ, and RS-SINRNote4 for E-UTRA intra-frequency cells |  |
| Intra-frequency UE Rx-Tx time difference | 2 | Intra-frequency UE Rx-Tx time difference measurements reported to E-UTRAN via RRC and to positioning server via LPP. Applies for UE supporting both LPP and UE Rx-Tx time difference measurement. |  |
| Intra-frequency RSTD Note 2 | 1 | Intra-frequency RSTD measurement reporting for UE supporting OTDOA; 1 report capable of minimum 16 cell measurements for the intra-frequency |  |
| Intra-frequency RSRP and RSRQ measurements for E-CID | 1 | Intra-frequency RSRP and RSRQ measurements for E-CID reported to E-SMLC via LPP [24]. One report capable of at least in total 9 intra-frequency RSRP and RSRQ measurements. Applicable to UE capable of reporting RSRP and RSRQ to E-SMLC via LPP. |  |
| Intra-frequency RSSI and channel occupancy measurements under operation with frame structure 3 | 1 | One report capable of one UE RSSI and channel occupancy measurement s per serving carrier frequency. Applicable for UE capable of performing and reporting UE RSSI and channel occupancy under operation with frame structure 3. |  |
| Inter-frequency | 10 / 28 | Events for any one or a combination of inter-frequency RSRP, RSRQ, and RS-SINRNote4 for E-UTRA inter-frequency cells (see note 3) |  |
| Inter-frequency RSTD Note 2 | 1 | Inter-frequency RSTD measurement reporting for UE supporting OTDOA; 1 report capable of minimum 16 cell measurements for at least one inter-frequency. Only applicable as specified in Section 8.1.2.6. |  |
| Inter-frequency RSSI and channel occupancy measurements under operation with frame structure 3 | 1 | One report capable of one UE RSSI and channel occupancy measurement s for an inter-frequency. Applicable for UE capable of performing and reporting UE RSSI and channel occupancy under operation with frame structure 3. |  |
| Inter-RAT (GSM, cdma2000 1 x RTT and HRPD) | 5 | Only applicable for UE with this (inter-RAT) capability. This requirement (Ecat = 5) is per supported RAT. |  |
| Inter-RAT (UTRAN FDD, UTRAN TDD) | 5 or 11 | Only applicable for UE with this (inter-RAT) capability. This requirement (Ecat = 5 or 11) is per supported RAT. For UE which indicate support for Increased UE carrier monitoring UTRA Ecat = 11. |  |
| Inter-RAT NR carrier frequency | 10 | Events for NR cells on all inter-RAT NR carrier frequencies for UE capable of EN-DC operation. |  |
| MBSFN measurements for MDT | 1 | MBSFN measurement reporting for UE supporting MBSFN measurements (MBSFN RSRP, MBSFN RSRQ, and MCH BLER) for MDT [2]; 1 report capable of minimum 1 MBSFN RSRP measurement [4], 1 MBSFN RSRQ measurement [4], and 1 MCH BLER measurement [4]. |  |
| Note 1: When the UE is configured with SCell, PSCell, or PCell carrier frequency, Ecat for Intra-frequency is applied per serving frequency. Note 2: When the UE is configured with one SCell carrier frequency, the UE shall be capable of supporting at least 2 reporting criteria for all RSTD measurements configured to be performed on PCell carrier frequency, SCell carrier frequency and inter-frequency carrier. When the UE is configured with two SCell carrier frequencies, the UE shall be capable of supporting at least 3 reporting criteria for all RSTD measurements configured to be performed on PCell carrier frequency, the two SCell carrier frequencies and inter-frequency carrier. These requirements apply when there is a single on-going LPP OTDOA location session. Note 3: Support of Ecat of 28 for Measurement category Inter-frequency is applied for a UE supporting increased number of carriers to monitor beyond 3. Note 4: For UEs supporting RS-SINR measurements (Editor’s note: the note is to be removed if the RS-SINR measurement support is mandatory). |  |  |  |

### >>>>>End of Change 1<<<<<