3GPP TSG-RAN WG4 Meeting #90bix                                         R4-1903374

Xi’an, China, 8th – 12th April, 2019

Agenda Item:   8.5.2

Source:        Samsung

Title:          Co-existence study for NR IAB

Document for:  Approval

# Introduction

This contribution will share our view on RAN4 IAB co-existence study for Rel16 based on discussion shared in [1].

# Discussion

In RAN4 the co-existence study is to derive the ACLR and ACS requirement based on static system simulation for interference with same direction from or to adjacent channel inter system. And the intra system interference constraint between backhaul link and access link shall not be covered in co-existence study for this purpose. Hence the TDM multiplexing between backhaul and access link can be assumed in assessment of co-existence study which is recognized can meet the evaluation purpose with simple way.

Proposal 1: TDM multiplexing should be considered in co-existence study with target at ALCR and ACS requirement.

To be simplified, the LTE relay(RN) assumption for resource multiplexing in[4] can be reused for IAB evaluation with minor update, in which the eNB should be replaced by gNB and RN should be replaced by IAB. It should be pointed that even though multiple hops (e.g.,1-4) are captured in NR IAB WID [2], whether it is should be considered in simulation for ACLR and ACS is questionable.

Resource partitioning at the RN:

-	in the downlink, eNB → RN and RN → UE links are time division multiplexed in a single carrier frequency (only one is active at any time)

-	in the uplink, UE → RN and RN → eNB links are time division multiplexed in a single carrier frequency (only one is active at any time)

Multiplexing of backhaul links in FDD

-	eNB → RN transmissions are done in the DL frequency band

-	RN → eNB transmissions are done in the UL frequency band

Multiplexing of backhaul links in TDD:

-	eNB → RN transmissions are done in the DL subframes of the eNB and RN

-	RN → eNB transmissions are done in the UL subframes of the eNB and RN

Furthermore, according TDM cases described in SI TR[3] in table 7.3.3-1, more restriction is introduced in for NR IAB compared with LTE Relay.

Table 7.3.2-1: Supported TDM cases(from TR38.874)

|  | TDM Between: |  |
| --- | --- | --- |
| Case | Link 1 | Link 2 |
| 1 | LP,DL | LC,DL |
| 2 | LP,UL | LC,UL |
| 3 | LP,DL | LC,UL |
| 4 | LP,UL | LC,DL |
| 5 | LP,DL | LA,DL |
| 6 | LP,UL | LA,UL |
| 7 | LP,DL | LA,UL |
| 8 | LP,UL | LA,DL |
| 9 | LP,DL | LA,DL and LC,DL |
| 10 | LP,UL | LA,UL and LC,UL |
| 11 | LP,DL | LA,UL and LC,UL |
| 12 | LP,UL | LA,DL and LA,DL |
| 13 | LC,DL | LA,DL |
| 14 | LC,UL | LA,UL |
| 15 | LC,DL | LA,UL |
| 16 | LC,UL | LA,DL |

According to above understanding there are four cases would exist for co-existence simulation as table 2 below.

Table 2: co-existence cases for IAB

| Case# | Aggressor system | Victim system |
| --- | --- | --- |
| Case 1 | BS-> UE +IAB-> UE on DU Or BS->UE+ BSp->IAB as MT | BS-> UE on adjacent channel |
| Case 2 | UE-> BS+ IAB as MT-> BSp Or UE-> BS+ UE->IAB as DU | UE->BS on adjacent channel |
| Case 3 | BS-> UE on adjacent channel | BSp-> IAB as MT |
| Case 4 | UE-> BS on adjacent channel | UE-> IAB as DU |

For access link, as pointed in [1], the same ACLR and ACS requirement defined in TS38.104 of small cell could be expected for IAB. It’s not anticipated that the IAB node could achieve more stringent performance compared with BS in both access and backhaul link considering the implementation reality. Hence the cases used to derive IAB backhaul link ACLR and ACS should be prioritized. Furthermore, as for FR1 there are requirement defined in LTE Relay TS can be reference for NR IAB, the FR2 study should be prioritized.

Proposal 2: the cases 2 and case 3 in table 2 to derive IAB backhaul link ACS and ACLR should be prioritized.

Proposal 3: Co-existence study for FR2 IAB should be prioritized.

In addition, to decide the detail simulation assumption for FR2 IAB we compared the simulation assumption for CPE study [5] and IAB assumption used in RAN1 evaluation captured in Annex A of [3], considering the similarity between CPE and IAB backhaul link. It can be noticed that there are many differences between simulation assumptions in deployment scenario. Considering the anticipated applied scenario of IAB it is reasonable to consider RAN1 simulation assumption as starting point except for the power class of backhaul link, for which further operators’ input and regulation restriction (if exists) would be needed.

Table 3: comparison between simulation assumption of CPE and IAB node

|  | CPE simulation assumption in RAN4 | IAB |
| --- | --- | --- |
| Network layout | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around | Heterogeneous: two layer Macro+Micro(IAB) Homogeneous: one layer Micro(IAB) |
| Macro BS antenna height | 35m | 35m |
| Target site height | CPE antenna height 4.5m | IAB antenna height 10m |
| Minimum distance with Macro BS | CPE-Macro BS: 35m | IAB node to donor BS: 40m |
| Macro BS ISD | 300m | 200m/500m |
| Maximum PA output power | 35dBm | 33dBm |
| Maximum EIRP | <55dBm | <68dBm |
| Carrier frequency | 30GHz | 4GHz, 30GHz |

# Summary

For co-existence study, initial consideration shared in this contribution with below proposals.

Proposal 1: TDM multiplexing should be considered in co-existence study with target at ALCR and ACS requirement.

Proposal 2: the cases 2 and case 3 in table 2 to derive IAB backhaul link ACS and ACLR should be prioritized.

Proposal 3: Co-existence study for FR2 IAB should be prioritized.

# Reference

[1] R4-1903373, Overview on IAB

[2] RP-190182, Integrated Access and Backhaul for NR

[3] TR36.826: E-UTRA Relay radio transmission and reception

[4] TR38.874: Study on Integrated Access and Backhaul

[5] R4-1709987, Simulation assumptions of Co-existence study for 55dBm CPE in mmWave spectrum