3GPP TSG-RAN4 Meeting #90BIS 	R4-1903419

# Xian, China, 8 – 12 April, 2019

Title:	Reply LS on UE behavior on reception of channels or RS in the same OFDM symbol

Response to:	R1-1809890

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN WG4

To:	RAN WG1

Cc:

Contact Person:

Name:	Jin-Yup Hwang

E-mail Address:	jinyup.hwang@lge.com

1. Overall Description:

RAN4 has discussed RAN1 LS on UE behavior on reception of channels or RS in the same OFDM symbol for FR2. Since RAN4 has not defined core requirements for CSI-RS based L3 measurement, CSI-RS based L3 measurement is precluded in this LS.

RAN4 would like to inform RAN1 the conclusion in RAN4 as follows:

RAN4 kindly remind RAN1 that additional to the following answers regarding restrictions due to FR2 beam sweeping, when performing reception of channels/RS in group 1 with different subcarrier spacing than channels/RS group 2, the UE which does not support simultaneousRxDataSSB-DiffNumerology cannot receive both channels/RS group 1 and group 2 simultaneously.

- Answer to Q1

Case 1 & 4

  - In L3 measurement (SSB based), UE cannot receiver both RSs in group 1 and group 2 simultaneously within SMTC duration.

  - Except for L3 measurement case, UE could receive both RSs in group 1 and group 2 simultaneously if the RSs are QCL-Type D and QCL association is known to the UE. Otherwise UE could not receive both RSs in group 1 and group 2 simultaneously.

    - When UE needs to sweep Rx beam for RLM/BFD/BM-RS, UE may or may not receive FDMed RSs in the same OFDM symbol depending on UE implementation even if the RSs are QCLed.

Case 2 & 3

  - If the TCI of the PDCCH/PDSCH includes at least one of the CSI-RS resource IDs of the CSI-RS resource set and qcl-info of the CSI-RS is configured, UE may be allocated and receive a PDCCH/PDSCH that is overlapping with one or more symbols configured with CSI-RS resource(s) with repetition set to “OFF”. Otherwise, UE is not expected to PDCCH/PDSCH on CSI-RS symbol.

    - When UE needs to sweep Rx beam for QCLed source refereeing the CSI-RS, UE may or may not receive PDCCH/PDSCH transmitted in the same OFDM symbol depending on UE implementation even if CSI-RS and PDCCH/PDSCH are QCLed.

- Answer to Q2

For rate matching issue by scheduling restriction, RAN4 has already agreed in RAN4 NR-AH#4 meeting as follows:

  - It is up to RAN1 whether or not to define specific behavior for rate matching around scheduling restricted symbols.

2. Actions:

To RAN1:

ACTION: RAN4 respectfully asks RAN1 to take the above RAN4 conclusion into consideration for their further work.

3. Date of Next TSG-RAN4 Meetings:

3GPP RAN4 #91 		13 - 17 May 2019, 		, US

3GPP RAN4 #92 		26 - 30 August 2019, 		, Slovenia