3GPP TSG-RAN4 Meeting #90bis	R4-1903462

# Xian, China,  8th - 12th April 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.843 | CR | 0013 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TR 37.843: Update of RC description in subclause 10.5.2.3A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-04-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | With this CR we close the open issue related to the driectivity of the test antenna. In current verison the directivity was put within “[]”. Based on information in companion contribution R4-1903451, R4-1903452, this issue can now be resolved. This CR is created ontop of R4-1902517 approved last meeting. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct value is stated - Removal of [] |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If this CR is not approved the description related to RC test method is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Subclause 10.5.2.3A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### 10.5.2.3A	Reverberation chamber

##### 10.5.2.3A.1	General

A reverberation chamber (RC) is an electrically large shielded metal enclosure that employs one or several “stirring” methods to randomize the fields, such as moving paddles, turntables, etc. In this way, a large number of uncorrelated samples is obtained. The volume in the room where the field is well-stirred is the working volume. Here the E-field, averaged over an entire stirring cycle, is independent of the location in the room, i.e. the field is spatially uniform.

For a proper analysis of the measured data a sufficient number of uncorrelated samples is required. The auto-correlation function is used to calculate the offset between statistically uncorrelated samples using the following expression [33]:

rk=1σ2N-1n=0N-1xn- x xmodn+k,N- x

where the modulus operator mod(x,y) is the remainder of x/y, here performing  a circular shift of the measurement samples over a distance k. The symbols ⟨x ⟩ and σ= "std" (x) denote the average value and standard deviation. The threshold value for uncorrelated samples is defined as [33]:

rlim≤0.371-7.22/N0.64

The distance klimbetween uncorrelated samples is calculated as the minimum k-value satisfying rk≤rlim. The number of uncorrelated samples is calculated as:

Nind=Nklim

When properly designed, this facility can be used for non-directional antenna measurements, such as TRP. When measuring TRP of sources with a directive pattern, special care must be taken to characterize the working volume of the chamber, see subclause 10.5.2.3A.2.

##### 10.5.2.3A.2	Chamber characterization

The purpose of the chamber characterization is to ensure that the effect of a non-uniform field distribution in the chamber has a negligible influence on the measurement result when the EUT is placed in the working volume. Lack of chamber uniformity is a major contributor to measurement uncertainty in reverberation chambers and should be handled with care.

The uniformity test can be quite time consuming and the test can be performed separate from the EUT measurement. Due to the non-negligible size of BS equipment the EUT can have a significant influence on the uniformity. To take this effect into account, either the EUT itself must be present in the room during characterization or an absorber with dimensions equal or larger than the EUT must be placed at the EUT’s location in the room.

The characterization procedure consists of placing a reference transmitter antenna (REF TX ant) at different locations and with different orientations in the room. The actual mode of RC operation shall be used, including stirrer movement, EUT movement, diversity antenna usage, etc. The directivity of the REF TX antenna will influence the spatial uniformity of the room. Therefore, the directivity of the reference antenna shall be at least as high as the maximal estimated directivity of the EUT emissions to be measured or a 10 dB.

Figure 10.5.2.3A.2-1:	Setup for characterization of a reverberation chamber

The working volume shall be at least half a wavelength from the chamber walls and other electromagnetic reflective objects according to [34]. According to [33] this distance may be restricted to 0.75 m below 100 MHz. The number of positions and orientations to use depends on the chamber size and the directivity of the REF TX. Measurements made at positions and orientations at the edges ofthe working volume are used to characterize the chamber and derive certain components of uncertainty.

The exact number of positions and orientations remain for further study, but at least (3) uncorrelated locations should be used and (6) uncorrelated orientations per position when directive spurs are to be detected.

The REF TX antenna and the measurement receive antenna (RX) are connected to a Vector Network Analyzer (VNA).  For each location/orientation n of the REF TX, RC sample, and desired frequency f, the power transfer function

Pn=<S21>2

is measured. The explicit dependence on RC sample and frequency is not written out here. At least 250 uncorrelated samples shall be used per position/orientation. Using a lower number is not compatible with the underlying analysis on measurement uncertainty, see [33, 35].

The following tests are performed and shall be verified for each frequency:

- Uniformity of transfer function: For each location/orientation evaluate Pn. The standard deviation of these average values shall be below (1.5 dB).

- Dynamic range: The dynamic range of each Pn shall be at least 20 dB. The method to compute it is defined below.

- Uncorrelated samples: At least 250 uncorrelated samples shall be used.

It is important to note that spatial uniformity (and number of uncorrelated samples) in an RC is harder to achieve for more directive antenna patterns. As such, the measurement uncertainty of a sub-optimally configured room will be higher and additional measures to randomize the fields should be considered.

##### 10.5.2.3A.3	Calibration

To correctly determine the TRP by the EUT a calibration step to account for losses in cables, antennas, etc. is required. A reference PTF is determined at each frequency of interest by using a relevant reference TX antenna (REF TX). The chamber should be set up identically to the actual set up of the TRP measurement, which means the EUT has to be in the room, the same stirring procedure has to be used, etc. T

A minimum Nref = 1 reference measurement is required, but it is noted that more measurements can be used to estimate the reference PTF and that this will lead to a reduction of the uncertainty by a factor 1/Nref. The radiation efficiency, η, of the reference antenna can be assumed to be the value declared by the manufacturer or can be determined in a separate antenna efficiency measurement.

Calibration procedure

- Place the EUT and the REF TX in the working volume of the RC. Turn off EUT power and EUT controls. See Figure 10.5.2.3A.4-1 (a). If more positions are used to determine the reference PTF the REF TX should be placed at uncorrelated positions within the test volume.

- Set the stirrers and turntables in the mode of operation used in the chamber characterization.

- Set the sampling rate as in the chamber characterization.

- Connect the REF TX and the RX antenna with a calibrated Network analyser (NA)

- Measure the scattering parameters over a complete stirring cycle for each frequency of interest and for each position and orientation.

- Calculate the reference transfer function Pr=S21r2

- Calculate the mis-match efficiency 	M1r=1-S11r2.

##### 10.5.2.3A.4	Procedure

- (b)											(c)

Figure 10.5.2.3A.4-1:	The procedure for TRP measurements uses three consecutive setups: (a) Reference measurement, (b) ambient noise measurement (b), and (c) TRP measurement. The blue dots indicate the measurement planes.

Calculate Ambient Power Level

- Connect the RX antenna to a calibrated Spectrum analyser (SA) using the same cables as in the calibration step. Turn on the EUT control. Keep the EUT RF power off. Terminate the REF TX in a 50Ω load. See Figure 10.5.2.3A.4-1 (b).

- Measure the voltage data Uamb and calculate the ambient power level as Pamb=Uamb250

- Calculate the ambient TRP level as TRPamb=ηM1rPrPamb

Calculate EUT TRP

- Turn on the EUT RF power and measure the received voltage UEUT, see Figure 10.5.2.3A.4-1 (c).

- Calculate the EUT power as PEUT=UEUT250

- Calculate the TRP from the EUT as TRPEUT=η M1rPrPEUT.

- The following tests shall be performed on the measurement data:

  - The dynamic range TRPEUTTRPamb  must be at least 20 dB

  - The number of uncorrelated samples, calculated via the auto-correlation function, see [35], shall be at least 250.

Note: A reverberation chamber does not represent a real-life deployment for the EUT and can be considered as a hostile environment due to the potentially high field strengths. Therefore, it is important to make sure that the EUT is operating properly throughout the entire measurement.