3GPP TSG-RAN WG4 Meeting #90bis												  	         R4-1903524

Xian, China 8th – 12th of April, 2019

Source: 	Nokia, Nokia Shanghai Bell

Title: 					FR2 MPR due to OBW

Agenda Item: 			6.5.11.1

Document for:	Discussion

# 1	Introduction

In RAN#83 new PC3 MPR requirement for FR2 was approved and in this contribution we present simulation results of a study where it was investigated if new MPR scheme is viable.

# 2	Discussion

## 2.1	New PC3 MPR requirement for FR2

In RAN#83 new PC3 MPR requirement for FR2 was approved and is presented below

Table 6.2.2.3-1 MPRWT for power class 3, BWchannel ≤ 200 MHz

|  |  | MPRWT, BWchannel ≤ 200 MHz |  |
| --- | --- | --- | --- |
|  |  | RBstart ≥ Ceil(1/3 NRB) AND RBend ≤ Ceil(2/3 NRB) | RBstart < Ceil(1/3 NRB) OR RBend > Ceil(2/3 NRB) |
| DFT-s-OFDM | Pi/2 BPSK | 0.0 | 2.0 |
|  | QPSK | 0.0 | 2.0 |
|  | 16QAM | 3.0 | 3.5 |
|  | 64QAM | 5.0 | 5.5 |
| CP-OFDM | QPSK | 3.5 | 4.0 |
|  | 16QAM | 5.0 | 5.0 |
|  | 64QAM | 7.5 | 7.5 |

Table 6.2.2.3-2 MPRWT for power class 3, BWchannel = 400 MHz

|  |  | MPRWT, BWchannel = 400 MHz |  |
| --- | --- | --- | --- |
|  |  | RBstart ≥ Ceil(1/4 NRB) AND RBend ≤ Ceil(3/4 NRB) AND LCRB≤Ceil(1/4 NRB) | RBstart < Ceil(1/4 NRB) OR RBend > Ceil(3/4 NRB) OR LCRB>Ceil(1/4 NRB) |
| DFT-s-OFDM | Pi/2 BPSK | 0.0 | 3.0 |
|  | QPSK | 0.0 | 3.0 |
|  | 16QAM | 4.5 | 4.5 |
|  | 64QAM | 6.5 | 6.5 |
| CP-OFDM | QPSK | 5.0 | 5.0 |
|  | 16QAM | 6.5 | 6.5 |
|  | 64QAM | 9.0 | 9.0 |

## 2.2. Simulation assumptions

PA model calibrated to obtain MPR = 0 dB for 100 MHz, SCS = 120 kHz, DFT-S-OFDM, QPSK with allocation 20RB@23.

Power class 3 (max. 23 dBm transmission power)

Modulation: pi/2-BPSK, QPSK, 16-QAM, 64-QAM

Carrier suppression 25 dB

Image suppression 25 dB

ACLR according to 38.101-2

SEM according to 38.101-2

General spurious emission limit according to 38.101-2

EVM and in-band emissions according to 38.101-2

Occupied bandwidth limit according to 38.101-2, interpreted to always have the measurement bandwidth centered at the channel center.

## 2.3	Results

As an example, we present 2 figures for 50 MHz bandwidth, all the results are in Annex.

Based on the simulations new MPR scheme is viable and have amble margins.

# 3	Conclusion

In RAN#83 new PC3 MPR requirement for FR2 was approved and in this contribution we have presented simulation results of a study where it was investigated if new MPR scheme is viable. As a conclusion we can state that new MPR scheme is viable and have amble margins.

# Annex