3GPP TSG-RAN WG4 Meeting #90bis												  	         R4-1903533

Xian, China 8th – 12th of April, 2019

Source: 	Nokia, AMX

Title: 					TP to TR 36.716-02-01: 3A-3A-5A_BCS0

Agenda Item: 			7.2.3

Document for:	Approval

# 1	Introduction

This contribution is a TP for

| # CC for DL | CA configurations *Unless otherwise stated, the number of UL CC is one. | REL-indep. from | contact name, company | contact email | other supporting companies (min. 3) | status (new, ongoing, completed, stopped) | supported next level fallback modes (in DL and UL) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | 3A-3A-5A_BCS0 | Rel-11 | Israel Soto, AMX | israel.soto@americamovil.com | Nokia, Ericsson, Huawei | new | 1B_2CC_3A-3A_BCS0 (completed) 2B_2CC_3A-5A_BCS2 (completed) |

****************************** Start of the TP*********************************************

## 5.X	CA_3-3-5

### 5.X.1	Channel bandwidths per operating band for CA

Table 5.X.1-1: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_3A-3A-5A | - | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  | 50 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |

### 5.X.2 	Co-existence studies

Table 5.X.2-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 3 | 1710 | 1785 | 1805 | 1880 | 3420 | 3570 | 5130 | 5355 | 6840 | 7520 |
| 5 | 824 | 849 | 869 | 894 | 1648 | 1698 | 2472 | 2547 | 3296 | 3396 |

There is no ul harmonic issue for 3+5

Table 5.X.2-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| 3 | 1710 | 1785 | 1805 | 1880 | 3610 | 3760 | 5415 | 5640 | 7220 | 7520 |
| 5 | 824 | 849 | 869 | 894 | 1738 | 1788 | 2607 | 2682 | 3476 | 3576 |

Band 5 receiver 2nd harmonic overlaps band 3 Tx.

### 5.X.3	∆TIB and ∆RIB values

3+5 which has been specified is a high-low combination with harmonic issues hence dTib and dRib are as follows

Table 5.X.3-1: IB,c

| CA_3-3-5 | 3 | 0.3 |
| --- | --- | --- |
|  | 5 | 0.3 |

Table 5.X.3-2: R IB,c

| CA_3-3-5 | 3 | 0 |
| --- | --- | --- |
|  | 5 | 0 |

### 5.X.4 	REFSENS

Band 5 receiver 2nd harmonic overlaps band 3 Tx but no MSD is specified for fallback CA_3A-5A

****************************** End of the TP*********************************************