3GPP TSG-RAN WG4 Meeting #90bis												  	         R4-1903538

Xian, China 8th – 12th of April, 2019

Source: 	Nokia, AMX

Title: 					TP to TR 36.716-03-01: 3A-3A-5A-28A_BCS0

Agenda Item: 			7.3.3

Document for:	Approval

# 1	Introduction

This contribution is a TP for

| 3-3-5-28 | 4 | 3A-3A-5A-28A_BCS0 | Rel-11 | Israel Soto, AMX | israel.soto@ americamovil.com | Nokia, Ericsson, Huawei, Samsung | new | 3B_3CC_3A-5A-28A_BCS0 (completed) 2B_3CC_3A-3A-5A_BCS0 (new) 2B_3CC_3A-3A-28A_BCS0 (completed) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

****************************** Start of the TP*********************************************¨

## 5.X	3A-3A-5A-28A_BCS0

### 5.X.1	Channel bandwidths per operating band for CA

Table 5.X.1-1: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA

| E-UTRA CA Configuration | Uplink CA configurations |  | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 |  | 20 | Maximum aggregated bandwidth |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | MHz | MHz | MHz | MHz | MHz |  | MHz | [MHz] |  |  |  |
| CA_3A-3A-5A-28A |  | - | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  | 70 | 0 |  |
|  |  |  | 5 |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |  |  |  |

### 5.X.2 	Co-existence studies

Table 5.X.2-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 3 | 1710 | 1785 | 1805 | 1880 | 3420 | 3570 | 5130 | 5355 | 6840 | 7520 |
| 5 | 824 | 849 | 869 | 894 | 1648 | 1698 | 2472 | 2547 | 3296 | 3396 |
| 28 | 703 | 748 | 758 | 803 | 1406 | 1496 | 2109 | 2244 | 2812 | 2992 |

No uplink harmonic interference to any of the downlinks.

Table 5.X.2-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| 3 | 1710 | 1785 | 1805 | 1880 | 3610 | 3760 | 5415 | 5640 | 7220 | 7520 |
| 5 | 824 | 849 | 869 | 894 | 1738 | 1788 | 2607 | 2682 | 3476 | 3576 |
| 28 | 703 | 748 | 758 | 803 | 1516 | 1606 | 2274 | 2274 | 3032 | 3212 |

Band 5 receiver 2nd harmonic overlaps band 3 Tx.

### 5.X.3	∆TIB and ∆RIB values

dTib and dRib values for CA_3-5-28 can be re-used

Table 5.X.3-1: IB,c

| CA_3-3-5-28 | 3 | 0.3 |
| --- | --- | --- |
|  | 5 | 0.5 |
|  | 28 | 0.5 |

Table 5.X.3-2: R IB,c

| CA_3-3-5-28 | 3 | 0 |
| --- | --- | --- |
|  | 5 | 0 |
|  | 28 | 0 |

### 5.X.4 	REFSENS

Band 5 receiver 2nd harmonic overlaps band 3 Tx but no MSD is specified for fallback CA_3A-5A

****************************** End of the TP*********************************************