3GPP TSG-RAN WG4 Meeting #90bis	R4-1903576

# Xi’an, China, 8th April – 12th April, 2019

Source:	NTT DOCOMO, INC.

Title:	Directivity of beam-based direction for in-band TX requirements

Agenda item:	6.8.5.1

Document for:	Approval

- Introduction

This contribution discusses the directivity for calculation of TRP for in-band TX requirements using beam-based directions method.

2. Derivation of TRP for main beam

The “beam-based directions” method is one way to obtain TRP value for in-band TX requirements from EIRP measurement result and directivity of the signal as shown in the following equation [1]:

TRPEstimate= EIRPpeakDEUT	(1).

In the last RAN4 #90 meeting, a WF was approved to promote the discussion on applicability of the “beam-based direction” [2]. The discussion was mainly focused on how to apply the method to in-band unwanted emission requirement. In the WF, there are two proposals as shown below:

Proposal 1: The beam-based direction may be applied to estimate TRP of ACLR and OBUE if it can be demonstrated that DEUT of the desired signals and in-band unwanted emissions (ACLR and OBUE) is equivalent.

Proposal 2: Methodologies for demonstrating the equivalence of DEUT between the desired signals and in-band unwanted emissions (ACLR and OBUE) is FFS.

In the proposal 1, the method may be applied if the DEUT for ACLR and OBUE is equivalent to that for the desired signal. However, the way to define the DEUT for the desired signal has not been discussed.

Observation1: the way to define the DEUT for the desired signal has not been discussed.

We propose two considerable ways to define the DEUT. One is to derive the DEUT by “Rated beam EIRP” and “Rated carrier OTA BS output power” that are declared by manufacturers. The other is to create new declaration of DEUT that is equal to or below to the value derived by “Rated beam EIRP” and “Rated carrier OTA BS Power.”

Proposal 1:

Define DEUT for the desired signal by following one of two options:

- Option1: Calculate DEUT by subtracting PRated,c,TRP from PRated,c,EIRP, and nothing new declaration is added.

- Option2:  Declare DEUT for desired signal that is equal to or less than (PRated,c,EIRP - PRated,c,TRP)

In order to apply the DEUT defined by one of above options, the OTA BS output power shall be measured under the condition that BS emits PRated,c,EIRP and PRated,c,TRP because the DEUT is derived by PRated,c,EIRP and PRated,c,TRP.

Proposal 2: BS shall set to the condition to emit PRated,c,EIRP and PRated,c,TRP during the test of OTA BS output power if the “beam-based directions” method is chosen to derive TPR of desired signal.

3. DEUT for ACLR

The ACLR is an in-band TX unwanted emission requirement. The frequency range for ACLR is next to the desired signal and the maximum difference between the center frequencies of desired signal and ACLR is 200MHz for NR FR1 and 400 MHz for NR FR2. The antenna gain has frequency dependence. Fig. 1 shows the antenna gain of patch antenna for 2 GHz and Fig. 2 shows the antenna gain of patch antenna for 28 GHz.

Fig.1 Antenna gain of a patch antenna for 2 GHz

Fig.2 Antenna gain of a patch antenna for 28 GHz.

As shown on these figures, antenna gain varies depending on the frequency and the gain at the frequency 200MHz or 400 MHz away from 2 GHz or 28 GHz is smaller than the gain at 2 GHz or 28 GHz, respectively. Therefore, it is possible that actual DEUT for the ACLR frequency range is slightly smaller than that for the desired signal frequency range even though DEUT for ACLR is equivalent to DEUT for desired signal.

Observation2: DEUT for ACLR can be smaller than DEUT for desired signal.

If the actual directivity for ACLR is smaller than DEUT for the desired signal, the estimated TRP of ACLR could be smaller than actual TRP of ACLR when we uses the DEUT for the desired signal to calculate TRP. Since the ACLR is a regulatory requirement, it is not allowed to under estimate the value.

Observation3: ACLR is regulatory requirement and the value of ACLR shall not under estimate.

From these perspective, we propose to introduce a margin (DEUT,ACLR) in DEUT for ACLR (DEUT,ACLR) and DEUT,ACLR  is defined by DEUT,ACLR = DEUT - DEUT,ACLR.

Proposal3: Use DEUT,ACLR = DEUT - DEUT,ACLR for deriving TRP of ACLR when the “beam-based directions” method is used. And the criteria to define DEUT,ACLR is FFS

And also in this case, the OTA ACLR shall be measured under the condition that BS emits PRated,c,EIRP and PRated,c,TRP in order to apply the DEUT,ACLR because the DEUT in DEUT,ACLR is derived by PRated,c,EIRP and PRated,c,TRP.

Proposal 4: BS shall set to the condition to emit PRated,c,EIRP and PRated,c,TRP during the test of OTA ACLR if the “beam-based directions” method is chosen to derive TPR of desired signal.

4. DEUT for OBUE

The OBUE is also an in-band TX unwanted emission requirement. The frequency range for the OBUE is defined by fOBUE. For the case of BS type 2-O, fOBUE is 1.5 GHz. It is unsure whether the DEUT is flat or not for 1.5 GHz bandwidth. In particular, if the BS does not support the whole band, it is more difficult to achieve uniform DEUT for wide frequency range. Furthermore, 1.5 GHz is much wider than the frequency difference between the desired signal and ACLR. Therefore, it can be possible the difference between actual DEUT for the OBUE and that for desired signal is much larger than that for ACLR.

Observation4 : The DEUT for OBUE will be different from DEUT for ACLR.

Also in this case, it is not allowed to under estimate the value of OBUE because the OBUE is also a regulatory requirement.

Observation5: OBUE is regulatory requirement and the value of OBUE shall not under estimate

From these perspective, we propose to introduce some margin  (DEUT,OBUE) in DEUT for OBUE (DEUT,OBUE)and DEUT,OBUE is defined by DEUT,OBUE = DEUT - DEUT,OBUE.

Proposal5: Use DEUT,OBUE = DEUT - DEUT,OBUE for deriving TRP of OBUE when the “beam-based directions” method is used. And the criteria to define DEUT,OBUE is FFS.

The OTA OBUE shall be measured under the condition that BS emits PRated,c,EIRP and PRated,c,TRP in order to apply the DEUT,ACLR because the DEUT in DEUT,ACLR is derived by PRated,c,EIRP and PRated,c,TRP.

Proposal 6: BS shall set to the condition to emit PRated,c,EIRP and PRated,c,TRP during the test of OTA OBUE if the “beam-based directions” method is chosen to derive TPR of desired signal.

# 4	Summary

In this document, we made the following proposal based on observations.

Observation1: the way to define the DEUT for the desired signal has not been discussed.

Proposal 1:

Define DEUT for the desired signal by following one of two options:

- Option1: Calculate DEUT by subtracting PRated,c,TRP from PRated,c,EIRP, and nothing new declaration is added.

- Option2:  Declare DEUT for desired signal that is equal to or less than (PRated,c,EIRP - Prated,c,TRP)

Proposal 2: BS shall set to the condition to emit PRated,c,EIRP and PRated,c,TRP during the test of OTA BS output power if the “beam-based directions” method is chosen to derive TPR of desired signal.

Observation2: DEUT for ACLR can be smaller than DEUT for desired signal.

Observation3: ACLR is regulatory requirement and the value of ACLR shall not under estimate.

Proposal3: Use DEUT,ACLR = DEUT - DEUT,ACLR for deriving TRP of ACLR when the “beam-based directions” method is used. And the criteria to define DEUT,ACLR is FFS

Proposal 4: BS shall set to the condition to emit PRated,c,EIRP and PRated,c,TRP during the test of OTA ACLR if the “beam-based directions” method is chosen to derive TPR of desired signal.

Observation4 : The DEUT for OBUE will be different from DEUT for ACLR.

Observation5: OBUE is regulatory requirement and the value of OBUE shall not under estimate

Proposal5: Use DEUT,OBUE = DEUT - DEUT,OBUE for deriving TRP of OBUE when the “beam-based directions” method is used. And the criteria to define DEUT,OBUE is FFS.

Proposal 6: BS shall set to the condition to emit PRated,c,EIRP and PRated,c,TRP during the test of OTA OBUE if the “beam-based directions” method is chosen to derive TPR of desired signal.

References

[1] 3GPP TS38.141-2, v15.1.0., Annex I.10

[2] R4-1902272,	“WF on directivity for TRP measurements”, NTT DOCOMO and NOKIA, RAN4#90, Mar. 2019.