3GPP TSG RAN WG4 Meeting #90bis	R4-1903662

Xian, China, 8th  – 12th April, 2019

Agenda item:	8.4.1

Source: 	MediaTek Inc.

Title: 	On AGC settling time

Document for:	Discussion/Decision

Introduction

In RAN1#94 meeting [1], a LS of clarification of AGC settling time, TX/RX switching time, time/frequency error, and IBE model for NR V2X was agreed to send to RAN4 for feedback. The tentative assumptions for the simulation are

- AGC

  - Up to [15] us in FR1. Up to [10] us in FR2.

- TX/RX switching time

  - [13] us in FR1 and [7] us in FR2

- Time error

  - Up to [0.4] us between a UE and its synchronization reference

- Frequency error

  - Up to [0.1] PPM between a UE and its synchronization reference

In RAN4#90 meeting, WF on AGC settling time was agreed as [2]

- AGC settling time includes the following components (AGC setting time =a+b+c+d)

  - (a) time offset from the start of the OFDM symbol to account for maximum signal propagation delay from different UEs

  - (b) time to receive the window of samples used for estimating the gain/power

  - (c) time for estimating and programming the desired gain setting

  - (d) time for the gain setting taking into effect

- Open issues :

  - FFS for AGC settling time considering the different waveform design for AGC symbols in RAN1

    - At least, CP-OFDM will be considered

  - FFS if same or different AGC settling time shall be applied for different SCS

  - AGC settling time will be derived under assumption of below two options of RBs size restrictions:

    - Option1

      - At least 10RBs for 15kHz

      - At least 5 RBs for 30kHz

      - At least 3 RBs for 60kHz

    - Option2

      - At least 10RBs for all SCSs

- Further study feasible AGC settling time for FR1

  - Companies are encouraged to bring analysis on

    - AGC accuracy and RX power estimation duration depending on the CBW, V2X signal RB size, SCS

    - AGC settling time duration dependency on the number of V2X CCs and RX chains

  - Aim to send LS with final AGC settling time value to RAN1 in RAN4 #90bis

In this contribution, we would like to share our view on AGC settling time.

AGC Settling Time

AGC settling time includes the following components (AGC setting time =a+b+c+d)

- time offset from the start of the OFDM symbol to account for maximum signal propagation delay from different UEs

- time to receive the window of samples used for estimating the gain/power

- time for estimating and programming the desired gain setting

- time for the gain setting taking into effect.

In LTE V2X, AGC settling time is set as one symbol with DFT-S-OFDM waveform. However, CP-OFDM is supported as the waveform in NR V2X. Time (b) highly depends on the waveform of received samples. Different waveforms would result different Time (b) and corresponding AGC settling time. It is not straightforward to directly reuse LTE V2X AGC settling time for NR V2X. Further evaluation should be conducted.

Observation 1: Different waveforms would result different measurement time and corresponding AGC settling time.

Proposal 1: NR V2X AGC settling time should be carefully evaluated under CP-OFDM waveform at least.

Time (a)

Time (a) is the time offset from the start of the OFDM symbol to account for maximum signal propagation delay from different UEs. Maximum signal propagation delay from different UEs would depend on the maximum communication range of NR V2X and timing offset from synchronization source. It is usually smaller than one cyclic prefix duration. For example, maximum propagation delay is less than 1.67 us, if maximum communication range is 500m. However, AGC settling time should be counted by the received signal from antenna port and maximum propagation delay should be considered in RAN1 design aspect thus be not taken into account in AGC settling time.

Proposal 2: RAN4 should not consider maximum signal propagation delay in AGC settling time.

Time (b)

Time (b) is the time to receive the window of samples used for estimating the gain/power and depends on the received waveform. In order to evaluate Time (b) with different waveforms, we carry out some simulations with CP-OFDM signal and random sequence as received samples. Only one UE signal is considered with subcarrier spacing 15 kHz and bandwidth 20 MHz. We assume the requirement of the probability of power difference (Pvar) between estimated power and true power larger than 3 dB is less than 10-3, i.e., Pr( |Pvar| > 3dB ) ≤ 10-3.

Fig. 1 shows Time (b) of received OFDM samples with different RBs and QPSK modulation scheme under TDL-C 300ns channel. Time (b) is smaller when more RBs are allocated because the time correlation of the samples are higher. If 5RBs are used for gain estimation, Time (b) is more than 30 us. Time (b) is about 5 us, if wideband CP-OFDM signal (100 RBs) is used. The measurement time with higher order modulation order is similar to that with QPSK, since it is more sensitive to FFT size and signal bandwidth than modulation order in CP-OFDM case.

Observation 2: Measurement time is smaller, if more OFDM RBs are used.

Figure 1. Time (b) for received CP-OFDM samples with different RBs and QPSK modulation under TDL-C 300ns channel.

For RB size restriction, there are two options:

    - Option1

      - At least 10RBs for 15kHz

      - At least 5 RBs for 30kHz

      - At least 3 RBs for 60kHz

    - Option2

      - At least 10RBs for all SCSs.

In NR V2X, the received signal for AGC settling consists of control channel signal and data channel signal. The minimum payload size should not vary with different SCSs. Therefore, we only consider Option 2 with fixed RBs for all SCSs.

Proposal 3: Only Option 2: at least 10 RBs for all SCSs should be considered in AGC settling time evaluation.

Time (b) also depends on the samples number of received signal. More samples used would result in smaller Time (b). Same samples number would result in same Time (b) in samples. Furthermore, higher SCS has smaller sample duration. In other words, Time (b) is inversely proportional to SCS with fixed evaluation RBs. We can also see the same effect from simulation results as described above. Time (b) can be summarized as Table 1.

Proposal 4: Time (b) is inversely proportional to SCS with fixed RBs of CP-OFDM signals.

Table 1. Time (b) for received CP-OFDM samples with 10/20 RBs and QPSK modulation under TDL-C 300ns channel.

|  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| --- | --- | --- | --- |
| 10 RBs | 25 us | 12.5 us | 6.25 us |
| 20 RBs | 15 us | 7.5 us | 3.75 us |

When using random sequence as the received samples, Time (b) is about 1.5 us which is less than 1/20 of the Time (b) by CP-OFDM signals with 5RBs as shown in Fig. 2. Since CP-OFDM signal has more dynamic power over time-domain than a random sequence with QPSK modulation, more CP-OFDM samples are needed to obtain averaged power with less variation. In other words, a time-domain sequence with lower PAPR or constant envelope can effectively reduce AGC measurement time.

Observation 3: Signals with lower PAPR or constant envelope can effectively reduce AGC settling time.

Figure 2. Time (b) for received random sequence under TDL-C 300 ns channel.

Time (c)

Time (c) is the time for estimating and programming the desired gain setting, which does not depend on different SCSs. It is common to use hardware to estimate the gain from received signals and programme the desired gain setting. From our hardware evaluation, Time (c) can be less than 1 us.

Proposal 5: Time (c) could be less than 1 us.

Time (d)

Time (d) is the time for the gain setting taking into effect and does not depend on different SCSs. Time (d) is highly dependent on the response property of amplifier. Based on our evaluation of commercial amplifiers, Time (d) can be less than 1 us.

Proposal 6: Time (d) could be less than 1 us.

Summary of AGC settling time

For SCS 15 kHz and random sequence as AGC training signal, AGC settling time can be less than 3.5 us (1.5+1+1 us). However, AGC settling time would be about 27 us (25+1+1 us), if CP-OFDM signal with 10 RBs is used for AGC training with SCS 15 kHz. Note that one OFDM symbol duration with SCS 15 kHz is about 72 us, if the symbol is at the beginning of one slot. Then AGC settling time can be less than one half OFDM symbol duration whether OFDM signal or random sequence is used for AGC training.

From the above analysis, it would waste a lot of resource and result in higher overhead ratio and lower spectral efficiency if one OFDM symbol is used for AGC training. In order to reduce the overhead ratio and enhance the system spectral efficiency, one half of the first OFDM symbol duration can be used for AGC settling with SCS 15 kHz at least. If random sequence is used, the overhead ratio can be effectively reduced.

Observation 4: AGC settling time can be less than one half OFDM symbol duration for all SCSs.

Based on the above analysis, we summarized AGC settling time in Table 2.

Table 2. AGC settling time for received CP-OFDM/random signal and QPSK modulation under TDL-C 300ns channel.

|  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| --- | --- | --- | --- |
| CP-OFDM within 10 RBs | 27 us | 14.5 us | 8.25 us |
| CP-OFDM within 20 RBs | 17 us | 9.5 us | 5.75 us |
| Random sequence | 3.5 us | 3.5 us | 3.5 us |

Proposal 7: To have better spectral efficiency, shorter AGC settling time should be supported in NR V2X.

Proposal 8: AGC settling time with CP-OFDM and random sequence can be as the following values.

|  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| --- | --- | --- | --- |
| CP-OFDM with 10 RBs | 27 us | 14.5 us | 8.25 us |
| CP-OFDM with 20 RBs | 17 us | 9.5 us | 5.75 us |
| Random sequence | 3.5 us | 3.5 us | 3.5 us |

Conclusions

The following summarizes the observations and proposals in this contribution.

Observation 1: Different waveforms would result different measurement time and corresponding AGC settling time.

Observation 2: Measurement time is smaller, if more OFDM RBs are used.

Observation 3: Signals with lower PAPR or constant envelope can effectively reduce AGC settling time.

Observation 4: AGC settling time can be less than one half OFDM symbol duration for all SCSs.

Proposal 1: NR V2X AGC settling time should be carefully evaluated under CP-OFDM waveform at least.

Proposal 2: RAN4 should not consider maximum signal propagation delay in AGC settling time.

Proposal 3: Only Option 2: at least 10 RBs for all SCSs should be considered in AGC settling time evaluation.

Proposal 4: Time (b) is inversely proportional to SCS with fixed RBs of CP-OFDM signals.

Proposal 5: Time (c) could be less than 1 us.

Proposal 6: Time (d) could be less than 1 us.

Proposal 7: To have better spectral efficiency, shorter AGC settling time should be supported in NR V2X.

Proposal 8: AGC settling time with CP-OFDM and random sequence can be as the following values.

|  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| --- | --- | --- | --- |
| CP-OFDM with 10 RBs | 27 us | 14.5 us | 8.25 us |
| CP-OFDM with 20 RBs | 17 us | 9.5 us | 5.75 us |
| Random sequence | 3.5 us | 3.5 us | 3.5 us |

References

- 3GPP RAN1 #94 August 2018 Chairman’s Notes

- R4-1902513, WF on the NR V2X RF parameters and transient period, LGE