3GPP TSG-RAN WG4 Meeting #90bis	R4-1903895

Xi’an, China, 8 – 12 April 2019

Agenda Item:	6.12.1.3.2

Source:	Ericsson

Title:	Simulation results of NR PBCH demodulation

Document for:	Discussion

# 1	Introduction

RAN4#90 endorsed the draft CR for NR PBCH demodulation requirements in Rel-15 [1]. We provide the simulation results based on the parameters in the draft CR.

# 2	Discussion

## 2.1	Simulation parameters

Table 1 and Table 2 show the common setup and simulation assumption of PBCH demodulation requirements respectively based on the CR [1].

Table 1	Common setup for PBCH demodulation requirements.

| Parameters | Unit | Values |
| --- | --- | --- |
| Cell ID |  | 0 |
| CP length |  | Normal |
| Number of SS/PBCH blocks within an SS burst set periodicity |  | 1 |
| SS/BPCH block index | ms | 0 |
| SS burst periodicity | bits | 20 |
| MIB size (without CRC) | bits | 24 |
| PBCH payload size (including CRC=24bits and 8bits of SS/PBCH block location information) |  | 56 |

Table 2	Simulation assumption for PBCH demodulation requirements.

| Parameters | Test 1 | Test 2 | Test 3 | Test 4 |
| --- | --- | --- | --- | --- |
| Frequency range | FR1 | FR1 | FR2 | FR2 |
| Channel BW | 10MHz | 40MHz | 100MHz | 100MHz |
| SS/PBCH SCS | 15kHz | 30kHz | 120kHz | 240kHz |
| SS Block pattern (Note 1) | Case A | Case B Case C | Case D | Case E |
| Antenna configuration | 1x2 1x4 | 1x2 1x4 | 1x2 | 1x2 |
| Propagation channel | TDLC300-100 | TDLA30-10 | TDLA30-300 | TDLA30-75 |
| Note 1: Depending on the operating band. Refers to TS38.101-1 Table 5.4.3.3-1 for which band uses which pattern |  |  |  |  |

## 2.2	Simulation results for 2Rx

Figure 1 shows the simulation results of FR1 PBCH and Figure 2 shows the simulation results of FR2 PBCH. Both the figures show the effective PBCH decoding error rate with the assumption 1) UE knows the SSB index and 2) UE does not know the SSB index. For case 1), the effective PBCH demodulation performance is given by:

Pm-bch (with SSB index knowledge) := PBCH_BLER.

On the other hand, the effective PBCH demodulation performance with case 2) is given by:

Pm-bch (without SSB index knowledge) := (1-Pssb) + Pssb * PBCH_BLER,

where Pssb is the SSB index acquisition success rate and PBCH_BLER is the PBCH BLER with the assumption UE knows the DMRS sequence (= SSB index).

Figure 1	Simulation results of PBCH for FR1 (test 1 and test 2).

Figure 2	Simulation results of PBCH for FR2 (test 3 and test 4).

## 2.3	Simulation results for 4Rx

Figure 3 shows our simulation results with 4Rx according to the latest TS38.101-4 [1].

Figure 3	Simulation results of PBCH for FR1 (test 1 and test 2).

## 2.4	Summary of simulation results

Table 3 summarize the simulation results to achieve 1% Pm-bch with/without SSB block index knowledge.

Table 3	Summary of simulation results

|  | Test 1 (FR1, SSB SCS=15kHz) | Test 2 (FR1, SSB SCS=30kHz) | Test 3 (FR2, SSB SCS=120kHz) | Test 4 (FR2, SSB SCS=240kHz) |
| --- | --- | --- | --- | --- |
| 2Rx with SSB index knowledge | -10.9 | -9.1 | -10.6 | -9.5 |
| 2Rx without SSB index knowledge | -9.5 | -7.7 | -7.8 | -7.3 |
| 4Rx with SSB index knowledge | -13.7 | -12.7 | N/A | N/A |
| 4Rx without SSB index knowledge | -12.7 | -11.7 | N/A | N/A |

# References

- R4-1902614, “CR on UE demodulation and CSI requirements for 38.101-4”, Samsung.