3GPP TSG-RAN WG4 Meeting #90bis 	R4-1903967

# Xi’an, China, 8th April – 12th April, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-02 | CR |  | rev |  | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS38.141-02 on TAE requirement (Section 6.6.4) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-04-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - As discussed in the companion contribution R4-1903873, NR TX diversity should be updated as NR TX polarization as there are no SFBC or STTD like TX diversity defined for NR; - For FR1+FR2 inter-band CA requested right now, however there are no such kind of TAE requirement is defined yet. As TAE performance of inter-band CA is mainly dependent on the synchronization signal recovery quality in the timing network, so it should be the same for FR1+FR1 inter-band CA and FR1+FR2 inter-band CA; - As there are no inter-band CA for FR2+FR2, it should be also reflected in the TS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the TX diversity as TX polarization; - Update the inter-band CA for FR1 with FR1+FR1 and FR1+FR2 cases; - Update the inter-band CA for FR2 with FR1+FR2 case; - Some editorial corrections; |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TAE requirements would be not clear enough. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 6.6.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

6.6.4	OTA time alignment error

6.6.4.1	Definition and applicability

This requirement shall apply to frame timing in TX polarization, MIMO transmission, carrier aggregation and their combinations.

Frames of the NR signals present in the radiated domain are not perfectly aligned in time. In relation to each other, the RF signals present in the radiated domain may experience certain timing differences.

For a specific set of signals/transmitter configuration/transmission mode, the OTA Time Alignment Error (OTA TAE) is defined as the largest timing difference between any two different NR signals. The OTA time alignment error requirement is defined as a directional requirement at the RIB and shall be met within the OTA coverage range.

6.6.4.2	Minimum requirement

The minimum requirement for BS type 1-O is in TS 38.104 [2], subclause 9.6.3.2.

The minimum requirement for BS type 2-O is in TS 38.104 [2], subclause 9.6.3.3.

6.6.4.3	Test purpose

To verify that the OTA time alignment error is within the limit specified by the minimum requirement.

6.6.4.4	Method of test

6.6.4.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested for single carrier: M; see subclause 4.9.1.

Base Station RF Bandwidth positions to be tested for multi-carrier and/or CA:

-	MRFBW in single-band operation, see subclause 4.9.1;

-	BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band operation, see subclause 4.9.1.

Directions to be tested: OTA coverage range reference direction (D.35).

Polarizations to be tested: For dual polarized systems the requirement shall be tested and met considering both polarizations. If the measurement antenna does not support dual polarization, time alignment error shall be measured under the condition that measurement antenna is aligned between the BS polarizations such that it receives half the power from each polarization.

6.6.4.4.2	Procedure

1)	Place the  BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the  BS with the test system.

3)	Move the  BS on the positioner in order that the direction to be tested aligns with the test antenna.

4)	Configure the beamforming settings of the  BS according to the direction of the testing.

5)	Set the BS type 1-Oto transmit NR-FR1-TM1.1 or any DL signal using TX polarization, MIMO transmission or carrier aggregation, using the configuration with the minimum number of cells and reference signals.

Set the BS type 2-O to transmit NR-FR2-TM 1.1 or any DL signal using TX polarization, MIMO transmission or carrier aggregation, using the configuration with the minimum number of cells and reference signals.

NOTE:	For TX polarization and MIMO transmission, different ports may be configured in NR-FR1-TM1.1 and NR-FR2-TM 1.1 (using p = 0 and 1).

For an  BS declared to be capable of single carrier operation only, set the  BS to transmit according to the applicable test configuration in subclause 4.8 using the corresponding test model at manufacturer's declared rated output power, PRated,c,TRP.

If the  BS supports intra band contiguous or non-contiguous Carrier Aggregation set the  BS to transmit using the applicable test configuration and corresponding power setting specified in subclauses 4.7.2 and 4.8.

If the  BS supports inter band carrier aggregation set the BR BS to transmit, for each band, a single carrier or all carriers, using the applicable test configuration and corresponding power setting specified in subclauses 4.7.2 and 4.8.

For BS type 1-O declared to be capable of multi-carrier operation, set the BS to transmit according to  the applicable test signal configuration and corresponding power setting specified in subclauses 4.7.2 and 4.8 using the corresponding test model on all carriers configured.

For BS type 2-O declared to be capable of multi-carrier operation, set the BS to transmit according to the applicable test signal configuration and corresponding power setting specified in subclauses 4.7.2 and 4.8 using the corresponding test model on all carriers configured.

6)	Measure the time alignment error between the different reference symbols on different beams on the carrier(s).

In addition, for a multi-band RIB, the following steps shall apply:

7)	For a multi-band RIB and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

6.6.4.5	Test Requirement

6.6.4.5.1	BS type 1-O

For MIMO or TX polarization transmissions, at each carrier frequency, OTA TAE shall not exceed 90 ns.

For intra-band contiguous carrier aggregation, with or without MIMO or TX polarization, OTA TAE shall not exceed 285 ns.

For intra-band non-contiguous carrier aggregation, with or without MIMO or TX polarization, OTA TAE shall not exceed 3.025 µs.

For inter-band carrier aggregation including FR1+FR1 and FR1+FR2, with or without MIMO or TX polarization, OTA TAE shall not exceed 3.025 µs.

6.6.4.5.2	BS type 2-O

For MIMO or TX polarization transmissions, at each carrier frequency, OTA TAE shall not exceed 90 ns.

For intra-band contiguous carrier aggregation, with or without MIMO or TX polarization, OTA TAE shall not exceed 155 ns.

For intra-band non-contiguous carrier aggregation, with or without MIMO or TX polarization, OTA TAE shall not exceed 3.025 µs.

For inter-band carrier aggregation including FR1+FR2, with or without MIMO or TX polarization, OTA TAE shall not exceed 3.025 µs.

<Start of modified section>