3GPP TSG-RAN4 WG4 Meeting #90Bis	R4-1904230

# Xi'an, China,  8 - 12 April, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-2 | CR | - | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR: Addition of the Test Tolerance for performance requirements in TS 38.141-2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-03-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The derivation of test requirements for performance tests after considering the test tolerance is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added Annex C.3 for derivation of test requirements for performance tests. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How to derive the test requirements for performance tests by considering the test tolerance will still be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex C.3 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Annex C (informative):
Test tolerances and derivation of test requirements

The test requirements explicitly defined in this specification have been calculated by relaxing the minimum requirements of the core specification TS 38.104 [2] using the test tolerances (TT) defined here. When the TT value is zero, the test requirement will be the same as the minimum requirement. When the TT value is non-zero, the test requirements will differ from the minimum requirements, and the formula used for this relaxation is given in the following tables.

<Unchanged Sections Skipped>

C.3	Measurement of Performance Requirements

Table C.3-1: Derivation of test requirements (FR1 OTA performance tests)

| Test | Minimum Requirement in TS 38.104 [2] |  | Test Tolerance (TTOTA) | Test requirement in the present document |  |
| --- | --- | --- | --- | --- | --- |
| 8.2.1 Performance requirements for PUSCH with transform precoding disabled | See TS 38.104 [2] subclause 11.2.1.1 |  | [0.8]dB | Formula: SNR + TT T-put limit unchanged |  |
| 8.2.2 Performance requirements for PUSCH with transform precoding enabled | See TS 38.104 [2] subclause 11.2.1.2 |  | [0.8]dB | Formula: SNR + TT T-put limit unchanged |  |
| 8.3.1 Performance requirements for PUCCH format 0 | See TS 38.104 [2] subclause 11.3.1.2 |  | [0.6]dB | Formula: SNR + TT False ACK limit unchanged Correct ACK limit unchanged |  |
| 8.3.2 Performance requirements for PUCCH format 1 | See TS 38.104 [2] subclause 11.3.1.3 | [0.6]dB | Formula: SNR + TT False ACK limit unchanged Correct ACK limit unchanged |
| 8.3.3 Performance requirements for PUCCH format 2 | See TS 38.104 [2] subclause 11.3.1.4 | [0.6]dB | Formula: SNR + TT False ACK limit unchanged Correct ACK limit unchanged UCI BLER limit unchanged |
| 8.3.4 Performance requirements for PUCCH format 3 | See TS 38.104 [2] subclause 11.3.1.5 | [0.6]dB | Formula: SNR + TT UCI BLER limit unchanged |
| 8.3.5 Performance requirements for PUCCH format 4 | See TS 38.104 [2] subclause 11.3.1.6 | [0.6]dB | Formula: SNR + TT UCI BLER limit unchanged |
| 8.4.1 PRACH false alarm probability and missed detection | See TS 38.104 [2] subclause 11.4.1 | [0.6]dB for fading cases [0.3]dB for AWGN cases | Formula: SNR + TT PRACH False detection limit unchanged PRACH detection limit unchanged |

Table C.3-2: Derivation of test requirements (FR2 OTA performance tests)

| Test | Minimum requirement in TS 38.104 [2] | Test Tolerance (TTOTA) | Test requirement in the present document |
| --- | --- | --- | --- |
| 8.2.1 Performance requirements for PUSCH with transform precoding disabled | See TS 38.104 [2] subclause 11.2.2.1 | TBD | TBD |
| 8.2.2 Performance requirements for PUSCH with transform precoding enabled | See TS 38.104 [2] subclause 11.2.2.2 | TBD | TBD |
| 8.3.1 Performance requirements for PUCCH format 0 | See TS 38.104 [2] subclause 11.3.2.2 | TBD | TBD |
| 8.3.2 Performance requirements for PUCCH format 1 | See TS 38.104 [2] subclause 11.3.2.3 | TBD | TBD |
| 8.3.3 Performance requirements for PUCCH format 2 | See TS 38.104 [2] subclause 11.3.2.4 | TBD | TBD |
| 8.3.4 Performance requirements for PUCCH format 3 | See TS 38.104 [2] subclause 11.3.2.5 | TBD | TBD |
| 8.3.5 Performance requirements for PUCCH format 4 | See TS 38.104 [2] subclause 11.3.2.6 | TBD | TBD |
| 8.4.1 PRACH false alarm probability and missed detection | See TS 38.104 [2] subclause 11.4.2 | TBD | TBD |