3GPP TSG-RAN WG4 Meeting #90bis	R4-1904383

Xi’an, Shaanxi, China, 8 – 12 April 2019

Source:	Nokia, Nokia Shanghai Bell

Title:	Initial system simulation results for Rel-16 HST

Agenda item:	7.15.3

Document for:	Discussion

# 1	Introduction

During the RAN4#90 meeting, the way forward on HST enhancement for RRM [1] was approved, which captured some agreements and open issues.

This document outlines initial system simulation assumptions for performance evaluation of enhanced RRM mobility requirements under Rel-16 HST scenarios. We have used the simulations assumptions that were used for the Rel-14 HST investigations [2].

# 2	Discussion

The system level simulations performed during Rel-14 were based on assumption similar to those in Table 1 (e.g. UE velocity has been changed). In this paper we have used the assumptions in table to perform a set of preliminary simulations.

We propose that for coming system level simulations, they would be based and conducted based on the simulation assumptions listed in Table 1.

Adapt the simulation assumptions in Table 1 for further system level simulations for Rel-16 HST.

Table 1: System level simulation assumptions

| Parameters | Value | Comments |
| --- | --- | --- |
| Cell layout | 6 cells | 3 sites; 2 cells per site 2 wrap-around areas from the left and from the right |
| Network synchronization | Synchronized |  |
| Inter-site distance (Ds) | 1000 m |  |
| Minimum distance between eNB and railroad track (Dmin) | 50 m |  |
| Carrier frequency | 2.0 GHz |  |
| Antenna deployment | 11 degrees horizontal 6 degrees down-tilt |  |
| Antenna configuration | 3D antenna, 1x2 SIMO in DL Isotropic, 1x2 SIMO in UL |  |
| Receiver types | MRC in DL MRC in UL |  |
| Antenna gain | BS: 17dBi UE: 0dBi |  |
| Antenna height | BS: 35m UE: 1.5m |  |
| User speed | 500km/h (138.9m/s) |  |
| UE distribution | All UEs are generated consecutively in the left most point with the scenario in 1.44 seconds (500 km/h, 30 UE/s). | This is analogical of generating UEs in the train of 200m length. |
| Number of UEs | 63 |  |
| User mobility model | Constant speed, wrap around |  |
| Distance-dependent path loss | Simple logarithmic | 128.1+37.6log10(d) dB |
| Penetration loss | 20 dB |  |
| Slow fading/Shadowing | Not used |  |
| Fast fading | Not used |  |
| Traffic type | No data traffic in UL and DL | One packet is sent only in the beginning and in the end of simulations |
| Background load | Fixed as 0, 25%, 50%, 75%, 100% |  |
| Cell detection delay | PSS/SSS detection delay shall be within 5* DRX cycles when SINR>=-6dB |  |
| L1 measurement period | Measurement period shall be within 3* DRX cycles when SINR>=-6dB |  |
| T310 | 1s |  |
| T312 | Disabled |  |
| HO | A3-based |  |
| A3 event parameters | Hysteresis: 0 dB Threshold: 0, 3 dB Time To Trigger: 0 ms |  |
| HO Preparation delay | Constant delay of 50ms |  |
| L3 filtering | OFF |  |
| DRX | Long cycle values: OFF, 40ms, 80ms, 160ms, 320ms, 640ms | Other parameters: DRX.inactivityTimeInTTIs: 10 DRX.onDurationInTTIs: 5 |
| RACH parameters | Constant delay of 40ms |  |
| RRC measurement quantity | RSRP |  |
| RSRP Measurement Error Std. | 2dB | Random error of +-2dB with a normal distribution is added to the RSRP measurement of each cell. |
| Threshold Qout,SNR | -8 dB |  |
| Threshold Qin,SNR | -6 dB |  |
| RRC messages sent over the air | HO command, HO complete Measurement report Re-Establishment (request, response, complete) |  |
| RLM evaluation period | 10 DRX cycles | Per requirements in 36.133 |
| Simulation time | 10 min |  |

The performance metrics used in the simulation are as follows:

- HO failure ratio: number of failed connection attempts (HOF+RLF) over the total number of connection attempts (HO+ HOF+RLF)

- ping-pong HO ratio: the number of HOs happening back and forth between sectors of the same eNB or between eNBs within 1 second

- HO per UE per second ratio: the number of HOs per UE per second

The simulation results with different cell detection and measurement options are shown, with different DRX cycles, A3 threshold options and background traffic load. Figures 1-3 are for HO failure ratio, ping-pong HO ratio and HO per UE per second ratio, respectively.

Figure 1: HO failure ratio

Figure 2: Ping-pong HO ratio

Figure 3: HO per UE per second ratio

From Figure 1, it can be seen that the HO performance degrades to an unacceptable level with DRX greater than 160 ms for 75% loading and all the considered A3 threshold options.

Another interesting observation is in Figure 2, where we can find there are ping-pongs in the HST scenario, which is worse for small DRX values and low A3 thresholds. This is also shown in Figure 3. One reason is that in HST scenario the HO related parameters (Hysteresis, Threshold and TTT) are usually set aggressively, which will make the filtered RSRP for serving and neighbouring cells become closer to each other and less reliable in the middle between the sectors.

Observation 1: HO failure rates increase with long DRX cycle length (as expected)

Observation 2: A3 threshold has little or no effect on HO failure rates but is effective in reducing ping-pong HO behaviour and the number of HO per UE per second

# 3	Conclusions

The document has provided initial system simulation results for Rel-16 HST scenario at 500 km/h for connected mode.

The following observations can be made:

Observation 1: HO failure rates increase with long DRX cycle length

Observation 2: A3 threshold has little or no effect on HO failure rates but is effective in reducing ping-pong behaviour and the number of HO per UE per second

The preliminary simulation studies carried out in this document can be used for further studies in RAN4. For this purpose, we propose:

- Adapt the simulation assumptions in Table 1 for further system level simulations for Rel-16 HST.

# References

- R4-1902520, Way forward on HST enhancement for RRM, Huawei, HiSilicon

- R4-168282, System level simulation results for HST, Nokia, Alcatel-Lucent Shanghai Bell