# 3GPP TSG-RAN WG4 Meeting #90bis                                                          R4-1904714

# XiAN, China, 8th - 12th April, 2019

Agenda Item:	6.12.2.1

Source:	ZTE Corporation

Title:	Updated excel script for deriving NR performance requirements

Document for:	Approval

# Introduction

In RAN4 #90bis, how to reduce TBDs derived according to the agreed rule for NR performance requirements are intensively discussed and an update was agreed by extending the threshold of impairment result span from 3dB to 4dB [1]. And also it was discovered the previous excel script [2] continues the iteration when the selected impairment result span is larger than the threshold, which is not exactly what the agreed procedure assumes. Therefore we update the excel script accordingly.

# Discussion

The script is shown below:

--------------------------------------

Function DerivingPerfReq(SimVal_in As Range) As Double

Dim i, N, M As Long

Dim Ideal(50) As Double

Dim Impairment(50) As Double

Dim Min_ideal, Max_ideal, Min_impairment, Max_impairment As Double

Dim Min_ideal_pos, Max_ideal_pos, Min_impairment_pos, Max_impairment_pos As Integer

Dim Avg_ideal, Avg_impairment As Double

Dim SimVal(100) As Double

Dim Avg As Double

Dim Avg_pos As Double

Dim Company(50) As Integer

Dim row_index, col_index As Integer

' Validating inputs

N = SimVal_in.Count

If (Int(N / 2) * 2) <> N Then

'MsgBox ("Simulation results must be provided in pairs of ideal and impairment results")

DerivingPerfReq = -101

Exit Function

End If

row_index = SimVal_in.Row

col_index = SimVal_in.Column

M = 0 ' Number of effective pairs

For i = 0 To N / 2 - 1

If IsEmpty(SimVal_in.Value2(1, 2 * i + 1)) Then

ElseIf IsEmpty(SimVal_in.Value2(1, 2 * i + 2)) Then

ElseIf IsNumeric(SimVal_in.Value2(1, 2 * i + 1)) And IsNumeric(SimVal_in.Value2(1, 2 * i + 2)) Then

SimVal(2 * M) = SimVal_in(2 * i + 1)

SimVal(2 * M + 1) = SimVal_in(2 * i + 2)

M = M + 1

End If

Next i

If M < 3 Then

'MsgBox ("Not enough Simulations: at least 3 companies")

DerivingPerfReq = -100

Exit Function

End If

Do While M >= 3

Min_ideal = SimVal(0)

Max_ideal = SimVal(0)

Min_impairment = SimVal(1)

Max_impairment = SimVal(1)

Min_ideal_pos = 0

Max_ideal_pos = 0

Min_impairment_pos = 1

Min_impairment_pos = 1

Avg_ideal = SimVal(0)

Avg_impairment = SimVal(1)

' Find the spans and average for each

For i = 1 To M - 1

If Min_ideal > SimVal(2 * i) Then

Min_ideal = SimVal(2 * i)

Min_ideal_pos = i

End If

If Max_ideal < SimVal(2 * i) Then

Max_ideal = SimVal(2 * i)

Max_ideal_pos = i

End If

If Min_impairment > SimVal(2 * i + 1) Then

Min_impairment = SimVal(2 * i + 1)

Min_impairment_pos = i

End If

If Max_impairment < SimVal(2 * i + 1) Then

Max_impairment = SimVal(2 * i + 1)

Max_impairment_pos = i

End If

Avg_ideal = Avg_ideal + SimVal(2 * i)

Avg_impairment = Avg_impairment + SimVal(2 * i + 1)

Next i

Avg_ideal = Avg_ideal / M

Avg_impairment = Avg_impairment / M

' Check if ideal span is within 2 dB

If (Max_ideal - Min_ideal) <= 2 Then

Exit Do

Else: ' remove outlier

' 1) find outlier

Avg = Abs(SimVal(0) - Avg_ideal)

Avg_pos = 0

For i = 1 To M - 1

If Abs(SimVal(2 * i) - Avg_ideal) > Avg Then

Avg = Abs(SimVal(2 * i) - Avg_ideal)

Avg_pos = i

End If

Next i

' 2) excluding the outlier

For i = Avg_pos To M - 2

SimVal(2 * i) = SimVal(2 * i + 2)

SimVal(2 * i + 1) = SimVal(2 * i + 3)

Next i

' M dec

M = M - 1

End If

Loop ' iterated until ideal result span is within the threshold

' either M < 3 or ideal results span met

If M < 3 Then

DerivingPerfReq = -103 ' Not enough input after outlier(s) removal

'check span for impairment results, threshold changed from 3dB to 4dB

ElseIf (Max_impairment - Min_impairment) <= 4 Then

DerivingPerfReq = Round(Avg_impairment, 1)

Else:

DerivingPerfReq = -102 ' Too large span for impairment results

End If

End Function

# Reference

[1] R4-19xxxx WF on requirement SNR derivation procedure, Nokia, ZTE

[2] R4-1902370 Excel script for deriving NR performance requirements, ZTE