3GPP TSG-RAN WG4 Meeting #90 	R4-1904742

# Xi’An, China, April 8th – 12th 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.104 | CR | - | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.104 – PUSCH requirements with CP-OFDM for FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-04-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduce the SNR requirement for PUSCH FR2 with CP-OFDM |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the SNR requirement for PUSCH FR2 with CP-OFDM tests based on simulation results Add PT-RS alternative |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | New requirement for PUSCH can’t be verified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.2.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.141-2 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<start of the change>

#### 11.2.2.1	Requirements for PUSCH with transform precoding disabled

##### 11.2.2.1.1	General

The performance requirement of PUSCH is determined by a minimum required throughput for a given SNR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in annex A. The performance requirements assume HARQ retransmissions.

Table 11.2.2.1.1-1: Test parameters for testing PUSCH

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Uplink-downlink allocation for TDD |  | 60 kHz and 120kHz SCS: 3D1S1U, S=10D:2G:2U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DMRS configuration type | 1 |
|  | Maximum number of OFDM symbols for front loaded DMRS | 1 |
|  | Number of additional DM-RS symbols | 0, 1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | EPRE ratio of PUSCH to DM-RS | -3 dB |
|  | DM-RS port | {0}, {0, 1} |
|  | DM-RS sequence generation | NID=0, nSCID =0 |
| Time domain resource | PUSCH mapping type | B |
|  | PUSCH starting symbol index | 0 |
|  | PUSCH symbol length | 10 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
|  | Frequency hopping | Disabled |
| TPMI index for 2Tx two-layer spatial multiplexing transmission |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |
| PT-RS configuration | Frequency density (KPT-RS) | 2 |
|  | Time density (LPT-RS) | 1 |

##### 11.2.2.1.2	Minimum requirements

The throughput shall be equal to or larger than the fraction of maximum throughput stated in the tables 11.2.2.1.2-1 to 11.2.2.1.2-5 at the given SNR for 1Tx and for 2Tx two-layer spatial multiplexing transmission.

Table 11.2.2.1.2-1: Minimum requirements for PUSCH, 50 MHz channel bandwidth, 60 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | DMRS configuration | PT-RS | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-1 | 1+0 |  |  |
|  |  |  |  |  |  |  | No | [-1.7] |
|  |  |  |  |  |  | 1+1 |  |  |
|  |  |  |  |  |  |  | No | [-1.7] |
|  |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-1 | 1+0 | Yes | [12.0] |
|  |  |  |  |  |  |  | No | [11.8] |
|  |  |  |  |  |  | 1+1 | Yes | [11.4] |
|  |  |  |  |  |  |  | No | [11.2] |
|  |  | Normal | TDLA30-75 Low | 70 % | G-FR2-A5-1 | 1+0 | Yes | [13.6] |
|  |  |  |  |  |  |  | No | [13.3] |
|  |  |  |  |  |  | 1+1 | Yes | [TBD] |
|  |  |  |  |  |  |  | No | [TBD] |
| 2 |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-6 | 1+0 |  |  |
|  |  |  |  |  |  |  | No | [1.9] |
|  |  |  |  |  |  | 1+1 |  |  |
|  |  |  |  |  |  |  | No | [TBD] |
|  |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-6 | 1+0 | Yes | [NA] |
|  |  |  |  |  |  |  | No | [TBD] |
|  |  |  |  |  |  | 1+1 | Yes | [19.8] |
|  |  |  |  |  |  |  | No | [19.1] |

Table 11.2.2.1.2-2: Minimum requirements for PUSCH, 100 MHz channel bandwidth, 60 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | DMRS configuration | PT-RS | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-2 | 1+0 |  |  |
|  |  |  |  |  |  |  | No | [-1.9] |
|  |  |  |  |  |  | 1+1 |  |  |
|  |  |  |  |  |  |  | No | [-1.8] |
|  |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-2 | 1+0 | Yes | [12.2] |
|  |  |  |  |  |  |  | No | [11.5] |
|  |  |  |  |  |  | 1+1 | Yes | [TBD] |
|  |  |  |  |  |  |  | No | [10.8] |
|  |  | Normal | TDLA30-75 Low | 70 % | G-FR2-A5-2 | 1+0 | Yes | [14.2] |
|  |  |  |  |  |  |  | No | [13.6] |
|  |  |  |  |  |  | 1+1 | Yes | [TBD] |
|  |  |  |  |  |  |  | No | [TBD] |
| 2 |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-7 | 1+0 |  |  |
|  |  |  |  |  |  |  | No | [TBD] |
|  |  |  |  |  |  | 1+1 |  |  |
|  |  |  |  |  |  |  | No | [18.7] |

Table 11.2.2.1.2-3: Minimum requirements for PUSCH, 50 MHz channel bandwidth, 120 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | DMRS configuration | PT-RS | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-3 | 1+0 |  |  |
|  |  |  |  |  |  |  | No | [-1.6] |
|  |  |  |  |  |  | 1+1 |  |  |
|  |  |  |  |  |  |  | No | [-1.8] |
|  |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-3 | 1+0 | Yes | [11.5] |
|  |  |  |  |  |  |  | No | [11.4] |
|  |  |  |  |  |  | 1+1 | Yes | [TBD] |
|  |  |  |  |  |  |  | No | [TBD] |
|  |  | Normal | TDLA30-75 Low | 70 % | G-FR2-A5-3 | 1+0 | Yes | [13.6] |
|  |  |  |  |  |  |  | No | [13.3] |
|  |  |  |  |  |  | 1+1 | Yes | [TBD] |
|  |  |  |  |  |  |  | No | [TBD] |
| 2 |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-8 | 1+0 |  |  |
|  |  |  |  |  |  |  | No | [1.5] |
|  |  |  |  |  |  | 1+1 | Yes | [TBD] |
|  |  |  |  |  |  |  | No | [1.6] |
|  |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-8 | 1+0 |  |  |
|  |  |  |  |  |  |  | No | [18.8] |
|  |  |  |  |  |  | 1+1 | Yes | [TBD] |
|  |  |  |  |  |  |  | No | [TBD] |

Table 11.2.2.1.2-4: Minimum requirements for PUSCH, 100 MHz channel bandwidth, 120 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | DMRS configuration | PT-RS | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-4 | 1+0 |  |  |
|  |  |  |  |  |  |  | No | [-2.1] |
|  |  |  |  |  |  | 1+1 |  |  |
|  |  |  |  |  |  |  | No | [-2.2] |
|  |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-4 | 1+0 | Yes | [11.9] |
|  |  |  |  |  |  |  | No | [11.3] |
|  |  |  |  |  |  | 1+1 | Yes | [TBD] |
|  |  |  |  |  |  |  | No | [10.7] |
|  |  | Normal | TDLA30-75 Low | 70 % | G-FR2-A5-4 | 1+0 | Yes | [14.8] |
|  |  |  |  |  |  |  | No | [13.1] |
|  |  |  |  |  |  | 1+1 | Yes | [TBD] |
|  |  |  |  |  |  |  | No | [TBD] |
| 2 |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-9 | 1+0 |  |  |
|  |  |  |  |  |  |  | No | [19.4] |
|  |  |  |  |  |  | 1+1 |  |  |
|  |  |  |  |  |  |  | No | [18.2] |

Table 11.2.2.1.2-5: Minimum requirements for PUSCH, 200 MHz channel bandwidth, 120 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | DMRS configuration | PT-RS | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-5 | 1+0 |  |  |
|  |  |  |  |  |  |  | No | [-1.9] |
|  |  |  |  |  |  | 1+1 |  |  |
|  |  |  |  |  |  |  | No | [-1.9] |
|  |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-5 | 1+0 | Yes | [11.9] |
|  |  |  |  |  |  |  | No | [11.7] |
|  |  |  |  |  |  | 1+1 | Yes | [TBD] |
|  |  |  |  |  |  |  | No | [TBD] |
|  |  | Normal | TDLA30-75 Low | 70 % | G-FR2-A5-5 | 1+0 | Yes | [14.1] |
|  |  |  |  |  |  |  | No | [13.6] |
|  |  |  |  |  |  | 1+1 | Yes | [TBD] |
|  |  |  |  |  |  |  | No | [TBD] |
| 2 |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-10 | 1+0 |  |  |
|  |  |  |  |  |  |  | No | [1.5] |
|  |  |  |  |  |  | 1+1 |  |  |
|  |  |  |  |  |  |  | No | [1.4] |
|  |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-10 | 1+0 | Yes | [TBD] |
|  |  |  |  |  |  |  | No | [TBD] |
|  |  |  |  |  |  | 1+1 | Yes | [19.1] |
|  |  |  |  |  |  |  | No | [18.3] |

<end of the change>