3GPP TSG-RAN WG4 Meeting #90bis	R4-1902989

# Xian, China, 8th– 12th April, 2019

Agenda item:	7.15.3

Source:	Intel Corporation

Title:	Performance evaluation for cell identification time in LTE high speed scenario

Document for:	Discussion

# Introduction

In this contribution, the link level simulation results are provided for cell identification in LTE high speed scenario. The purpose of the simulation is to validate whether the existing cell identification delay requirement is applicable in high speed scenario.

# Simulation assumption

The simulation assumptions for cell identification are given in Table 1, Table 2, Table 3 and Table 4. As an initial study, the simulation assumption is the same high speed scenario study for R14 LTE [1].

Table 1: Link Simulation Parameters for cell identification

| Parameter | Unit | Cell 2 | Cell 3 | Cell 1 |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel number | - | Channel 1 | Channel 1 | Channel 1 |
| Data and Control PSD relative to RS PSD | dB | 0 | 0 | 0 |
| P-SCH and S-SCH PSD relative to RS PSD | dB | 0 | 0 | 0 |
| Number of RB’s |  | 6 | 6 | 6 |
| RB Utilization | % | 100 | 100 | 100 |
| Data Modulation | - | QPSK | QPSK | QPSK |
| Frame Structure Type | - | 1 | 1 | 1 |
| CP Length | - | Normal | Normal | Normal |
| Frequency Offset relative to UE frequency reference | Hz | 0 | 0 | 0 |
| 1) Relative Delay of 1st Path (synchronous) | μs | 0 | 0 | CP/2 |
| 2) Relative Delay of 1st Path (asynchronous): Fixed delay | ms | 0 | 1.5 | 3.0 |
| Ior/Noc | dB | 5.18 | 0.29 | Test 1: 1.25 Test 2: 6.25 |
| Number of Tx antennas | - | 1 | 1 | 1 |
| P-SCH Sequence ID | - | See Table 3, Table 4 | See Table 3, Table 4 | See Table 3, Table 4 |
| S-SCH Sequence ID [5] | - | See Table 3, Table 4 | See Table 3, Table 4 | See Table 3, Table 4 |
| Propagation Condition | - | AWGN with 0Hz offset between desired and interfering cells (for alignment), AWGN with 1630Hz offset between desired and interfering cells; EVA1630 |  |  |
| Ioc Model | - | AWGN |  |  |

Table 2: Other simulation assumption parameters for cell identification

| Simulation parameters | Comments/values |
| --- | --- |
| Prior knowledge of Cell 1 and Cell 2 by the UE | Yes |
| Cell 1, 2, 3 carrier frequency | Same |
| False detect threshold | Required as in a real UE implementation |
| UE having apriori knowledge of system being synchronous or synchronous (by signalling) | No |
| Duty cycle | 100% (to represent non-DRX case) |
| Performance criterion for comparison | 90th percentile acquisition time for “correct” cell detection of both P-SCH and S-SCH sequence id’s. |
| CP length detection | Both short or long CP may be present, hence UE needs to detect CP length |
| Receive antennas | 2 (uncorrelated) |

Table 3: Cell ID Combinations to be simulated

| case # | Cell 1 (Desired Cell) |  | Cell 2 (Interferer 1) |  | Cell 3 (Interferer 2) |  | Scenario |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | psc3 | ssc3a, ssc3b | psc1 | ssc1a, ssc1b | psc2 | ssc2a, ssc2b | Synchronous |
| 2 | psc3 | ssc3a, ssc3b | psc1 | ssc1a, ssc1b | psc2 | ssc2a, ssc2b | Asynchronous |

Table 4: PSC, SSC indices for simulations

| Label | Code index |
| --- | --- |
| psc1 | 29 |
| psc2 | 25 |
| psc3 | 34 |

| Label | Code index | Cell group index |
| --- | --- | --- |
| (ssc1a, ssc1b) | (6, 8) | 36 |
| (ssc2a, ssc2b) | (10, 12) | 40 |
| (ssc3a, ssc3b) | (7, 9) | 37 |

# Simulation results

Observation 1: For cell identification, PSS/SSS detection delay shall be at least 10 DRX cycles when SINR ≥ -6dB.

# Reference

[1] R4-162424, “Simulation assumptions on RRM requirements in idle mode under high speed scenarios,” Huawei,

HiSilicon, RAN4 Meeting #78bis.