3GPP TSG-RAN WG4 Meeting #90 bis	R4-1904909

Xian, China, 8 April – 12 April 2019

Source:	Ericsson, Swisscom

Title:	TP for TR 37.716-21-11: 3A-20A_n1A

Agenda item:			9.4.2

Document for:	Approval

# 1. Introduction

Rel-16 LTE-NR DC combinations LTE 2 band + NR 1 band combination are defined in the WID [1]. This contribution is a text proposal for TR 37.716-21-11 to add 3A-20A_n1A.

# 2. Text Proposal

# ---Start of changes---

## 5.1.x	3A-20A_n1A

### 5.1.x.1	Operating bands for EN-DC

Table 5.1B.5.3-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 3-20_n1 | CA_3-20 | n1 | DC_3_n1 |

### 5.1.x.2	Configuration for DC

Table 5.5B.5.3-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-20A_n1A | DC_3A_n1A DC_20A_n1A | CA_3A-20A | n1 |

### 5.1.x.3		∆TIB and ∆RIB values

For DC_3-20_n1, the TIB,c and RIB,c values are reused from the LTE combination CA_1-3-20, and are given in the tables below.

Table 5.1.x.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-20_n1 | 3 | 0.3 |
|  | 20 | 0.3 |
|  | n1 | 0.3 |

Table 5.1.x.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-20_n1 | 3 | 0 |
|  | 20 | 0 |
|  | n1 | 0 |

### 5.1.x.4	REFSENS requirements

Co-existence analysis from DC_3_n1 shows that there is no impact from DC_3_n1 UL to Band 20 DL.

Co-existence analysis from DC_20_n1 shows that there is no impact from DC_20_n1 UL to Band 3 DL.

Therefore, MSD does not need to be defined.

---End of changes---

# Reference

[1]	RP-190379, “Revised WID on Dual Connectivity (EN-DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL)”, Huawei