3GPP TSG-RAN WG4 Meeting #90Bis	R4-1905204

# Xi’an, China, April 8 – 12, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.817-01 | CR | 0012 | rev | 1 | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TR 38.817-01 on GSCN raster ranges |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-04-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There are 4 changes in the spreadsheet: - The calculation for the highest sync raster location in a band in incorrect by ½ subcarrier. The calculation of the upper end of the range may be incorrect. If the base station uses the incorrect value, the transmission of the SSB may cross the band edge. - The spreadsheet does not support non-unit step sizes for many of the bands in FR1. If future bands use non-unit step sizes, the spreadsheet calculation may be incorrect. - The equation used for bands with SCS based raster (M=3) is different. - The step size for n65 uses the wrong value. It should be 1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The spread sheet (attached) is included in the TR. - The calculation is updated from (20*12/2-1) to (20*12-1)/2 in column M - For step sizes: the lower GSCN value is updated to be ceiling(arg, step) while the upper GSCN valued is to updated to be floor(arg, step) in columns Q and T. respectively. - The equation for SCS-based raster below 3000 MHz is updated to align with the other bands. Note that this does not change any numbers in cells Q32, Q34 and T32, T33. - The step size for band n65 changed to 1 in N36. - An explanatory note is added to clarify how new band entries are introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The last entries of the GSCN range for a band may be incorrect when the incorrect calculated value is used |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.5, Spreadsheet replacement |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*********************Next changed section*****************

#### 4.3.1.5	Calculations of sync raster GSCN per operating band

For each operating band, the GSCN that can be used in the band are tabulated in subclause 5.4.3.3 of TS 38.104 [7], TS 38.101-1 [4] and TS 38.101-2 [5]. Selection of GSCN for Table(s) is done the following way:

-	Include GSCN that correspond to SS block that completely fit within the channel bandwidth, accounting for guard bands needed.

-	Guard bands are calculated based on the minimum channel BW in table 5.3.5-1 and table 5.3.5-2 of TS 38.104 [7], the SCS for the SS block and the corresponding NRB (spectrum utilization), assuming that the SS block can be in any position within the transmission BW configuration, including at positions adjacent to the edges.

-	For GSCN ranges with step size <N>, the GSCN numbers selected should be multiples of N (this ensures that overlapping bands will have the same GSCN sequences)

For specific combinations of minimum channel bandwidth and SS block SCS, the GSCN ranges are down-selected using a step size <N>, as shown in Table 4.3.1.5-1.

Table 4.3.1.5-1: Down selection factors (step size).

| Frequency range | Minimum channel bandwidth | SS block SCS | Down selection factor (step size) |
| --- | --- | --- | --- |
| 0 – 3.0 GHz | 10 MHz | 15 kHz | <3> |
| 3.0 - 24.25 GHz | 40 MHz | 30 kHz | <16> |
| 24.25 – 100 GHz | 100 MHz | 240 kHz | <2> |

To determine the GSCN range, let fmin denote the lowest frequency location of the SS block within a band after accounting for the guard band G in subclause 5.3.3 of TS 38.104 [7] corresponding to the minimum channel BW and the bandwidth encompassing the width of all subcarriers from RE#0 of RB#0 to the centre of RE#0 of RB#10 of the SS block. Let fmax denote the high frequency location of the SS block within a band after accounting for the guard band and the bandwidth from the centre of RE#0 of RB#10 and encompassing the width of all subcarriers until RE#11 of RB#19 of the SS block. Let Flow represent the lowest frequency of the frequency range in subclause 4.3.1.4, Fraster represent the raster spacing in MHz, Fshift represent the raster shift in MHz (applicable for the frequency range 0 to 3000MHz).

The lowest frequency location fmin is computed as

fmin=Flow+G+∆fNRBSSNRERB+12

where ∆f is the subcarrier spacing, NRERB=12 is the number of REs per RB, NRBSS=20 is the number of RBs in the SS block.

The highest frequency location fmax is computed as

fmax=Flow+WBand-G-∆fNRBSSNRERB-12

where WBand is the width of the operating band.

The first possible raster location within a band (prior to applying the step size) is given by

fmin≤Flow+N∆Fraster+M∆Fshift

while the last possible raster location within a band (prior to applying the step size) is given by

fmax≥Flow+N∆Fraster+M∆Fshift

where Fshift is defined to be 0 outside the frequency range 0 to 3000MHz and where N (and M) are defined in subclause 4.3.1.4.

For the first possible raster location, Nmin is the smallest integer satisfying

Nmin≥1∆Frasterfmin-Flow-max⁡M∆Fshift

and the corresponding value of M (if defined and if multiple values are defined)

M≥1∆Fshiftfmin-Flow-Nmin∆Fraster

For the last possible raster location, Nmax is the largest integer satisfying

Nmax≤fmax-Flow-minM∆Fshift∆Fraster

and the corresponding value of M (if defined and if multiple values are defined)

M≤fmax-Flow-Nmax∆Fraster∆Fshift

For the frequency ranges in subclause 4.3.1.4, table 4.3.1.5-2 indicates the formulas used to compute the values of N and M (if necessary).

Table 4.3.1.5-2: Formulas to compute the minimum and maximum values of GSCN

| Type | Parameter | Range | Formula |
| --- | --- | --- | --- |
| For GSCNmin | N (compute first) | 0-3000 MHz | N=fmin-Flow-maxM∆Fshift∆Fraster,Mϵ1,3,53Note |
|  |  | ≥ 3000 MHz | N=fmin-Flow∆Fraster |
|  | M | 0-3000 MHz | M'=maxfmin-Flow-N∆Fraster∆Fshift,1 M=M'+M'+1mod2 |
|  |  | 0-3000 MHz, NOTE | 3 |
|  |  | ≥ 3000 MHz | N/A |
| For GSCNmax | N (compute first) | 0-3000 MHz | N=fmax-Flow-minM∆Fshift∆Fraster,Mϵ1,3,53Note |
|  |  | ≥ 3000 MHz | N=fmax-Flow∆Fraster |
|  | M | 0-3000 MHz | M=minfmax-Flow-N∆Fraster∆Fshift,5 M=M'+M'+1mod2 |
|  |  | 0-3000 MHz, NOTE | 3 |
|  |  | ≥ 3000 MHz | N/A |
| NOTE: Refers to Note in Table 5.4.3.1-1 in TS 38.101-1 [4] for bands with SCS-based raster below 3000 MHz. |  |  |  |

To compute the GSCN ranges with step size <N>, the N and M (when defined) values computed with the formulas in table 4.3.1.5-2 are used to determine GSCNmin and GSCNmax according to the appropriate raster entry formula from subclause 4.3.1.4. The first entry in the GSCN range, GSCNmin, is determined by

GSCNmin=NGSCNmin'N

and the last entry in the range, GSCNmax, is determined by

GSCNmax=NGSCNmax'N.

The method described above for calculating the GSCN range [GSCNmin, GSCNmax] for an operating band is implemented in the Excel spread sheet NR sync raster calculations (2019-06).xlsx, which is attached to the Technical Report. The spread sheet was used to generate the tables in subclause 5.4.3.3 of TS 38.104 [7], TS 38.101-1 [4] and TS 38.101-2 [5].