3GPP TSG RAN WG4 Meeting #91	R4-1905309

Reno, USA, May 13th – 17th, 2019

3GPP TSG-RAN WG1 Meeting #96bis			R1-1905895

Xi’an, China, 8th April – 12th April, 2019

Title:	LS on UL wideband operation for NR-U

Response to:

Release:	Rel-16

Work Item:	NR_unlic-Core

Source:	RAN WG1

To:	RAN WG4

Cc:

Contact Person:

#### Name:	Seonwook Kim

E-mail Address:	seonwook.kim@lge.com

Attachments:	none

1. Description:

RAN1 has discussed UL transmission on a carrier with bandwidth wider than LBT bandwidth (i.e., 20 MHz at least for 5 GHz) in unlicensed spectrum and made the following agreement:

Agreement:

For UL transmissions in a serving cell with carrier bandwidth greater than LBT bandwidth, for the case where UE performs CCA before UL transmission, support at least Alt. 1 among the following alternatives

- Alt. 1: UE transmits the PUSCH only if CCA is successful at UE in all LBT bandwidths of the scheduled PUSCH.

- Alt. 2: UE transmits the PUSCH in all or a subset of LBT bandwidths of the scheduled PUSCH for which CCA is successful at the UE.

  - Decision on whether this alternative is supported will depend on feedback from RAN4

  - FFS on restrictions to the subset of LBT bandwidths, e.g., only contiguous LBT bandwidths allowed, based on feedback from RAN4

- Necessity of guard bands within the scheduled PUSCH should be determined by RAN4

- FFS: Whether this applies also to configured grant PUSCH

- FFS: Whether this applies also to PUCCH

RAN1 identified two alternatives for wideband NR-U carrier operations in UL. For Alt. 2, RAN1 respectfully asks RAN4 to provide feedback on feasibility with or without potential transmission restrictions on a subset of LBT bandwidths of the scheduled PUSCH for which CCA is successful at the UE, and necessity of guard bands within the scheduled PUSCH.

2. Actions:

To RAN WG4

ACTION: 	RAN1 respectfully asks RAN4 to provide feedback on the issues above.

3. Date of Next TSG-RAN WG1 Meetings:

RAN1#97	13th – 17th May 2019		Reno, Nevada, USA

RAN1#98	26th – 30th August 2019		Prague, Czech Republic