3GPP TSG-RAN WG4 Meeting #91	R4-1905339

Reno, USA, 13th – 17th May 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR |  | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removal of brackets in A-MPR section |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-04-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Removal of brackets in A-MPR section as otherwise the requirements are untestable. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removal of brackets in A-MPR section. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A-MPR requirements are untestable. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS TS36.521 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*************************** Start of changes *******************************************

Table 6.2.3.2-2: A-MPR' valuesAccess

|  | Modulation | A-MPR’ (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | PC3_A1 | PC3_A2 | PC2_A3 | PC2_A4 |
| DFT-S-OFDM | pi/2-BPSK | 3.5 | 3.5 | 3.5 | 5.5 |
|  | QPSK | 4 | 4 | 4.5 | 6 |
|  | 16-QAM | 4 | 4 | 5 | 6 |
|  | 64-QAM |  | 4.5 | 5 | 6.5 |
|  | 256-QAM | 4.5 | 6 | 6.5 | 8 |
| CP-OFDM | QPSK | 5.5 | 5.5 | 6.5 | 7.5 |
|  | 16-QAM | 5.5 | 5.5 | 6.5 | 7.5 |
|  | 64-QAM | 5.5 | 5.5 | 6.5 | 7.5 |
|  | 256-QAM | 6.5 | 8 | 7.5 | 10 |
| NOTE: The A-MPR’ values in this table apply for both A-MPR relative to 23 dBm for PC3 and A-MPR relative to 26 dBm for PC2 |  |  |  |  |  |

#### 6.2.3.3	A-MPR for NS_10

Table 6.2.3.3-1: A-MPR for "NS_10"

| Channel bandwidth (MHz) | Parameters | Region A |
| --- | --- | --- |
| 15 | RBstart | 0 – 10 |
|  | LCRB (RBs) | 1 – 20 |
|  | A-MPR (dB) | ≤ 36 |
| 20 | RBstart | 0 – 15 |
|  | LCRB (RBs) | 1 – 20 |
|  | A-MPR (dB) | ≤ 66 |
| NOTE 1: RBstart indicates the lowest RB index of transmitted resource blocks NOTE 2: LCRB is the length of a contiguous resource block allocation NOTE 3: For intra-subframe frequency hopping which intersects Region A, notes 1 and 2 apply on a per slot basis. For intra-slot or intra-subslot frequency hopping which intersects Region A, notes 1 and 2 apply on a Tno_hopping basis. NOTE 4: For intra-subframe frequency hopping which intersect Region A, the larger A-MPR value may be applied for both slots in the subframe. For intra-slot frequency hopping which intersects Region A, the larger A-MPR value may be applied for the slot. For intra-subslot frequency hopping which intersects Region A, the larger A-MPR value may be applied for the subslot. NOTE 5: The total maximum output power reduction for NS_10 is obtained by taking the maximum value of MPR + A-MPR specified in Table 6.2.3-1 and Table 6.2.4-1 in TS 36.101 and A-MPR specified in Table 6.2.3.3-1. NOTE 6: The A-MPR for CP-OFDM shall also add the corresponding MPR specified in Table 6.2.2-1. |  |  |

*************************** Un changed section *******************************************

#### 6.2.3.8	A-MPR for NS_37

Table 6.2.3.8-1: A-MPR for B11/B21 protection (NS_37) for 10MHz, 15MHz (1447.9-1462.9MHz)

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | Region A (Outer/Inner) |  |  | Region B (Outer/Inner) |  |  | Region C (Outer/Inner) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart,(MHz/12/SCS) | LCRB (MHz/12/SCS) | A-MPR | RBstart (MHz/12/SCS) | LCRB (MHz/12/SCS) | A-MPR | RBstart (MHz/12/SCS) | LCRB (MHz/12/SCS) | A-MPR |
| 10 MHz | 1452.9 < FC ≤ 1457.9 | ≥ 0 | > 40 | ≤ A1 | N/A | N/A | N/A | N/A | N/A | N/A |
| 15 MHz | FC = 1455.4 | ≥ 0 | > 55 | ≤ A1 | < 0.54 | < 1.08 | ≤ A2 | > 13.86 | < 1.08 | ≤ A2 |
| NOTE 1: A-MPR values in Table 6.2.3.8-2. NOTE 2: The backoff applied is max (MPR, A-MPR) where MPR is defined in Table 6.2.2-1 NOTE 3: For any undefined region, MPR applies NOTE 4: No A-MPR for SCS = 60 kHz. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.8-2: A-MPR for modulation and waveform type

| Modulation/Waveform | A1 |  | A2 |
| --- | --- | --- | --- |
|  | Outer | Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 1 | N/A | ≤ 3 |
| DFT-s-OFDM QPSK | ≤ 1.5 | N/A | ≤ 3 |
| DFT-s-OFDM 16 QAM | ≤ 2.5 | N/A | ≤ 3 |
| DFT-s-OFDM 64 QAM | ≤ 3 | N/A | ≤ 3 |
| DFT-s-OFDM 256 QAM | N/A | N/A | N/A |
| CP-OFDM QPSK | ≤ 3.5 | N/A | ≤ 3 |
| CP-OFDM 16 QAM | ≤ 3.5 | N/A | ≤ 3 |
| CP-OFDM 64 QAM | N/A | N/A | N/A |
| CP-OFDM 256 QAM | N/A | N/A | N\A |
| NOTE 1: The backoff applied is max(MPR, A-MPR) where MPR is defined in Table 6.2.2-1 NOTE 2: Outer and inner allocations are defined in clause 6.2.2 |  |  |  |

#### 6.2.3.9	A-MPR for NS_38

Table 6.2.3.9-1: A-MPR for EESS (NS_38) Protection (1430-1470MHz)

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | Region A (Outer/Inner) |  |  | Region B (Outer/Inner) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart | LCRB | A-MPR | RBstart+LCRB(MHz/12/SCS) | A-MPR |
| 10 MHz | 1435 ≤ FC < 1442 | <= -1.8 MHz/12/SCS + LCRB/2 | > 3.6 | ≤ 12 | ≤ 2.16 | ≤ 9 |
| 15 MHz | 1437.5 ≤ FC < 1447.5 | <= -1.8 MHz/12/SCS + LCRB/2 | > 3.6 | ≤ 13 | ≤ 3.6 | ≤ 10 |
| 20 MHz | 1440 ≤ FC < 1450 | <= -1.8 MHz/12/SCS + LCRB/2 | > 3.6 | ≤ 13 | ≤ 5.4 | ≤ 10 |
| NOTE 1: The backoff applied is max(MPR, A-MPR) where MPR is defined in Table 6.2.2-1 NOTE 2: Outer and inner allocations are defined in clause 6.2 NOTE 3: For any undefined region, MPR applies NOTE 4: A-MPR applies to all modulation and waveform types. |  |  |  |  |  |  |

#### 6.2.3.10	A-MPR for NS_39

Table 6.2.3.10-1: A-MPR for own RX (NS_39) Protection for 10MHz,15MHz, 20MHz (1440-1470MHz)

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | Region A (Outer/Inner) |  |
| --- | --- | --- | --- |
|  |  | RBstart+LCRB(MHz/12/SCS) | A-MPR |
| 10MHz | 1462 < FC ≤ 1465 | > 7.9 | ≤ 6 |
| 15MHz | 1456.3 < FC ≤ 1462.5 | > 11.2 | ≤ 6 |
| 20MHz | 1450.8 < FC ≤ 1460 | > 14.4 | ≤ 6 |
| NOTE 1: The backoff applied is max(MPR, A-MPR) where MPR is defined in Table 6.2.2-1 NOTE 2: Outer and inner allocations are defined in clause 6.2 NOTE 3: For any undefined region, MPR applies NOTE 4: A-MPR applies to all modulation and waveform types. |  |  |  |

*************************** End of changes *******************************************