3GPP TSG-RAN WG4 Meeting #91              	R4-1905431

Reno, US, 13th– 17th May, 2019

Agenda item:	6.12.2.2.1

Source:	Samsung

Title:	Updated simulation results for NR UCI on PUSCH

Document for:	Discussion

# 1	Introduction

During RAN#90 meeting, the remained issues for performance requirement of UCI on PUSCH are agreed and captured in the WF [1].

In this contribution, the view of remained issue for UCI on PUSCH is provided. Also, the initial simulation results with alignment and impairment are provided.

# 2	Discussion

## 2.1	Payload Content

For UCI requirement, two test cases with small number and large number of information bits were specified, separately.  As for 7 bits UCI cases, there is no CRC attached.  For the purpose of BS conducted test, the payload content should be fixed as

- 7bits:

- [0 1 0 1 0] for CSI part1

- Other options are not precluded

- [1 0] for CSI part2

- Other options are not precluded

As specified, UCI is encoded with (32, O) RM code for the information bit O is less than 11bits.  The code block is encoded by

where  represents the basis sequence as defined in the spec 38.212. The payload content with different order will result in the differential of encoded sequence.

Based on mapping order of CSI field of CSI report for CSI part 1, the CSI field only includes with 1 bit RI information and 4 bit CQI information, under the assumption of 5bit UCI CSI part1 information. Hence, it should be clearly indicated what is mapping order in the fixed payload content.

Proposal 1:  The payload of 7 bit UCI information is fixed as [c0, c1… c4] = [0 1 0 1 0] for CSI part1 and [c0, c1] = [1 0] for CSI part2, the lowest order information c0 is mapped to RI information.

## 2.2	Test Applicability

As captured in WF [1], both type A and type B with slot-based time domain allocation was specified for requirement of UCI for FR1. The requirements for type A and type B are only applicable according to the BS declaration. For the test applicability with different allocation scheme in the requirement of UCI on PUSCH, it should be same with the test applicability for PUSCH.

Proposal 2:  The test applicability for UCI on PUSCH with different allocation type is same with the test applicability for PUSCH with different allocation type.

# 3	Simulation Results

In this section, based on the agreed simulation assumption for UCI on PUSCH as table 1, the ideal and impairment simulation results are presented as table 2 and table 3

Table 1: Simulation assumption for UCI on PUSCH for CP-OFDM

| Parameter | value |  |
| --- | --- | --- |
|  | FR1 | FR2 |
| Transform precoding | Disabled | Disabled |
| Number of Tx | 1 | 1 |
| Number of Rx | 2 | 2 |
| Number of layers | 1 | 1 |
| [Uplink-downlink allocation for TDD] | 30 kHz SCS: 7D1S2U, S=6D:4G:4U | 120kHz SCS: 3D1S1U, S=10D:2G:2U |
| Code block group based PUSCH transmission | Disabled | Disabled |
| MCS index | 16 | 16 |
| Propagation Condition | MCS 16: TDL-C 300ns, 100 Hz | MCS 16: TDL-A 30ns, 300Hz |
| The number of HARQ transmission | 1 | 1 |
| Transmission scheme | Identity matrix (TPMI index 0) | Identity matrix (TPMI index 0) |
| DMRS type | type 1 | type 1 |
| Number of DMRS symbols | 1+1 (2,11) for type A 1+1 (0,10) for type B | 1 (0) type B |
| symbols length | 14 | 10 |
| Start symbol index | 0 | 0 |
| Time domain resource allocation type | type A, type B | type B |
| Frequency domain resource | Full RB allocation of the applicable BW | Full RB allocation of the applicable BW |
| Carrier frequency (GHz) | 4 | 30 |
| Propagation channel | MCS 16: TDL-C 300ns, 100 Hz | MCS 16: TDL-A 30 ns, 300Hz |
| SCS and BW | 30 KHz: 10MHz 120Khz: 50MHz |  |
| Timing offset | 0 | 0 |
| Frequency offset | 0 | 0 |
| PTRS | Not configured | Configured with KPTRS =2, LPTRS =1 |
| Code block group | Disabled | Disabled |
| Frequency hopping | Disabled | Disabled |
| Limited buffer rate matching | Disabled | Disabled |
| UCI | With PUSCH data | With PUSCH data |
| Payload | Case1: 7( csi part1=5, csi_part=2) Case2:40 (csi_part1=20, csi_part2=20 ) | Case1: 7 (csi part1=5, csi_part=2) Case2 : 40( csi_part1=20, csi_part2=20) |
| Payload content | Case 1:[0 1 0 1 0] for CSI part1 with 5bit;[1 0] for CSI part2 with 2bit Case 2: Random payload content selection for (20,20) configuration | Case 1:[0 1 0 1 0] for CSI part1 with 5bit;[1 0] for CSI part2 with 2bit Case 2: Random payload content selection for (20,20) configuration |
| alpha Uci_on_pusch_scallng | 1 | 1 |
| BetaOffset-HARQ_ACK_index | 11 | 11 |
| BetaOffsetCSI-part1_Index and BetaOffsetCSI-part2_Index | 13 | 13 |
| Test Metric | 0.1% BLER of UCI with CSI part1, 1% BLER of UCI with CSI part2 | 0.1% BLER of UCI with CSI part1, 1% BLER of UCI with CSI part2 |

Table2: The alignment performance for NR PUSCH with 1 Tx with CP-OFDM

| SCS,BW | DMRS configuration | channel | UCI bit | Target SNR with 0.1% BLER of CSI part1 | Target SNR with 1% BLER of CSI part2 |
| --- | --- | --- | --- | --- | --- |
| 30kHz: 10 | 1+1 (2,11) | TDL-C 300ns, 100Hz | 7 bit (csi1=5, csi2 =2) | 4.51 | -1.83 |
|  | 1+1 (2,11) |  | 40 bit (csi1=20, csi2 =20) | 2.73 | 0.45 |
|  | 1+1 (0,10) |  | 7 bit (csi1=5, csi2 =2) | 4.77 | -1.89 |
|  | 1+1 (0,10) |  | 40 bit (csi1=20, csi2 =20) | 1.86 | 0.43 |

Table3: The impairment performance for NR PUSCH with 1 Tx with CP-OFDM

| SCS,BW | DMRS Configuration | channel | UCI bit | Target SNR with 0.1% BLER of CSI part1 | Target SNR with 1% BLER of CSI part2 |
| --- | --- | --- | --- | --- | --- |
| 30kHz: 10 | 1+1 (2,11) | TDL-C 300ns, 100Hz | 7 bit (csi1=5, csi2 =2) | 6.51 | 0.17 |
|  | 1+1 (2,11) |  | 40 bit (csi1=20, csi2 =20) | 4.73 | 2.45 |
|  | 1+1 (0,10) |  | 7 bit (csi1=5, csi2 =2) | 6.77 | 0.11 |
|  | 1+1 (0,10) |  | 40 bit (csi1=20, csi2 =20) | 3.86 | 2.43 |

# 3	Conclusion

In this contribution, the review on the remained issue of UCI on PUSCH was provided. Also, the initial ideal and impairment simulation results are provided.

Proposal 1:  The payload of 7 bit UCI information is fixed as [c0, c1… c4] = [0 1 0 1 0] for CSI part1 and [c0, c1] = [1 0] for CSI part2, the lowest order information c0 in CSI part1 is mapped to the RI information.

Proposal 2:  The test applicability for UCI on PUSCH with different allocation type is same with the test applicability for PUSCH with different allocation type.

# References

R4-1904716 WF on performance requirements of UCI Multiplexing on PUSCH, Samsung.