3GPP TSG-RAN4 Meeting #91	R4-1905462

# Reno, USA, 13th – 17th, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | - | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | × | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add FRC for sub-band CQI test cases: 6.2.2, 6.2.3, A.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-03-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | FRC for sub-band CQI test cases still TBD. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add FRC for sub-band CQI test cases - Correction erros for existing FRC of CSI |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CSI requirements cannot be verified properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2, 6.2.3, A.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | × | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | × |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | × | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

< ---------------                                                         Start of change                                                                         --------------------->

## 6.2	Reporting of Channel Quality Indicator (CQI)

This section includes the requirements for the reporting of channel quality indicator (CQI).

### 6.2.1	1RX requirements

(Void)

### 6.2.2	2RX requirements

This sub-clause includes the requirements for reporting of CQI for UE equipped with 2 receiver antennas.

#### 6.2.2.1	FDD

##### 6.2.2.1.1	CQI reporting definition under AWGN conditions

The reporting accuracy of the channel quality indicator (CQI) under frequency non-selective conditions is determined by the reporting variance and the BLER performance using the transport format indicated by the reported CQI median. The purpose is to verify that the reported CQI values are in accordance with the CQI definition given in TS 38.214 [12]. To account for sensitivity of the input SNR the reporting definition is considered to be verified if the reporting accuracy is met for at least one of two SNR levels separated by an offset of [1] dB.

6.2.2.1.1.1	Minimum requirement for periodic CQI reporting

For the parameters specified in Table 6.2.2.1.1.1-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified by the following:

a)	The reported CQI value according to the reference channel shall be in the range of ±1 of the reported median more than [90]% of the time.

b)	If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, then the BLER using the transport format indicated by the (median CQI+1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, then the BLER using transport format indicated by (median CQI-1) shall be less than or equal to 0.1.

Table 6.2.2.1.1.1-1: CQI reporting definition test

| Parameter |  |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  |  | MHz | 10 |  |  |  |
| Duplex Mode |  |  |  |  | FDD |  |  |  |
| Subcarrier spacing |  |  |  | kHz | 15 |  |  |  |
| SNR |  |  |  | dB | [8] | [9] | [14] | [15] |
| Propagation channel |  |  |  |  | AWGN |  |  |  |
| Antenna configuration |  |  |  |  | 2×2 with static channel specified in Annex B.1 |  |  |  |
| Beamforming Model |  |  |  |  | As specified in Section Annex B.4.1 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  |  | 4 |  |  |  |
|  | CDM Type |  |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  |  | Row 5,4 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | 9 |  |  |  |
|  | CSI-RS periodicity and offset |  |  | slot | 5/1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  |  | 2 |  |  |  |
|  | CDM Type |  |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | Row 3,(6,-) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  |  | slot | 5/1 |  |  |  |
| CSI-IM configuration | CSI-IM RE pattern |  |  |  | 0 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  |  | (4, 9) |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  |  | slot | 5/1 |  |  |  |
| ReportConfigType |  |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  |  | Table 2 |  |  |  |
| reportQuantity |  |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  |  | RB | N/A |  |  |  |
| CSI-Report periodicity and offset |  |  |  | slot | 5/1 |  |  |  |
| aperiodicTriggeringOffset |  |  |  |  | Not configured |  |  |  |
| Codebook configuration |  | Codebook Type |  |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  |  | Not configured |  |  |  |
|  |  | CodebookSubsetRestriction |  |  | 010000 |  |  |  |
|  |  | RI Restriction |  |  | N/A |  |  |  |
| Physical channel for CSI report |  |  |  |  | PUCCH |  |  |  |
| CQI/RI/PMI delay |  |  |  | ms | 8 |  |  |  |
| Maximum number of HARQ transmission |  |  |  |  | 1 |  |  |  |
| Measurement channel |  |  |  |  | As specified in Table A.4-1, TBS.2-2 |  |  |  |

##### 6.2.2.1.2	CQI reporting under fading conditions

6.2.2.1.2.1	Minimum requirement for wideband CQI reporting

The purpose of the requirements is to verify that the UE is tracking the channel variations and selecting the largest transport format possible according to the prevailing channel state for the frequency non-selective scheduling.

The reporting accuracy of CQI under frequency non-selective fading conditions is determined by the reporting variance, the relative increase of the throughput obtained when the transport format is indicated by the reported CQI compared to the throughput obtained when a fixed transport format is configured according to the reported median CQI, and a minimum BLER using the transport formats indicated by the reported CQI. To account for sensitivity of the input SNR the sub-band CQI reporting under frequency selective fading conditions is considered to be verified if the reporting accuracy is met for at least one of two SNR levels separated by an offset of [1] dB.

For the parameters specified in Table 6.2.2.1.2.1-1 and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified by the following:

a)	A CQI index not in the set {median CQI -1, median CQI, median CQI +1} shall be reported at least α% of the time where α% is specified in Table 6.2.2.1.2.1-2;

b)	The ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index and that obtained when transmitting a fixed transport format configured according to the wideband CQI median shall be ≥ γ, where γ is specified in Table 6.2.2.1.2.1-2;

c)	When transmitting the transport format indicated by each reported wideband CQI index, the average BLER for the indicated transport formats shall be greater than or equal to [0.02].

Table 6.2.2.1.2.1-1: Wideband CQI reporting test under frequency non-selective fading conditions

| Parameter |  |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  |  | MHz | 10 |  |  |  |
| Subcarrier spacing |  |  |  | kHz | 15 |  |  |  |
| Duplex Mode |  |  |  |  | FDD |  |  |  |
| SNR |  |  |  | dB | [6] | [7] | [12] | [13] |
| Propagation channel |  |  |  |  | TDLA30-5 |  |  |  |
| Antenna configuration |  |  |  |  | 2×2 |  |  |  |
| Correlation configuration |  |  |  |  | ULA high |  |  |  |
| Beamforming Model |  |  |  |  | As specified in Section Annex B.4.1 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  |  | 4 |  |  |  |
|  | CDM Type |  |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  |  | Row 5,4 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | 9 |  |  |  |
|  | CSI-RS periodicity and offset |  |  | slot | 5/1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  |  | 2 |  |  |  |
|  | CDM Type |  |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | Row 3,(6,-) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  |  | slot | 5/1 |  |  |  |
| CSI-IM configuration | CSI-IM RE pattern |  |  |  | 0 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  |  | (4, 9) |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  |  | slot | 5/1 |  |  |  |
| ReportConfigType |  |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  |  | Table 2 |  |  |  |
| reportQuantity |  |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  |  | RB | 8 |  |  |  |
| Csi-ReportingBand |  |  |  |  | 1111111 |  |  |  |
| CSI-Report periodicity and offset |  |  |  | slot | 5/1 |  |  |  |
| aperiodicTriggeringOffset |  |  |  |  | Not configured |  |  |  |
| Codebook configuration |  | Codebook Type |  |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  |  | Not configured |  |  |  |
|  |  | CodebookSubsetRestriction |  |  | 000001 |  |  |  |
|  |  | RI Restriction |  |  | N/A |  |  |  |
| Physical channel for CSI report |  |  |  |  | PUCCH |  |  |  |
| CQI/RI/PMI delay |  |  |  | ms | 8 |  |  |  |
| Maximum number of HARQ transmission |  |  |  |  | 1 |  |  |  |
| Measurement channel |  |  |  |  | As specified in Table A.4-1, TBS.2-1 |  |  |  |

Table 6.2.2.1.2.1-2: Minimum requirements

| Parameters | Test 1 | Test 2 |
| --- | --- | --- |
|  [%] | [20] | [20] |
|  | [1.05] | [1.05] |

6.2.2.1.2.2	Minimum requirement for sub-band CQI reporting

The purpose of the requirements is to verify that the preferred sub-bands can be used for frequency-selective scheduling under the frequency-selective fading conditions.

The accuracy of sub-band channel CQI reporting under the frequency-selective fading conditions is determined by a double-sided percentile of  the reported differential CQI offset level 0 per sub-band, and the relative increase of the throughput obtained when transmitting the transport format indicated by the corresponding reported sub-band CQI on a randomly selected sub-band among the sub-bands with the highest reported differential CQI offset level compared to the throughput when transmitting a fixed transport format according to the wideband CQI median on a randomly selected sub-band among all the sub-bands.

For the parameters specified in Table 6.2.2.1.2.2-1 and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified by the following:

a)	A sub-band differential CQI offset level of 0 shall be reported at least α% of the time but less than β% of the time for each sub-band, where α and β are specified in Table 6.2.2.1.2.2-2;

b)	The ratio of the throughput obtained when transmitting the corresponding transport format on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level and that obtained when transmitting the transport format indicated by the reported wideband CQI median on a randomly selected sub-band among all the sub-bands shall be ≥ γ, where γ is specified in Table 6.2.2.1.2.2-2;

c)	When transmitting the corresponding transport format on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level, the average BLER for the indicated transport format shall be greater than or equal to TBD.

The requirements only apply for sub-bands of full size and the random scheduling across the sub-bands is done by selecting a new sub-band in each TTI for FDD.

Table 6.2.2.1.2.2-1: Sub-band CQI reporting test under frequency-selective fading conditions

| Parameter |  |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  |  | MHz | 10 |  |  |  |
| Subcarrier spacing |  |  |  | kHz | 15 |  |  |  |
| Duplex Mode |  |  |  |  | FDD |  |  |  |
| SNR |  |  |  | dB | [8] | [9] | [14] | [15] |
| Propagation channel |  |  |  |  | Two tap model specified in Annex B.2.4 with a=1, fD = 5Hz, and τd=0.45μs |  |  |  |
| Antenna configuration |  |  |  |  | 2×2 |  |  |  |
| Correlation configuration |  |  |  |  | As per Annex B.1 |  |  |  |
| Beamforming Model |  |  |  |  | As specified in Section Annex B.4.1 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  |  | 4 |  |  |  |
|  | CDM Type |  |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  |  | Row 5,4 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | 9 |  |  |  |
|  | CSI-RS periodicity and offset |  |  | slot | 5/1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  |  | 2 |  |  |  |
|  | CDM Type |  |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | Row 3,(6,-) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  |  | slot | 5/1 |  |  |  |
| CSI-IM configuration | CSI-IM RE pattern |  |  |  | 0 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  |  | (4, 9) |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  |  | slot | 5/1 |  |  |  |
| ReportConfigType |  |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  |  | Table 2 |  |  |  |
| reportQuantity |  |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  |  | Subband |  |  |  |
| pmi-FormatIndicator |  |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  |  | RB | 8 |  |  |  |
| CSI-Report periodicity and offset |  |  |  | slot | 5/1 |  |  |  |
| aperiodicTriggeringOffset |  |  |  |  | Not configured |  |  |  |
| Codebook configuration |  | Codebook Type |  |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  |  | Not configured |  |  |  |
|  |  | CodebookSubsetRestriction |  |  | 000001 |  |  |  |
|  |  | RI Restriction |  |  | N/A |  |  |  |
| Physical channel for CSI report |  |  |  |  | TBD |  |  |  |
| CQI/RI/PMI delay |  |  |  | ms | 8 |  |  |  |
| Maximum number of HARQ transmission |  |  |  |  | 1 |  |  |  |
| Measurement channel |  |  |  |  | As specified in Table A.4-2, TBS.2-5 |  |  |  |

Table 6.2.2.1.2.2-2: Minimum requirements

| Parameters | Test 1 | Test 2 |
| --- | --- | --- |
| α [%] | [2] | [2] |
| β [%] | [55] | [55] |
|  | [1.05] | [1.05] |

#### 6.2.2.2	TDD

##### 6.2.2.2.1	CQI reporting definition under AWGN conditions

6.2.2.2.1.1	Minimum requirement for periodic CQI reporting

The purpose of the requirements is to verify that the reported CQI values are in accordance with the CQI definition given in TS38.214 [12]. The reporting accuracy of CQI under AWGN condition is determined by the reporting variance and BLER performance using the transport format indicated by the reported CQI median.

For the parameters specified in Table 6.2.2.2.1.1-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified by the following:

a)	The reported CQI value according to the reference channel shall be in the range of ±1 of the reported median more than [90]% of the time.

b)	If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, then the BLER using the transport format indicated by the (median CQI+1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, then the BLER using transport format indicated by (median CQI-1) shall be less than or equal to 0.1.

Table 6.2.2.2.1.1-1: CQI reporting definition test

| Parameter |  |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  |  | MHz | 40 |  |  |  |
| Subcarrier spacing |  |  |  | kHz | 30 |  |  |  |
| Duplex Mode |  |  |  |  | TDD |  |  |  |
| TDD UL-DL pattern |  |  |  |  | FR1.30-1 |  |  |  |
| SNR |  |  |  | dB | [8] | [9] | [14] | [15] |
| Propagation channel |  |  |  |  | AWGN |  |  |  |
| Antenna configuration |  |  |  |  | 2×2 with static channel specified in Annex B.1 |  |  |  |
| Beamforming Model |  |  |  |  | As specified in Section Annex B.4.1 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  |  | 4 |  |  |  |
|  | CDM Type |  |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  |  | Row 5,4 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | 9 |  |  |  |
|  | CSI-RS periodicity and offset |  |  | slot | 10/1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  |  | 2 |  |  |  |
|  | CDM Type |  |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | Row 3,(6,-) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  |  | slot | 10/1 |  |  |  |
| CSI-IM configuration | CSI-IM RE pattern |  |  |  | 0 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  |  | (4, 9) |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  |  | slot | 10/1 |  |  |  |
| ReportConfigType |  |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  |  | Table 2 |  |  |  |
| reportQuantity |  |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  |  | RB | N/A |  |  |  |
| CSI-Report periodicity and offset |  |  |  | slot | 10/1 |  |  |  |
| aperiodicTriggeringOffset |  |  |  |  | Not configured |  |  |  |
| Codebook configuration |  | Codebook Type |  |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  |  | Not configured |  |  |  |
|  |  | CodebookSubsetRestriction |  |  | 010000 |  |  |  |
|  |  | RI Restriction |  |  | N/A |  |  |  |
| Physical channel for CSI report |  |  |  |  | PUCCH |  |  |  |
| CQI/RI/PMI delay |  |  |  | ms | 9.5 |  |  |  |
| Maximum number of HARQ transmission |  |  |  |  | 1 |  |  |  |
| Measurement channel |  |  |  |  | As specified in Table A.4-1, TBS.2-4 |  |  |  |

##### 6.2.2.2.2	Wideband CQI reporting under fading conditions

6.2.2.2.2.1	Minimum requirement for wideband CQI reporting

The purpose of the requirements is to verify that the UE is tracking the channel variations and selecting the largest transport format possible according to the prevailing channel state for the frequency non-selective scheduling.

The reporting accuracy of CQI under frequency non-selective fading conditions is determined by the reporting variance, the relative increase of the throughput obtained when the transport format is indicated by the reported CQI compared to the throughput obtained when a fixed transport format is configured according to the reported median CQI, and a minimum BLER using the transport formats indicated by the reported CQI.

For the parameters specified in Table 6.2.2.2.2.1-1 and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified by the following:

a)	A CQI index not in the set {median CQI -1, median CQI, median CQI +1} shall be reported at least α% of the time where α% is specified in Table 6.2.2.2.2.1-2;

b)	The ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index and that obtained when transmitting a fixed transport format configured according to the wideband CQI median shall be ≥ γ, where γ is specified in Table 6.2.2.2.2.1-2;

c)	When transmitting the transport format indicated by each reported wideband CQI index, the average BLER for the indicated transport formats shall be greater than or equal to [0.02].

Table 6.2.2.2.2.1-1: Wideband CQI reporting test under frequency non-selective fading conditions

| Parameter |  |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  |  | MHz | 40 |  |  |  |
| Subcarrier spacing |  |  |  | kHz | 30 |  |  |  |
| Duplex Mode |  |  |  |  | TDD |  |  |  |
| TDD UL-DL pattern |  |  |  |  | FR1.30-1 |  |  |  |
| SNR |  |  |  | dB | 6 | 7 | 12 | 13 |
| Propagation channel |  |  |  |  | TDLA30-5 |  |  |  |
| Antenna configuration |  |  |  |  | 2×2 |  |  |  |
| Correlation configuration |  |  |  |  | ULA high |  |  |  |
| Beamforming Model |  |  |  |  | As specified in Section Annex B.4.1 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  |  | 4 |  |  |  |
|  | CDM Type |  |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  |  | Row 5,4 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | 9 |  |  |  |
|  | CSI-RS periodicity and offset |  |  | slot | 10/1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  |  | 2 |  |  |  |
|  | CDM Type |  |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | Row 3,(6,-) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  |  | slot | 10/1 |  |  |  |
| CSI-IM configuration | CSI-IM RE pattern |  |  |  | 0 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  |  | (4, 9) |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  |  | slot | 10/1 |  |  |  |
| ReportConfigType |  |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  |  | Table 2 |  |  |  |
| reportQuantity |  |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  |  | RB | 16 |  |  |  |
| Csi-ReportingBand |  |  |  |  | 1111111 |  |  |  |
| CSI-Report periodicity and offset |  |  |  | slot | 10/1 |  |  |  |
| aperiodicTriggeringOffset |  |  |  |  | Not configured |  |  |  |
| Codebook configuration |  | Codebook Type |  |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  |  | Not configured |  |  |  |
|  |  | CodebookSubsetRestriction |  |  | 000001 |  |  |  |
|  |  | RI Restriction |  |  | N/A |  |  |  |
| Physical channel for CSI report |  |  |  |  | PUCCH |  |  |  |
| CQI/RI/PMI delay |  |  |  | ms | 9.5 |  |  |  |
| Maximum number of HARQ transmission |  |  |  |  | 1 |  |  |  |
| Measurement channel |  |  |  |  | As specified in Table A.4-1, TBS.2-3 |  |  |  |

Table 6.2.2.2.2.1-2: Minimum requirements

| Parameters | Test 1 | Test 2 |
| --- | --- | --- |
|  [%] | [20] | [20] |
|  | [1.05] | [1.05] |

6.2.2.2.2.2	Minimum requirement for sub-band CQI reporting

The purpose of the requirements is to verify that the preferred sub-bands can be used for frequency-selective scheduling under the frequency-selective fading conditions.

The accuracy of sub-band channel CQI reporting under the frequency-selective fading conditions is determined by a double-sided percentile of  the reported differential CQI offset level 0 per sub-band, and the relative increase of the throughput obtained when transmitting the transport format indicated by the corresponding reported sub-band CQI on a randomly selected sub-band among the sub-bands with the highest reported differential CQI offset level compared to the throughput when transmitting a fixed transport format according to the wideband CQI median on a randomly selected sub-band among all the sub-bands.

For the parameters specified in Table 6.2.2.2.2.2-1 and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified by the following:

a)	A sub-band differential CQI offset level of 0 shall be reported at least α% of the time but less than β% of the time for each sub-band, where α and β are specified in Table 6.2.2.2.2.2-2;

b)	The ratio of the throughput obtained when transmitting the corresponding transport format on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level and that obtained when transmitting the transport format indicated by the reported wideband CQI median on a randomly selected sub-band among all the sub-bands shall be ≥ γ, where γ is specified in Table 6.2.2.2.2.2-2;

c)	When transmitting the corresponding transport format on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level, the average BLER for the indicated transport format shall be greater than or equal to TBD.

The requirements only apply for sub-bands of full size and the random scheduling across the sub-bands is done by selecting a new sub-band in each available downlink transmission instance for TDD.

Table 6.2.2.2.2.2-1: Sub-band CQI reporting test under frequency-selective fading conditions

| Parameter |  |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  |  | MHz | 40 |  |  |  |
| Subcarrier spacing |  |  |  | kHz | 30 |  |  |  |
| Duplex Mode |  |  |  |  | TDD |  |  |  |
| TDD UL-DL pattern |  |  |  |  | FR1.30-1 |  |  |  |
| SNR |  |  |  | dB | [8] | [9] | [14] | [15] |
| Propagation channel |  |  |  |  | Two tap model specified in Annex B.2.4 with a=1, fD = 5Hz, and τd=0.1125μs |  |  |  |
| Antenna configuration |  |  |  |  | 2×2 |  |  |  |
| Correlation configuration |  |  |  |  | As per Annex B.1 |  |  |  |
| Beamforming Model |  |  |  |  | As specified in Section Annex B.4.1 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  |  | 4 |  |  |  |
|  | CDM Type |  |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  |  | Row 5,4 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | 9 |  |  |  |
|  | CSI-RS periodicity and offset |  |  | slot | 10/1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  |  | 2 |  |  |  |
|  | CDM Type |  |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | Row 3,(6,-) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  |  | slot | 10/1 |  |  |  |
| CSI-IM configuration | CSI-IM RE pattern |  |  |  | 0 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  |  | (4, 9) |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  |  | slot | 10/1 |  |  |  |
| ReportConfigType |  |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  |  | Table 2 |  |  |  |
| reportQuantity |  |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  |  | Subband |  |  |  |
| pmi-FormatIndicator |  |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  |  | RB | 16 |  |  |  |
| CSI-Report periodicity and offset |  |  |  | slot | 10/1 |  |  |  |
| aperiodicTriggeringOffset |  |  |  |  | Not configured |  |  |  |
| Codebook configuration |  | Codebook Type |  |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  |  | Not configured |  |  |  |
|  |  | CodebookSubsetRestriction |  |  | 000001 |  |  |  |
|  |  | RI Restriction |  |  | N/A |  |  |  |
| Physical channel for CSI report |  |  |  |  | TBD |  |  |  |
| CQI/RI/PMI delay |  |  |  | ms | 9.5 |  |  |  |
| Maximum number of HARQ transmission |  |  |  |  | 1 |  |  |  |
| Measurement channel |  |  |  |  | As specified in Table A.4-2, TBS.2-6 |  |  |  |

Table 6.2.2.2.2.2-2: Minimum requirements

| Parameters | Test 1 | Test 2 |
| --- | --- | --- |
| α [%] | [2] | [2] |
| β [%] | [55] | [55] |
|  | [1.05] | [1.05] |

### 6.2.3	4RX requirements

This sub-clause includes the requirements for reporting of CQI for UE equipped with 4 receiver antennas.

#### 6.2.3.1	FDD

##### 6.2.3.1.1	CQI reporting definition under AWGN conditions

The purpose of the requirements is to verify that the reported CQI values are in accordance with the CQI definition given in TS38.214 [12]. The reporting accuracy of CQI under AWGN condition is determined by the reporting variance and BLER performance using the transport format indicated by the reported CQI median.

6.2.3.1.1.1	Minimum requirement for period CQI reporting

For the parameters specified in Table 6.2.3.1.1.1-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified by the following:

a)	The reported CQI value according to the reference channel shall be in the range of ±1 of the reported median more than [90]% of the time.

b)	If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, then the BLER using the transport format indicated by the (median CQI+1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, then the BLER using transport format indicated by (median CQI-1) shall be less than or equal to 0.1.

Table 6.2.2.1.1.1-1: CQI reporting definition test

| Parameter |  |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  |  | MHz | 10 |  |  |  |
| Subcarrier spacing |  |  |  | kHz | 15 |  |  |  |
| Duplex Mode |  |  |  |  | FDD |  |  |  |
| SNR |  |  |  | dB | [5] | [6] | [11] | [12] |
| Propagation channel |  |  |  |  | AWGN |  |  |  |
| Antenna configuration |  |  |  |  | 2×4 with static channel specified in Annex B.1 |  |  |  |
| Beamforming Model |  |  |  |  | As specified in Section Annex B.4.1 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  |  | 4 |  |  |  |
|  | CDM Type |  |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  |  | Row 5,4 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | 9 |  |  |  |
|  | CSI-RS periodicity and offset |  |  | slot | 5/1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  |  | 2 |  |  |  |
|  | CDM Type |  |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | Row 3,(6,-) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  |  | slot | 5/1 |  |  |  |
| CSI-IM configuration | CSI-IM RE pattern |  |  |  | 0 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  |  | (4, 9) |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  |  | slot | 5/1 |  |  |  |
| ReportConfigType |  |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  |  | Table 2 |  |  |  |
| reportQuantity |  |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  |  | RB | N/A |  |  |  |
| CSI-Report periodicity and offset |  |  |  | slot | 5/1 |  |  |  |
| aperiodicTriggeringOffset |  |  |  |  | Not configured |  |  |  |
| Codebook configuration |  | Codebook Type |  |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  |  | Not configured |  |  |  |
|  |  | CodebookSubsetRestriction |  |  | 010000 |  |  |  |
|  |  | RI Restriction |  |  | N/A |  |  |  |
| Physical channel for CSI report |  |  |  |  | PUCCH |  |  |  |
| CQI/RI/PMI delay |  |  |  | ms | 8 |  |  |  |
| Maximum number of HARQ transmission |  |  |  |  | 1 |  |  |  |
| Measurement channel |  |  |  |  | As specified in Table A.4-1, TBS.2-2 |  |  |  |

##### 6.2.3.1.2	Wideband CQI reporting under fading conditions

6.2.3.1.2.1	Minimum requirement for wideband CQI reporting

The purpose of the requirements is to verify that the UE is tracking the channel variations and selecting the largest transport format possible according to the prevailing channel state for the frequency non-selective scheduling.

The reporting accuracy of CQI under frequency non-selective fading conditions is determined by the reporting variance, the relative increase of the throughput obtained when the transport format is indicated by the reported CQI compared to the throughput obtained when a fixed transport format is configured according to the reported median CQI, and a minimum BLER using the transport formats indicated by the reported CQI.

For the parameters specified in Table 6.2.3.1.2.1-1 and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified by the following:

a)	A CQI index not in the set {median CQI -1, median CQI, median CQI +1} shall be reported at least α% of the time where α% is specified in Table 6.2.3.1.2.1-2;

b)	The ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index and that obtained when transmitting a fixed transport format configured according to the wideband CQI median shall be ≥ γ, where γ is specified in Table 6.2.3.1.2.1-2;

c)	When transmitting the transport format indicated by each reported wideband CQI index, the average BLER for the indicated transport formats shall be greater than or equal to [0.02].

Table 6.2.3.1.2.1-1: Wideband CQI reporting test under frequency non-selective fading conditions

| Parameter |  |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  |  | MHz | 10 |  |  |  |
| Subcarrier spacing |  |  |  | kHz | 15 |  |  |  |
| Duplex Mode |  |  |  |  | FDD |  |  |  |
| SNR |  |  |  | dB | [3] | [4] | [9] | [10] |
| Propagation channel |  |  |  |  | TDLA30-5 |  |  |  |
| Antenna configuration |  |  |  |  | 2×4 |  |  |  |
| Correlation configuration |  |  |  |  | XP High |  |  |  |
| Beamforming Model |  |  |  |  | As specified in Section Annex B.4.1 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  |  | 4 |  |  |  |
|  | CDM Type |  |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  |  | Row 5,4 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | 9 |  |  |  |
|  | CSI-RS periodicity and offset |  |  | slot | 5/1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  |  | 2 |  |  |  |
|  | CDM Type |  |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | Row 3,(6,-) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  |  | slot | 5/1 |  |  |  |
| CSI-IM configuration | CSI-IM RE pattern |  |  |  | 0 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  |  | (4, 9) |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  |  | slot | 5/1 |  |  |  |
| ReportConfigType |  |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  |  | Table 2 |  |  |  |
| reportQuantity |  |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  |  | RB | 8 |  |  |  |
| csi-ReportingBand |  |  |  |  | 1111111 |  |  |  |
| CSI-Report periodicity and offset |  |  |  | slot | 5/1 |  |  |  |
| aperiodicTriggeringOffset |  |  |  |  | Not configured |  |  |  |
| Codebook configuration |  | Codebook Type |  |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  |  | Not configured |  |  |  |
|  |  | CodebookSubsetRestriction |  |  | 000001 |  |  |  |
|  |  | RI Restriction |  |  | N/A |  |  |  |
| Physical channel for CSI report |  |  |  |  | PUCCH |  |  |  |
| CQI/RI/PMI delay |  |  |  | ms | 8 |  |  |  |
| Maximum number of HARQ transmission |  |  |  |  | 1 |  |  |  |
| Measurement channel |  |  |  |  | As specified in Table A.4-1, TBS.2-1 |  |  |  |

Table 6.2.3.1.2.1-2: Minimum requirements

| Parameters | Test 1 | Test 2 |
| --- | --- | --- |
|  [%] | [5] | [5] |
|  | [1.05] | [1.05] |

6.2.3.1.2.2	Minimum requirement for sub-band CQI reporting

The purpose of the requirements is to verify that the preferred sub-bands can be used for frequency-selective scheduling under the frequency-selective fading conditions.

The accuracy of sub-band channel CQI reporting under the frequency-selective fading conditions is determined by a double-sided percentile of  the reported differential CQI offset level 0 per sub-band, and the relative increase of the throughput obtained when transmitting the transport format indicated by the corresponding reported sub-band CQI on a randomly selected sub-band among the sub-bands with the highest reported differential CQI offset level compared to the throughput when transmitting a fixed transport format according to the wideband CQI median on a randomly selected sub-band among all the sub-bands.

For the parameters specified in Table 6.2.3.1.2.2-1 and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified by the following:

a)	A sub-band differential CQI offset level of 0 shall be reported at least α% of the time but less than β% of the time for each sub-band, where α and β are specified in Table 6.2.3.1.2.2-2;

b)	The ratio of the throughput obtained when transmitting the corresponding transport format on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level and that obtained when transmitting the transport format indicated by the reported wideband CQI median on a randomly selected sub-band among all the sub-bands shall be ≥ γ, where γ is specified in Table 6.2.3.1.2.2-2;

c)	When transmitting the corresponding transport format on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level, the average BLER for the indicated transport format shall be greater than or equal to TBD.

The requirements only apply for sub-bands of full size and the random scheduling across the sub-bands is done by selecting a new sub-band in each TTI for FDD.

Table 6.2.3.1.2.2-1: Sub-band CQI reporting test under frequency-selective fading conditions

| Parameter |  |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  |  | MHz | 10 |  |  |  |
| Subcarrier spacing |  |  |  | kHz | 15 |  |  |  |
| Duplex Mode |  |  |  |  | FDD |  |  |  |
| SNR |  |  |  | dB | [5] | [6] | [11] | [12] |
| Propagation channel |  |  |  |  | Two tap model specified in Annex B.2.4 with a=1, fD = 5Hz, and τd=0.45μs |  |  |  |
| Antenna configuration |  |  |  |  | 2×4 |  |  |  |
| Correlation configuration |  |  |  |  | As per Annex B.1 |  |  |  |
| Beamforming Model |  |  |  |  | As specified in Section Annex B.4.1 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  |  | 4 |  |  |  |
|  | CDM Type |  |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  |  | Row 5,4 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | 9 |  |  |  |
|  | CSI-RS periodicity and offset |  |  | slot | 5/1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  |  | 2 |  |  |  |
|  | CDM Type |  |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | Row 3,(6,-) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  |  | slot | 5/1 |  |  |  |
| CSI-IM configuration | CSI-IM RE pattern |  |  |  | 0 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  |  | (4, 9) |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  |  | slot | 5/1 |  |  |  |
| ReportConfigType |  |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  |  | Table 2 |  |  |  |
| reportQuantity |  |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  |  | Subband |  |  |  |
| pmi-FormatIndicator |  |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  |  | RB | 8 |  |  |  |
| CSI-Report periodicity and offset |  |  |  | slot | 5/1 |  |  |  |
| aperiodicTriggeringOffset |  |  |  |  | Not configured |  |  |  |
| Codebook configuration |  | Codebook Type |  |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  |  | Not configured |  |  |  |
|  |  | CodebookSubsetRestriction |  |  | 000001 |  |  |  |
|  |  | RI Restriction |  |  | N/A |  |  |  |
| Physical channel for CSI report |  |  |  |  | TBD |  |  |  |
| CQI/RI/PMI delay |  |  |  | ms | 8 |  |  |  |
| Maximum number of HARQ transmission |  |  |  |  | 1 |  |  |  |
| Measurement channel |  |  |  |  | As specified in Table A.4-2, TBS.2-5 |  |  |  |

Table 6.2.3.1.2.2-2: Minimum requirements

| Parameters | Test 1 | Test 2 |
| --- | --- | --- |
| α [%] | [2] | [2] |
| β [%] | [55] | [55] |
|  | [1.05] | [1.05] |

#### 6.2.3.2	TDD

##### 6.2.3.2.1	CQI reporting definition under AWGN

6.2.3.2.1.1	Minimum requirement for CQI periodic reporting

The purpose of the requirements is to verify that the reported CQI values are in accordance with the CQI definition given in TS38.214 [12]. The reporting accuracy of CQI under AWGN condition is determined by the reporting variance and BLER performance using the transport format indicated by the reported CQI median.

For the parameters specified in Table 6.2.3.2.1.1-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified by the following:

a)	The reported CQI value according to the reference channel shall be in the range of ±1 of the reported median more than [90]% of the time.

b)	If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, then the BLER using the transport format indicated by the (median CQI+1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, then the BLER using transport format indicated by (median CQI-1) shall be less than or equal to 0.1.

Table 6.2.3.2.1.1-1: CQI reporting definition test

| Parameter |  |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  |  | MHz | 40 |  |  |  |
| Subcarrier spacing |  |  |  | kHz | 30 |  |  |  |
| Duplex Mode |  |  |  |  | TDD |  |  |  |
| TDD UL-DL pattern |  |  |  |  | FR1.30-1 |  |  |  |
| SNR |  |  |  | dB | [5] | [6] | [11] | [12] |
| Propagation channel |  |  |  |  | AWGN |  |  |  |
| Antenna configuration |  |  |  |  | 2×4 with static channel specified in Annex B.1 |  |  |  |
| Beamforming Model |  |  |  |  | As specified in Section Annex B.4.1 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  |  | 4 |  |  |  |
|  | CDM Type |  |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  |  | Row 5,4 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | 9 |  |  |  |
|  | CSI-RS periodicity and offset |  |  | slot | 10/1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  |  | 2 |  |  |  |
|  | CDM Type |  |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | Row 3,(6,-) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  |  | slot | 10/1 |  |  |  |
| CSI-IM configuration | CSI-IM RE pattern |  |  |  | 0 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  |  | (4, 9) |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  |  | slot | 10/1 |  |  |  |
| ReportConfigType |  |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  |  | Table 2 |  |  |  |
| reportQuantity |  |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  |  | RB | N/A |  |  |  |
| CSI-Report periodicity and offset |  |  |  | slot | 10/1 |  |  |  |
| aperiodicTriggeringOffset |  |  |  |  | Not configured |  |  |  |
| Codebook configuration |  | Codebook Type |  |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  |  | Not configured |  |  |  |
|  |  | CodebookSubsetRestriction |  |  | 010000 |  |  |  |
|  |  | RI Restriction |  |  | N/A |  |  |  |
| Physical channel for CSI report |  |  |  |  | PUCCH |  |  |  |
| CQI/RI/PMI delay |  |  |  | ms | 9.5 |  |  |  |
| Maximum number of HARQ transmission |  |  |  |  | 1 |  |  |  |
| Measurement channel |  |  |  |  | As specified in Table A.4-1, TBS.2-4 |  |  |  |

##### 6.2.3.2.2	Wideband CQI reporting under fading conditions

6.2.3.2.2.1	Minimum requirement for wideband CQI reporting

The purpose of the requirements is to verify that the UE is tracking the channel variations and selecting the largest transport format possible according to the prevailing channel state for the frequency non-selective scheduling.

The reporting accuracy of CQI under frequency non-selective fading conditions is determined by the reporting variance, the relative increase of the throughput obtained when the transport format is indicated by the reported CQI compared to the throughput obtained when a fixed transport format is configured according to the reported median CQI, and a minimum BLER using the transport formats indicated by the reported CQI.

For the parameters specified in Table 6.2.3.2.2.1-1 and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified by the following:

a)	A CQI index not in the set {median CQI -1, median CQI, median CQI +1} shall be reported at least α% of the time where α% is specified in Table 6.2.3.2.2.1-2;

b)	The ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index and that obtained when transmitting a fixed transport format configured according to the wideband CQI median shall be ≥ γ, where γ is specified in Table 6.2.3.2.2.1-2;

c)	When transmitting the transport format indicated by each reported wideband CQI index, the average BLER for the indicated transport formats shall be greater than or equal to [0.02].

Table 6.2.3.2.2.1-1: Wideband CQI reporting test under frequency non-selective fading conditions

| Parameter |  |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  |  | MHz | 40 |  |  |  |
| Subcarrier spacing |  |  |  | kHz | 30 |  |  |  |
| Duplex Mode |  |  |  |  | TDD |  |  |  |
| TDD UL-DL pattern |  |  |  |  | FR1.30-1 |  |  |  |
| SNR |  |  |  |  | dB |  | [3] |  | [4] | [9] |  | [10] |
| Propagation channel |  |  |  |  | TDLA30-5 |  |  |  |
| Antenna configuration |  |  |  |  | 2×4 |  |  |  |
| Correlation configuration |  |  |  |  | XP High |  |  |  |
| Beamforming Model |  |  |  |  | As specified in Section Annex B.4.1 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  |  | 4 |  |  |  |
|  | CDM Type |  |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  |  | Row 5,4 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | 9 |  |  |  |
|  | CSI-RS periodicity and offset |  |  | slot | 10/1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  |  | 2 |  |  |  |
|  | CDM Type |  |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | Row 3,(6,-) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  |  | slot | 10/1 |  |  |  |
| CSI-IM configuration | CSI-IM RE pattern |  |  |  | 0 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  |  | (4, 9) |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  |  | slot | 10/1 |  |  |  |
| ReportConfigType |  |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  |  | Table 2 |  |  |  |
| reportQuantity |  |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  |  | RB | 16 |  |  |  |
| csi-ReportingBand |  |  |  |  | 1111111 |  |  |  |
| CSI-Report periodicity and offset |  |  |  | slot | 10/1 |  |  |  |
| aperiodicTriggeringOffset |  |  |  |  | Not configured |  |  |  |
| Codebook configuration |  | Codebook Type |  |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  |  | Not configured |  |  |  |
|  |  | CodebookSubsetRestriction |  |  | 000001 |  |  |  |
|  |  | RI Restriction |  |  | N/A |  |  |  |
| Physical channel for CSI report |  |  |  |  | PUCCH |  |  |  |
| CQI/RI/PMI delay |  |  |  | ms | 9.5 |  |  |  |
| Maximum number of HARQ transmission |  |  |  |  | 1 |  |  |  |
| Measurement channel |  |  |  |  | As specified in Table A.4-1, TBS.2-3 |  |  |  |

Table 6.2.3.2.2.1-2: Minimum requirements

| Parameters | Test 1 | Test 2 |
| --- | --- | --- |
|  [%] | [5] | [5] |
|  | [1.05] | [1.05] |

6.2.3.2.2.2	Minimum requirement for sub-band CQI reporting

The purpose of the requirements is to verify that the preferred sub-bands can be used for frequency-selective scheduling under the frequency-selective fading conditions.

The accuracy of sub-band channel CQI reporting under the frequency-selective fading conditions is determined by a double-sided percentile of  the reported differential CQI offset level 0 per sub-band, and the relative increase of the throughput obtained when transmitting the transport format indicated by the corresponding reported sub-band CQI on a randomly selected sub-band among the sub-bands with the highest reported differential CQI offset level compared to the throughput when transmitting a fixed transport format according to the wideband CQI median on a randomly selected sub-band among all the sub-bands.

For the parameters specified in Table 6.2.3.2.2.2-1 and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified by the following:

a)	A sub-band differential CQI offset level of 0 shall be reported at least α% of the time but less than β% of the time for each sub-band, where α and β are specified in Table 6.2.3.2.2.2-2;

b)	The ratio of the throughput obtained when transmitting the corresponding transport format on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level and that obtained when transmitting the transport format indicated by the reported wideband CQI median on a randomly selected sub-band among all the sub-bands shall be ≥ γ, where γ is specified in Table 6.2.3.2.2.2-2;

c)	When transmitting the corresponding transport format on a randomly selected sub-band among the sub-bands with the highest differential CQI offset level, the average BLER for the indicated transport format shall be greater than or equal to TBD.

The requirements only apply for sub-bands of full size and the random scheduling across the sub-bands is done by selecting a new sub-band in each available downlink transmission instance for TDD.

Table 6.2.3.2.2.2-1: Sub-band CQI reporting test under frequency-selective fading conditions

| Parameter |  |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  |  | MHz | 40 |  |  |  |
| Subcarrier spacing |  |  |  | kHz | 30 |  |  |  |
| Duplex Mode |  |  |  |  | TDD |  |  |  |
| TDD UL-DL pattern |  |  |  |  | FR1.30-1 |  |  |  |
| SNR |  |  |  | dB | [5] | [6] | [11] | [12] |
| Propagation channel |  |  |  |  | Two tap model specified in Annex B.2.4 with a=1, fD = 5Hz, and τd=0.1125μs |  |  |  |
| Antenna configuration |  |  |  |  | 2×4 |  |  |  |
| Correlation configuration |  |  |  |  | As per Annex B.1 |  |  |  |
| Beamforming Model |  |  |  |  | As specified in Section Annex B.4.1 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  |  | 4 |  |  |  |
|  | CDM Type |  |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  |  | Row 5,4 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | 9 |  |  |  |
|  | CSI-RS periodicity and offset |  |  | slot | 10/1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  |  | 2 |  |  |  |
|  | CDM Type |  |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | Row 3,(6,-) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  |  | slot | 10/1 |  |  |  |
| CSI-IM configuration | CSI-IM RE pattern |  |  |  | 0 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  |  | (4, 9) |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  |  | slot | 10/1 |  |  |  |
| ReportConfigType |  |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  |  | Table 2 |  |  |  |
| reportQuantity |  |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  |  | Subband |  |  |  |
| pmi-FormatIndicator |  |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  |  | RB | 16 |  |  |  |
| CSI-Report periodicity and offset |  |  |  | slot | 10/1 |  |  |  |
| aperiodicTriggeringOffset |  |  |  |  | Not configured |  |  |  |
| Codebook configuration |  | Codebook Type |  |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  |  | Not configured |  |  |  |
|  |  | CodebookSubsetRestriction |  |  | 000001 |  |  |  |
|  |  | RI Restriction |  |  | N/A |  |  |  |
| Physical channel for CSI report |  |  |  |  | TBD |  |  |  |
| CQI/RI/PMI delay |  |  |  | ms | 9.5 |  |  |  |
| Maximum number of HARQ transmission |  |  |  |  | 1 |  |  |  |
| Measurement channel |  |  |  |  | As specified in Table A.4-2, TBS.2-6 |  |  |  |

Table 6.2.2.1.2.2-2: Minimum requirements

| Parameters | Test 1 | Test 2 |
| --- | --- | --- |
| α [%] | [2] | [2] |
| β [%] | [55] | [55] |
|  | [1.05] | [1.05] |

< ---------------                                                         Start of next change                                                                  --------------------->

# A.4	CSI reference measurement channels

This section defines the DL signal applicable to the reporting of channel status information (Clause X).

Tables in this section specifies the mapping of CQI index to Information Bit payload, which complies with the CQI definition specified in TS 38.214 [12, Section 5.2.2.1] and with MCS definition specified in TS 38.214 [12, Section 5.1.3].

Table A.4-1: Mapping of CQI Index to Information Bit payload (CQI table 1)

| TBS Scheme |  |  |  | TBS.1-1 | TBS.1-2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MCS table |  |  |  | 64QAM |  |  |  |  |  |
| Number of allocated PDSCH resource blocks |  |  |  | 66 | 66 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  | 12 | 12 |  |  |  |  |
| Number of PDSCH MIMO layers |  |  |  | 1 | 2 |  |  |  |  |
| Number of DMRS rEs (Note 1) |  |  |  | 24 | 24 |  |  |  |  |
| Overhead for TBS determination |  |  |  | 6 | 6 |  |  |  |  |
| Available RE-s |  |  |  | 7920 | 7920 |  |  |  |  |
| CQI index | Spectral efficiency | MCS index | Modulation | Information Bit Payload per Slot |  |  |  |  |  |
| 0 | OOR | OOR | OOR | N/A | N/A |  |  |  |  |
| 1 | 0.1523 | 0 | QPSK | 1800 | 3624 |  |  |  |  |
| 2 | 0.2344 | 0 |  | 1800 | 3624 |  |  |  |  |
| 3 | 0.3770 | 2 |  | 2856 | 5640 |  |  |  |  |
| 4 | 0.6016 | 4 |  | 4480 | 8968 |  |  |  |  |
| 5 | 0.8770 | 6 |  | 6528 | 13064 |  |  |  |  |
| 6 | 1.1758 | 8 |  | 8712 | 17928 |  |  |  |  |
| 7 | 1.4766 | 11 | 16QAM | 11016 | 22032 |  |  |  |  |
| 8 | 1.9141 | 13 |  | 14343 | 28680 |  |  |  |  |
| 9 | 2.4063 | 15 |  | 17928 | 35856 |  |  |  |  |
| 10 | 2.7305 | 18 | 64QAM | 20496 | 40976 |  |  |  |  |
| 11 | 3.3223 | 20 |  | 25104 | 50184 |  |  |  |  |
| 12 | 3.9023 | 22 |  | 29192 | 58384 |  |  |  |  |
| 13 | 4.5234 | 24 |  | 33816 | 67584 |  |  |  |  |
| 14 | 5.1152 | 26 |  | 38936 | 77896 |  |  |  |  |
| 15 | 5.5547 | 28 |  | 42016 | 83976 |  |  |  |  |
| Note 1: Number of DMRS rEs includes the overhead of the DM-RS CDM groups without data |  |  |  |  |  |  |  |  |  |

Table A.4-2: Mapping of CQI Index to Information Bit payload (CQI table 2)

| TBS Scheme |  |  |  | TBS.2-1 | TBS.2-2 | TBS.2-3 | TBS.2-4 | TBS.2-5 | TBS.2-6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MCS table |  |  |  | 256QAM |  |  |  |  |  |
| Number of allocated PDSCH resource blocks |  |  |  | 52 | 52 | 106 | 106 | 8 | 16 |
| Number of consecutive PDSCH symbols |  |  |  | 12 | 12 | 12 | 12 | 12 | 12 |
| Number of PDSCH MIMO layers |  |  |  | 1 | 2 | 1 | 2 | 1 | 1 |
| Number of DMRS rEs (Note 1) |  |  |  | 24 | 24 | 24 | 24 | 24 | 24 |
| Overhead for TBS determination |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 |
| Available RE-s for PDSCH |  |  |  | 6240 | 6240 | 12720 | 12720 | 960 | 1920 |
| CQI index | Spectral efficiency | MCS index | Modulation | Information Bit Payload per Slot |  |  |  |  |  |
| 0 | OOR | OOR | OOR | N/A | N/A | N/A | N/A | N/A | N/A |
| 1 | 0.1523 | 0 | QPSK | 1480 | 2976 | 2976 | 5896 | 224 | 456 |
| 2 | 0.3770 | 1 |  | 2408 | 4744 | 4744 | 9480 | 368 | 736 |
| 3 | 0.8770 | 3 |  | 5504 | 11016 | 11016 | 22536 | 848 | 1736 |
| 4 | 1.4766 | 5 | 16QAM | 9224 | 18432 | 18960 | 37896 | 1416 | 2856 |
| 5 | 1.9141 | 7 |  | 12040 | 24072 | 24576 | 49176 | 1864 | 3752 |
| 6 | 2.4063 | 9 |  | 15112 | 30216 | 30728 | 61480 | 2408 | 4608 |
| 7 | 2.7305 | 11 | 64QAM | 16896 | 33816 | 34816 | 69672 | 2600 | 5248 |
| 8 | 3.3223 | 13 |  | 20496 | 40976 | 42016 | 83976 | 3240 | 6400 |
| 9 | 3.9023 | 15 |  | 24576 | 49176 | 49176 | 98376 | 3752 | 7424 |
| 10 | 4.5234 | 17 |  | 28168 | 56368 | 57376 | 114776 | 4352 | 8712 |
| 11 | 5.1152 | 19 |  | 31752 | 63528 | 65576 | 131176 | 4864 | 9736 |
| 12 | 5.5547 | 21 | 256QAM | 34816 | 69672 | 69672 | 139376 | 5248 | 10760 |
| 13 | 6.2266 | 23 |  | 38936 | 77896 | 79896 | 159880 | 6016 | 12040 |
| 14 | 6.9141 | 25 |  | 43032 | 86040 | 88064 | 176208 | 6656 | 13320 |
| 15 | 7.4063 | 27 |  | 46104 | 92200 | 94248 | 188576 | 7040 | 14088 |
| Note 1: Number of DMRS rEs includes the overhead of the DM-RS CDM groups without data |  |  |  |  |  |  |  |  |  |

< -----------------                                                        End of change                                                                  --------------------->