3GPP TSG-RAN WG4 Meeting #91	R4-1905539

Reno, USA, 13th Apr – 17th May, 2019

Source: 	ZTE

Title:	on IAB EMC

Agenda Item:	8.5.5

Document for:	Approval

Introduction

The new WID of integrated access and backhaul is ongoing and it is agreed to have a new TS for IAB RF specification. In this contribution, we provide some analysis of IAB EMC based on current IAB discussion and two options are given to capture the IAB EMC content in RAN4 spec.

Discussion

Repeater EMC captured in E-UTRA spec

The repeater EMC is captured in current TS 36.113[1] as the scope shows:

-	repeaters of E-UTRA meeting the requirements of TS 36.106 [4], with conformance demonstrated by compliance to TS 36.143 [5].

If we look into the spec, specific content of repeater are listed in corresponding subclauses:

Table 1 subclauses containing specific content of repeaters in TS 36.113

| Subclause 4.4 | Test conditions for repeaters |
| --- | --- |
| Subclause 5.5 | Performance assessment for repeaters |
| Subclause 6.5 | Performance criteria for continuous phenomena for repeaters |
| Subclause 6.6 | Performance criteria for transient phenomena for repeaters |
| Subclause 7 | Applicability of emission and immunity test |
| Subclause 8 | Emission test for repeater |
| Subclause 9 | Immunity test for repeater |

From table 1 above, we can see that almost every part of the TS 36.113 is impacted by adding repeater in the scope.

Observation 1: Almost every subclause of BS EMC specification are impacted when introducing repeater in the spec.

IAB EMC

For IAB functions, based on the TR 38.874[2], the function of IAB can be shown as:

Figure 1 Different IAB link types

From figure 1, when considering in parent BH perspective, the IAB node is more like a UE while for DL child BH and Access link the IAB node is more like a BS. In E-UTRA EMC spec TS 36.113, the EMC requirements are similar for LTE BS and LTE repeater. However, if we compare current EMC requirements for UE in TS 38.124[3] and BS in TS 38.113[4], Here we list some difference in table 2.

Table 2 Difference between UE EMC and BS EMC

|  | Requirements of UE in TS 38.124 | Requirements of BS in TS 38.113 |
| --- | --- | --- |
| Radiated immunity | Frequency range: 80 MHz – 1000 MHz and [1400] MHZ to [2700] MHz Test level: 3V/m | Frequency range: 80 MHz – 6000 MHz Test level: 3V/m |
| Exclusion band | 85MHz | For BS type 1-C and 1-H: 20/60MHz For BS type 1-O: 60/200MHz |
| Radiated Emission | Ideal mode and traffic mode | Same requirement as UE traffic mode |
| Other Emission requirements | Not defined | Conducted emission Harmonic current emission Voltage fluctuation and flicker |

As the immunity requirements in TS 38.124 hasn’t finished yet, we believe there will be more difference between UE and BS besides which has been shown in table 2.

Observation 2: There are a lot of difference between UE EMC and BS EMC requirements.

As the IAB is functioning like both UE and BS, the EMC requirements for IAB should consider both UE and EMC requirements. However, based on observation 2 above, more analysis and work are needed to a combined requirement for IAB node.

Introducing IAB EMC requirement in 38 series

Current TS 38.113 scope only covers:

The present document specifies the applicable test conditions, performance assessment and performance criteria for base stations and associated ancillary equipment in the following categories:

-	BS equipped with antenna connectors or TAB connectors which are possible to be terminated during EMC testing, meeting the BS type 1-C and BS type 1-H RF requirements of TS 38.104 [2], with conformance demonstrated by compliance to TS 38.141-1 [3].

-	BS not equipped with antenna connectors nor TAB connectors, i.e. with antenna elements radiating during the EMC testing, meeting the BS type 1-O and BS type 2-O RF requirements of TS 38.104 [2], with conformance demonstrated by compliance to TS 38.141-2 [4].

Observation 3: IAB is not in the scope of TS 38.113.

Current work item of IAB[5] listed that:

  - Define RF requirements for both backhaul and access links of an IAB-node including requirements for co-existence (e.g. ACLR, ACS). This may include defining a new power class for MT.

In practical, when we define the RF requirements of a base station or IAB node here, the EMC requirement should be defined correspondingly. Otherwise the IAB node cannot work properly without the EMC requirements. So we propose to add the IAB EMC into current IAB work item.

Proposal 1: Add EMC requirements of an IAB-node into the IAB work item.

To introduce IAB EMC requirements in 38 series, there are two options provided:

Option 1: Same method as LTE repeater, add the IAB EMC requirements in TS 38.113 specification.

Option 2: Create a new TS to specifically address the EMC requirements for IAB as TS 38.XXX.

Base on observation 3, if we want to add IAB EMC requirement in TS 38.113, the scope need to be updated.

Also considering observation 1, almost every subclause of TS 38.113 would be impacted if we introduce the IAB EMC.

And considering table 2 and observation 2, unlike LTE repeater, the IAB node will have more difference to BS requirements .

So to reduce the specification complexity and give us more specific study of IAB EMC, we prefer option 2.

Proposal 2: To have a new TS to capture IAB EMC requirements in Rel-16.

Conclusion

In this contribution, we give analysis of IAB EMC requirements and the conclusions are:

Observation 1: Almost every subclause of BS EMC specification are impacted when introducing repeater in the spec.

Observation 2: There are a lot of difference between UE EMC and BS EMC requirements.

Observation 3: IAB is not in the scope of TS 38.113.

Proposal 1: Add EMC requirements of an IAB-node into the IAB work item.

Proposal 2: To have a new TS to capture IAB EMC requirements in Rel-16.

- References

TS 36.113 v16.0.0

TR 38.874 v16.0.0

TS 38.124 v15.2.0

TS 38.113 v15.5.0

RP-190712