3GPP TSG-RAN WG4 Meeting #91	R4-1905577

Reno, US, 13th– 17th May, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.141 | CR | 1220 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 36.141: category B option 2 unwanted emissions for 2500-2690 MHz band |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15 |  |  |  |  |  | Date: |  |  | 2019-05-17 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In European regulation, BS operating within the band 2500-2690 MHz must follow category B option 2 unwanted emssions. Currently, these bands are not shown in the scope of the option 2 requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Band 7, 38 and 69 are added to scope of option 2 mask |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 3GPP spec is not aligned with ECC regulation |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.3.5.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | 36.104 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

------------------------------ Modified section ------------------------------

6.6.3.5.2.2	Category B (Option 2)

The limits in this subclause are intended for Europe and may be applied regionally for BS operating in band 1, 3, 7, 8, 32, 33, 34, 38, 65 or 69.

For a BS operating in band 1, 3, 7, 8, 32, 33, 34, 38, 65 or 69, emissions shall not exceed the maximum levels specified in Table 6.6.3.5.2.2-1 below for 5, 10, 15 and 20 MHz channel bandwidth:

Table 6.6.3.5.2.2-1: Regional Wide Area BS operating band unwanted emission limits in band 1, 3, 7, 8, 32, 33, 34, 38, 65 or 69 for 5, 10, 15 and 20 MHz channel bandwidth for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Note 1, 2) | Measurement bandwidth (Note 6) |
| --- | --- | --- | --- |
| 0 MHz f < 0.2 MHz | 0.015MHz f_offset < 0.215MHz | -12.5dBm | 30 kHz |
| 0.2 MHz f < 1 MHz | 0.215MHz f_offset < 1.015MHz |  | 30 kHz |
| (Note 8) | 1.015MHz f_offset < 1.5 MHz | -24.5dBm | 30 kHz |
| 1 MHz f min( 10 MHz , fmax) | 1.5 MHz f_offset < min(10.5 MHz, f_offsetmax) | -11.5dBm | 1 MHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -15 dBm (Note 9) | 1 MHz |
| NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be -15dBm/1MHz. NOTE 2: For BS supporting multi-band operation with Inter RF Bandwidth gap < 20MHz the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. |  |  |  |

For a BS operating in band 3, 8 or 65, emissions shall not exceed the maximum levels specified in Table 6.6.3.5.2.2-2 below for 3 MHz channel bandwidth:

Table 6.6.3.5.2.2-2: Regional Wide Area BS operating band unwanted emission limits in band 3, 8 or 65 for 3 MHz channel bandwidth for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Note 1, 2) | Measurement bandwidth (Note 6) |
| --- | --- | --- | --- |
| 0 MHz f < 0.05 MHz | 0.015 MHz f_offset < 0.065 MHz |  | 30 kHz |
| 0.05 MHz f < 0.15 MHz | 0. 065 MHz f_offset < 0.165 MHz |  | 30 kHz |
| 0.15 MHz f < 0.2 MHz | 0.165MHz f_offset < 0.215MHz | -12.5dBm | 30 kHz |
| 0.2 MHz f < 1 MHz | 0.215MHz f_offset < 1.015MHz |  | 30 kHz |
| (Note 8) | 1.015MHz f_offset < 1.5 MHz | -24.5dBm | 30 kHz |
| 1 MHz f 6 MHz | 1.5MHz f_offset < 6.5 MHz, | -11.5dBm | 1 MHz |
| 6 MHz f fmax | 6.5 MHz f_offset < f_offsetmax | -15 dBm | 1 MHz |
| NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be -15dBm/1MHz. NOTE 2: For BS supporting multi-band operation with Inter RF Bandwidth gap < 20MHz the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. |  |  |  |

For a BS operating in band 3, 8 or 65, emissions shall not exceed the maximum levels specified in Table 6.6.3.5.2.2-3 below for 1.4 MHz channel bandwidth:

Table 6.6.3.5.2.2-3: Regional Wide Area BS operating band unwanted emission limits in band 3, 8 or 65 for 1.4 MHz channel bandwidth for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Note 1, 2) | Measurement bandwidth (Note 6) |
| --- | --- | --- | --- |
| 0 MHz f < 0.05 MHz | 0.015 MHz f_offset < 0.065 MHz |  | 30 kHz |
| 0.05 MHz f < 0.15 MHz | 0. 065 MHz f_offset < 0.165 MHz |  | 30 kHz |
| 0.15 MHz f < 0.2 MHz | 0.165MHz f_offset < 0.215MHz | -12.5 dBm | 30 kHz |
| 0.2 MHz f < 1 MHz | 0.215MHz f_offset < 1.015MHz |  | 30 kHz |
| (Note 8) | 1.015MHz f_offset < 1.5 MHz | -24.5 dBm | 30 kHz |
| 1 MHz f 2.8 MHz | 1.5 MHz f_offset < 3.3 MHz | -11.5 dBm | 1 MHz |
| 2.8 MHz f fmax | 3.3 MHz f_offset < f_offsetmax | -15 dBm | 1 MHz |
| NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be -15dBm/1MHz. NOTE 2: For BS supporting multi-band operation with Inter RF Bandwidth gap < 20MHz the test requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. |  |  |  |

------------------------------ End of modified section ------------------------------