3GPP TSG-RAN4 Meeting #91	R4-1905602

# Reno, USA, 13th – 17th May, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 38.133 | CR |  | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Maintenance draftCR on idle mode test cases for FR2 (section A.7.1) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-03-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Correction is needed on FR2 Idle reselection test cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Removal of brackets - Editorial changes in the tables to remove the errors - Correct SintrasearchP from not sent to 50dB since it is not optional to configure - Fix Io for intra case |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test cases configurations are not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex A.7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

## A.7.1	SA: RRC_IDLE state mobility

### A.7.1.1	Cell re-selection to NR

#### A.7.1.1.1	Cell reselection to FR2 intra-frequency NR case

##### A.7.1.1.1.1	Test Purpose and Environment

This test is to verify the requirement for the intra frequency NR cell reselection requirements specified in clause 4.2.2.3.

##### A.7.1.1.1.2	Test Parameters

The test scenario comprises of 1 NR carrier and 2 cells as given in tables A.7.1.1.1.2-1, A.7.1.1.1.2-2 and A.7.1.1.1.2-3. The test consists of three successive time periods, with time duration of T1, T2, and T3 respectively. Only cell 1 is already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing cell 2.

Table A.7.1.1.1.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 2 | 240 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.7.1.1.1.2-2: General test parameters for intra frequency NR cell re-selection test case

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2 | Cell1 |  |
|  | Neighbour cells |  | 1, 2 | Cell2 |  |
| T2 end condition | Active cell |  | 1, 2 | Cell2 |  |
|  | Neighbour cells |  | 1, 2 | Cell1 |  |
| Final condition | Visited cell |  | 1, 2 | Cell1 |  |
| RF Channel Number |  |  | 1, 2 | 1 |  |
| Time offset between cells |  |  | 1, 2 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR2 |  |
|  |  |  | 2 | SSB.2 FR2 |  |
| SMTC configuration |  |  | 1, 2 | SMTC pattern 1 |  |
| DRX cycle length |  | s | 1, 2 | 1.28 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2 | 87 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2 | Not configured |  |
| T1 |  | s | 1, 2 | >7 | During T1, Cell 2 shall be powered off, and during the off time the physical cell identity shall be changed, The intention is to ensure that Cell 2 has not been detected by the UE prior to the start of period T2 |
| T2 |  | s | 1, 2 | 135 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |
| T3 |  | s | 1, 2 | 35 | T3 needs to be defined so that cell re-selection reaction time is taken into account. |

Table A.7.1.1.1.2-3: Cell specific test parameters for intra frequency NR cell re-selection test case in AWGN

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| TDD configuration |  | 1, 2 | TDDConf.3.1 |  |  | TDDConf.3.1 |  |  |
| PDSCH RMC configuration |  | 1 | SR.3.1 TDD |  |  | N/A |  |  |
|  |  | 2 | SR.3.1 TDD |  |  |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.3.1 TDD |  |  | CR.3.1 TDD |  |  |
|  |  | 2 | CR.3.1 TDD |  |  | CR.3.1 TDD |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.3.1 TDD |  |  | CCR.3.1 TDD |  |  |
|  |  | 2 | CCR.3.1 TDD |  |  | CCR.3.1 TDD |  |  |
| OCNG Pattern |  | 1, 2 | OP.1 defined in A.3.2.1 |  |  | OP.1 defined in A.3.2.1 |  |  |
| Initial DL BWP configuration |  | 1, 2 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2 | SSB |  |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1 | -140 |  |  | -140 |  |  |
|  |  | 2 | -137 |  |  | -137 |  |  |
| Pcompensation | dB | 1, 2 | 0 |  |  | 0 |  |  |
| Qhysts | dB | 1, 2 | 0 |  |  | 0 |  |  |
| Qoffsets, n | dB | 1, 2 | 0 |  |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2 | SS-RSRP |  |  | SS-RSRP |  |  |
| AoA setup |  | 1, 2 | Setup 1 defined in A.3.15.1 |  |  | Setup 1 defined in A.3.15.1 |  |  |
|  | dB | 1 | 17 | -3.09 | 2.83 | -infinity | 2.83 | -3.09 |
|  |  | 2 |  |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | -98 |  |  |  |  |  |
|  |  | 2 | -95 |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -107 |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |
|  | dB | 1 | 17 | 14 | 17 | -infinity | 17 | 14 |
|  |  | 2 |  |  |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1 | -81 | -84 | -81 | -infinity | -81 | -84 |
|  |  | 2 | -78 | -81 | -78 | -infinity | -78 | -81 |
| Io | dBm/95.04 MHz | 1 | -51.93 | -50.19 | -50.19 | -51.93 | -50.19 | -50.19 |
|  |  | 2 | -51.93 | -50.19 | -50.19 | -51.93 | -50.19 | -50.19 |
| Treselection | s | 1, 2 | 0 | 0 | 0 | 0 | 0 | 0 |
| SintrasearchP | dB | 1, 2 | 50 |  |  | 50 |  |  |
| Propagation Condition |  | 1, 2 | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

##### A.7.1.1.1.3	Test Requirements

The cell reselection delay to a newly detectable cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure on Cell 2.

The cell re-selection delay to a newly detectable cell shall be less than 130 s.

The cell reselection delay to an already detected cell is defined as the time from the beginning of time period T3, to the moment when the UE camps on cell 1, and starts to send preambles on the PRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure on cell 1.

The cell re-selection delay to an already detected cell shall be less than 27 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a newly detectable cell can be expressed as: Tdetect, NR_Intra + TSI-NR, and to an already detected cell can be expressed as: Tevaluate, NR_ intra + TSI-NR,

Where:

Tdetect, NR_Intra	See Table 4.2.2.3-1 in clause 4.2.2.3

Tevaluate, NR_ intra	See Table 4.2.2.3-1 in clause 4.2.2.3

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 129.28 s, allow 130 s for the cell re-selection delay to a newly detectable cell and 26.88 s for the cell re-selection delay to an already detected cell in the test case, which we allow 27 s.

#### A.7.1.1.2	Cell reselection to FR2 inter-frequency NR case

##### A.7.1.1.2.1	Test Purpose and Environment

This test is to verify the requirement for the inter frequency NR cell reselection requirements specified in clause 4.2.2.4.

##### A.7.1.1.2.2	Test Parameters

The test scenario comprises of 2 cells on 2 different NR carriers respectively as given in tables A.7.1.1.2.2-1, A.7.1.1.2.2-2 and A.7.1.1.2.2-3. The test consists of three successive time periods, with time duration of T1, T2, and T3 respectively. Both cell 1 and cell 2 are already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas and cell 2 is of higher priority than cell 1. Furthermore, UE has not registered with network for the tracking area containing cell 2.

Table A.7.1.1.2.2-1: Supported test configurations

| Configuration | Description for serving cell | Description for target cell |
| --- | --- | --- |
| 1 | 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode | 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 2 | 240 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode | 240 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

Table A.7.1.1.2.2-2: General test parameters for FR2 inter frequency NR cell re-selection test case

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2 | Cell2 | The UE camps on cell 2 in the initial phase and during T1 period the UE reselects to cell 1 |
| T1 end condition | Active cell |  | 1, 2 | Cell1 | The UE shall perform reselection to cell 1 during T1 |
|  | Neighbour cells |  | 1, 2 | Cell2 |  |
| T3 end condition | Active cell |  | 1, 2 | Cell2 | The UE shall perform reselection to cell 2 with higher priority during T3 |
| RF Channel Number |  |  | 1, 2 | 1, 2 |  |
| Time offset between cells |  |  | 1, 2 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR2 |  |
|  |  |  | 2 | SSB.2 FR2 |  |
| SMTC configuration |  |  | 1, 2 | SMTC pattern 1 |  |
| DRX cycle length |  | s | 1, 2 | 1.28 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2 | 87 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2 | Not configured |  |
| T1 |  | s | 1, 2 | 35 | T1 needs to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2 | >7 | During T2, cell 2 shall be powered off, and during the off time the physical cell identity shall be changed. The intention is to ensure that cell 2 has not been detected by the UE prior to the start of period T3. |
| T3 |  | s | 1, 2 | 95 | T3 needs to be defined so that cell re-selection reaction time is taken into account. |

Table A.7.1.1.2.2-3: Cell specific test parameters for FR2 inter frequency NR cell re-selection test case in AWGN

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| TDD configuration |  | 1, 2 | TDDConf.3.1 |  |  | TDDConf.3.1 |  |  |
| PDSCH RMC configuration |  | 1, 2 | SR.3.1 TDD |  |  | N/A |  |  |
| RMSI CORESET parameters |  | 1, 2 | CR.3.1 TDD |  |  | CR.3.1 TDD |  |  |
| RMSI CORESET RMC configuration |  | 1, 2 | CCR.3.1 TDD |  |  | CCR.3.1 TDD |  |  |
| OCNG Pattern |  | 1, 2 | OP.1 defined in A.3.2.1 |  |  | OP.1 defined in A.3.2.1 |  |  |
| Initial DL BWP configuration |  | 1, 2 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2 | SSB |  |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1 | -140 |  |  | -140 |  |  |
|  |  | 2 | -137 |  |  | -137 |  |  |
| Pcompensation | dB | 1, 2 | 0 |  |  | 0 |  |  |
| Qhysts | dB | 1, 2 | 0 |  |  | 0 |  |  |
| Qoffsets, n | dB | 1, 2 | 0 |  |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2 | SS-RSRP |  |  | SS-RSRP |  |  |
| AoA setup |  | 1, 2 | Setup 1 defined in A.3.15.1 |  |  | Setup 1 defined in A.3.15.1 |  |  |
|  | dB | 1 | 15 | 15 | 15 | -3 | -infinity | 13 |
|  |  | 2 |  |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | -98 |  |  |  |  |  |
|  |  | 2 | -95 |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -107 |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |
|  | dB | 1 | 15 | 15 | 15 | -3 | -infinity | 13 |
|  |  | 2 |  |  |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1 | -83 | -83 | -83 | -101 | -infinity | -85 |
|  |  | 2 | -80 | -80 | -80 | -98 | -infinity | -82 |
| Io | dBm/95.04 MHz | 1 | -53.88 | -53.88 | -53.88 | -67.25 | -infinity | -55.80 |
|  |  | 2 | -53.88 | -53.88 | -53.88 | -67.25 | -infinity | -55.80 |
| Treselection | s | 1, 2 | 0 | 0 | 0 | 0 | 0 | 0 |
| SnonintrasearchP | dB | 1, 2 | 50 |  |  | Not sent |  |  |
| Threshx, high | dB | 1, 2 | 48 |  |  | 48 |  |  |
| Threshserving, low | dB | 1, 2 | 44 |  |  | 44 |  |  |
| Threshx, low | dB | 1, 2 | 50 |  |  | 50 |  |  |
| Propagation Condition |  | 1, 2 | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

##### A.7.1.1.2.3	Test Requirements

The cell reselection delay to a higher priority cell is defined as the time from the beginning of time period T3, to the moment when the UE camps again on cell 2, and starts to send preambles on the PRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure on cell 2.

The cell re-selection delay to a higher priority cell shall be less than 87 s.

The cell reselection delay to a lower priority cell is defined as the time from the beginning of time period T1, to the moment when the UE camps on cell 1, and starts to send preambles on the PRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure on cell 1.

The cell re-selection delay to a lower priority cell shall be less than 27 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a higher priority cell can be expressed as: Thigher_priority_search + Tevaluate, NR_ inter + TSI-NR, and to a lower priority cell can be expressed as: Tevaluate, NR_ inter + TSI-NR,

Where:

Thigher_priority_search	See clause 4.2.2.7

Tevaluate, NR_ inter	See Table 4.2.2.4-1 in clause 4.2.2.4

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 86.88 s, allow 87 s for the cell re-selection delay to a higher priority cell and 26.88 s for the cell re-selection delay to a lower priority cell in the test case, which we allow 27 s.

<End of Changes>