3GPP TSG-RAN WG4 Meeting #91	     R4-1905705

Reno, USA, 13th – 17th May 2019

Agenda item:       6.10.7.4.1

Source:                Intel Corporation

Title:	                     Discussion on TCI state switching requirements

Document for:     Discussion

# Introduction

In RAN4#90bis TCI state switching requirements were extensively discussed the following agreements were reached and way forward [1] was approved.

Definition of Known/Unknown TCI state - Definition of known TCI state for UE supporting Power Class 1 - L1-RSRP or L3-RSRP report made in [x] ms - The TCI state is detectable during the TCI state switching period for both known and unknown TCI state switching requirement MAC CE based TCI State switch - Definition of switching delay: From the slot with PDSCH carrying the activation command to the slot when PDCCH can be received based on new TCI state - For UE supporting Power Class 1 - Requirements are defined for known and unknown TCI state - The switching delay for unknown case may include time for L1-RSRP measurement - Requirement for switching delay for known case: THARQ + 3ms - No interruption allowed due to MAC based TCI switching except for intra-band non-contiguous CA case - FFS on the case if the propagation time is changed due to TCI state switching - Whether to define scheduling restriction requirement for MAC based TCI switching depends on the conclusions for MRTD. DCI based TCI State switch - Definition of switching delay: From the last symbol of PDCCH to first symbol of PDSCH - For UE supporting Power Class 1 - Requirements are defined for known TCI state - No requirements for unknown TCI state - Requirement for switching delay for known TCI state: UE capability for timeDurationForQCL - In RRM test for DCI based TCI state switch, PDSCH is always scheduled with lowest MCS for new TCI state - No interruption allowed due to DCI based TCI switching except for intra-band non-contiguous CA case - FFS on the case if the propagation time is changed due to TCI state switching RRC based TCI State switch - RRC based TCI state change requirements shall be defined MAC CE based active TCI state list update - Requirements shall be introduced for MAC CE based active state list update

In this contribution we discuss the remaining open issues related to TCI state switching requirements.

# Discussion

The open issues related to TCI state switching requirements are:

- Applicability of known/unknown TCI state for UE supporting power class 2/3/4

- Value of X for defining known TCI state for Power class 1 and other power classes if supported

- Requirements for the case that UE has never reported L1-RSRP or L3-RSRP for target TCI state prior to TCI state switch activation

- DCI based TCI state switch requirements for UE supporting power class 2/3/4

- RRC based TCI state switching delay and interruption

- Interruption due to TCI state switch for intra band non-contiguous CA

- Delay requirement for MAC CE based active state list update and DCI based switch to newly active TCI

## Definition of Known TCI state

In RAN4#90bis the definition of known TCI state for UE supporting power class 1 was agreed as

  - L1-RSRP or L3-RSRP report made in [x] ms

The value of x for UE supporting power class 1 is FFS and also whether definition of known TCI state is applicable to other power classes.

The TCI state is said to be known if the UE can switch to the new TCI state without any additional measurements. If we assume that the UE is making and reporting L1-RSRP measurements based on SSB, the time to determine the best Rx beam for a TCI state is 8xTSSB, where TSSB is the periodicity of the SSB associated with the TCI state. If the UE is assumed to update its L1-RSRP measurements for beam tracking every 8xTSSB ms, we could define the validity time for that measurement as 8xTSSB ms.

Observation #1: In case of SSB based L1-RSRP measurements for beam tracking, the validity of the measurement could be defined as 8xTSSB ms.

In case of a fixed or low mobility UE the SSB periodicity could be assumed to be high, such as 160ms. The validity of L1-RSRP report for a power class 1 UE would be 1280ms. For a mobile UE, the UE is required perform initial access, cell identification and acquisition with SSB periodicity of 20ms. Assuming 20ms as the nominal SSB periodicity for a mobile UE, the L1-RSRP measurement and report would be made every 160ms and the report validity could be defined as 160ms.

Using SSB based measurements as the baseline, the validity of a measurement could be defined as 1280 ms for a fixed UE and 160ms for a mobile UE.

The resources configured for measurement could be periodic or aperiodic and also the report for the L1-RSRP measurements could be periodic or aperiodic. When a report is made based on a measurement, it is difficult to determine how long ago the measurement was made by the UE prior to the report. If the definition of unknown TCI state is based on the time of L1-RSRP report, there is a possibility that the measurement was made beyond the validity time of the measurement and the measurement is outdated and no longer considered as known TCI state.

Figure 1: Definition of known TCI state

Observation #2: When a measurement report is made by the UE, it is difficult to determine how long ago the measurement was made.

Observation #3: If the definition of known TCI state is based on time of L1-RSRP report, the measurement at the UE might be outdated and no longer known

We propose to define known TCI state based on when the resource for beam reporting and measurement is configured. TCI state is defined as known if the time between TCI state switch and resource transmission for beam reporting and measurement is within X ms and at least one report is available at the gNB for the resource between the measurement occasion and TCI state switch.

Proposal #1: Define known/ unknown TCI state based on time of last transmission for the resource for beam reporting/measurement

  - There shall be at least 1 report from the UE between the measurement opportunity and TCI state switch

  - Known TCI state if TCI state switch is within X ms of last transmission of the resource for beam reporting

For TCI state switch to a known TCI state, the UE is not allowed additional time to make measurements for Rx beam refinement. When the channel and propagation conditions are not changing dynamically, based on the validity time of measurement defined above, we can define X for known TCI state as [1280] ms for power class 1 and as [160] ms for power class 2/3/4. The conditions for known TCI state shall also include the pre-condition that the TCI state is detectable during the TCI state change. The UE is scheduled with the lowest possible code rate with the new TCI state – lowest MCS for PDSCH and highest aggregation level for PDCCH in order to successfully receive with the new TCI state without additional Rx beam refinement.

Proposal #2: Known TCI state is defined as TCI state switch within Xms of last transmission for the resource for beam reporting/measurement

  - X = [1280] ms for UE supporting power class 1

  - X = [160] ms for UE supporting power class 2/3/4

  - Target TCI state shall detectable

  - The UE shall be configured with highest possible aggregation level for PDCCH with the new TCI state for MAC CE based switch

  - The UE shall be scheduled with lowest possible MCS for PDSCH with the new TCI state for DCI based switch

The known TCI state definition above shall apply to both MAC CE and DCI based TCI state switching.

Proposal #3: The known TCI state definition shall apply to MAC CE and DCI based TCI state switching

Another open issue for TCI state switching is whether requirements shall be for the case that UE has never reported L1-RSRP or L3-RSRP for target TCI state prior to TCI state switch activation. Although such a scenario might be possible, we believe it’s not common and also the motivation by network to change to a TCI state not measured and reported by the UE.

Observation #4: The motivation to introduce requirements for the case that UE has never reported L1-RSRP or L3-RSRP for target TCI state prior to TCI state switch activation is unclear

Proposal #4: No requirements shall be introduced for the case that UE has never reported L1-RSRP or L3-RSRP for target TCI state prior to TCI state switch activation

## MAC CE based TCI state switching

In RAN4#90bis it was agreed to define MAC CE based TCI state switch delay for known TCI state for UE supporting power class 1 as

TTCI-Switch, MAC-CE, known = THARQ + 3ms

For unknown TCI state, the additional time needed is for L1-RSRP measurement and Rx beam refinement. The TCI switching delay for unknown TCI state could be defined as time for Rx beam refinement plus TCI state switch delay for known TCI state.

TTCI-Switch, MAC-CE, unknown = TRxBeamAcq + THARQ + 3ms

Where TRxBeamAcq is the time for L1-RSRP measurement and Rx beam refinement and can be defined as

  - For SSB: 8xTSSB-RS

  - For periodic CSI-RS: NxTCSI-RS ; Where N=8 for Repetition ‘OFF’ set; N=ceil(MaxNumRxBeam/#CSI-RS in set) for Repetition ‘ON’ set

  - For aperiodic CSI-RS: TL1-RSRP ; Where TL1-RSRP is the time for Rx beam measurement/ refinement for the target TCI state

Proposal #5: Define MAC CE based TCI state switch to unknown TCI state as 				TTCI-Switch, MAC-CE, unknown = TRxBeamAcq + THARQ + 3ms

Proposal #6: Define TRxBeamAcq as: For SSB: 8xTSSB-RS ; For periodic CSI-RS: NxTCSI-RS [Where N=8 for Repetition ‘OFF’ set; N=ceil(MaxNumRxBeam/#CSI-RS in set) for Repetition ‘ON’ set] ; For aperiodic CSI-RS: TL1-RSRP [Where TL1-RSRP is the time for Rx beam measurement/ refinement for the target TCI state]

The conditions for known and unknown TCI state are defined for all power class, hence the requirements could be defined for all power class.

Proposal #7: Requirements for MAC CE based TCI state switching for known and unknown TCI state shall be defined for all UE power class

## DCI based TCI state switching

For DCI based TCI state switching it was agreed in RAN4#90bis to define requirements for known TCI state for UE supporting power class 1. Since the conditions for known and unknown TCI state are defined for all power class, hence DCI based TCI state requirements could be defined for all power class.

Proposal #8: Requirements for DCI based TCI state switching for known TCI state shall be defined for all UE power class

## RRC based TCI state switching

In RAN4#90bis it was agreed to introduce requirements for RRC based TCI state switching. For RRC based TCI state change, the assumption is that only one TCI state is configured in RRC and a RRC re-configuration is required to change the active TCI state. The gNB could change the active TCI state based on L3 RSRP measurement received by the UE. Based on the same criteria for known and unknown TCI state as defined in Section 2.1, RRC based TCI stae switch can be defined for known and unknown TCI state.  For the case where only 1 TCI state is configured via RRC, there is no active TCI list and hence no prior measurement at the UE for that TCI state for Rx beam.

For RRC based TCI state switch, the delay needs to account for RRC processing delay (TRRC proc), RRC application delay (TRRCAppl) to account for any UE preparation time for new RRC configuration. In case of unknown TCI state time for L1 RSRP measurement and Rx beam, time/ frequency offset acquisition (TRxBeamAcq). Also, the switching delay applies when RRC re-configuration message is only for TCI state change.

Proposal #9: RRC based TCI state switching delay applies when RRC re-configuration message is only for TCI state switch

The total time delay for RRC based TCI state for known TCI state can be defined as:

TTCI-Switch, RRC, known = TRRC proc + TRRC Appl

Proposal #10: RRC based TCI state switching delay for unknown TCI state is defined as 			TTCI-Switch, RRC, unknown = TRRC proc + TRRC Appl + TMeas,Acq

The total time delay for RRC based TCI state for unknown TCI state can be defined as:

TTCI-Switch, RRC, unknown = TRRC proc + TRRC Appl + TMeas,Acq

Proposal #11: RRC based TCI state switching delay for unknown TCI state is defined as 			TTCI-Switch, RRC, unknown = TRRC proc + TRRC Appl + TMeas,Acq

In case of RRC based TCI state change, once the RRC message is decoded, the UE will be unable to receive PDCCH/PDSCH on the DL or transmit on the UL. However it is difficult for the network to determine when the RRC message is decoded. Hence we propose to start the scheduling restriction from the time when PDSCH carrying RRC message is received by the UE. The scheduling restriction ends when the UE is ready to receive with the new TCI state, at the end of the TCI state switching delay.

Proposal #12: For RRC based TCI state switching, scheduling restriction is defined from symbol after the PDSCH carrying RRC re-configuration is received by the UE until the end of TCI state switching delay

## Interruption

In RAN4#90bis it was agreed that there shall be no interruptions on other CCs in case of TCI state switching for intra band contiguous CA. But it was FFS if there should be interruption in case of non-contiguous CA. The discussion for MRTD for intra band non-contiguous CA is still ongoing. Based on the proposals in our companion paper [2], the options for MRTD for non-contiguous CA are favourable: 								Option 1: half CP of the larger SCS among the CCs for FR1 and for FR2

Option 3: 260ns for both FR1 and FR2

With the assumption that the MRTD will be defined as one of the options proposed above, we don’t need to define any interruption on other CCs for MAC-CE, DCI or RRC based TCI state switching. Otherwise interruption needs to be defined for intra band non-contiguous CA.

Proposal #13: For intra band non-contiguous CA, no interruption is defined if MRTD is defined as no longer than 260ns or half CP

## Active TCI state list update

In RAN4#90bis it was agreed to define delay requirements for MAC CE based active state list update. However this is not testable unless followed by DCI based TCI state switch to newly activated TCI state. Based on the assumption of known TCI state, for active TCI state list update, a MAC CE activation will be required and the delay for MAC CE based activation is THARQ + 3ms. The network can send a DCI based TCI switch to the newly activated TCI state in slot THARQ + 3ms + 1slot. The UE shall be able to receive PDSCH on the new TCI state after timeDurationForQCL. Hence the delay for PDSCH reception on newly activated known TCI state can be defined as

TNewActive,known = THARQ + 3ms +1slot + timeDurationForQCL

Proposal #14: Define delay requirement for PDSCH reception on newly activated known TCI state as 	TNewActive,known = THARQ + 3ms +1slot + timeDurationForQCL

# Conclusion

In this paper we present our views on TCI state switching delay requirements and propose the following:

Definition of known/unknown TCI state:

Proposal #1: Define known/ unknown TCI state based on time of last transmission for the resource for beam reporting/measurement

  - There shall be at least 1 report from the UE between the measurement opportunity and TCI state switch

  - Known TCI state if TCI state switch is within X ms of last transmission of the resource for beam reporting

Proposal #2: Known TCI state is defined as TCI state switch within Xms of last transmission for the resource for beam reporting/measurement

  - X = [1280] ms for UE supporting power class 1

  - X = [160] ms for UE supporting power class 2/3/4

  - Target TCI state shall detectable

  - The UE shall be configured with highest possible aggregation level for PDCCH with the new TCI state for MAC CE based switch

  - The UE shall be scheduled with lowest possible MCS for PDSCH with the new TCI state for DCI based switch

Proposal #3: The known TCI state definition shall apply to MAC CE and DCI based TCI state switching

Proposal #4: No requirements shall be introduced for the case that UE has never reported L1-RSRP or L3-RSRP for target TCI state prior to TCI state switch activation

MAC CE based TCI state switching:

Proposal #5: Define MAC CE based TCI state switch to unknown TCI state as 				TTCI-Switch, MAC-CE, unknown = TRxBeamAcq + THARQ + 3ms

Proposal #6: Define TRxBeamAcq as: For SSB: 8xTSSB-RS ; For periodic CSI-RS: NxTCSI-RS [Where N=8 for Repetition ‘OFF’ set; N=ceil(MaxNumRxBeam/#CSI-RS in set) for Repetition ‘ON’ set] ; For aperiodic CSI-RS: TL1-RSRP [Where TL1-RSRP is the time for Rx beam measurement/ refinement for the target TCI state]

Proposal #7: Requirements for MAC CE based TCI state switching for known and unknown TCI state shall be defined for all UE power class

DCI based TCI state switching:

Proposal #8: Requirements for DCI based TCI state switching for known TCI state shall be defined for all UE power class

RRC based TCI state switching:

Proposal #9: RRC based TCI state switching delay applies when RRC re-configuration message is only for TCI state switch

Proposal #10: RRC based TCI state switching delay for unknown TCI state is defined as 			TTCI-Switch, RRC, unknown = TRRC proc + TRRC Appl + TMeas,Acq

Proposal #11: RRC based TCI state switching delay for unknown TCI state is defined as 			TTCI-Switch, RRC, unknown = TRRC proc + TRRC Appl + TMeas,Acq

Proposal #12: For RRC based TCI state switching, scheduling restriction is defined from symbol after the PDSCH carrying RRC re-configuration is received by the UE until the end of TCI state switching delay

Interruption:

Proposal #13: For intra band non-contiguous CA, no interruption is defined if MRTD is defined as no longer than 260ns or half CP

Active TCI state list update:

Proposal #14: Define delay requirement for PDSCH reception on newly activated known TCI state as 	TNewActive,known = THARQ + 3ms +1slot + timeDurationForQCL

# Reference

- R4-1904840, “Way forward on TCI State Switching Requirements”, Intel Corporation

- R4-1905751, “On MTTD and MRTD for intra-band contiguous EN-DC”, Intel Corporation