3GPP TSG-RAN4 Meeting #91	R4-1905768

# Reno, USA, May 13th – 17th 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | CRNum | rev | - | Current version: | V15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to 38.101-3: Configured output power for inter-band NE-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT_Core |  |  |  |  |  | Date: |  |  | 2019-05-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There is no configured transmitted power for inter-band NE-DC in current 38.101-3 in Rel-15. Based on approved WFs R4-1905082 and R4-1905086, the configured transmitted power for inter-band NE-DC can be defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Create new clause: 6.2B.4a - Configured output power for NE-DC |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 38.101-3 is not complete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clause 6.2B.4a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-3 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

< start of changes >

### 6.2B.4a	Configured output power for NE-DC

#### 6.2B.4a.1	Configured output power level

##### 6.2B.4a.1.1	Intra-band contiguous NE-DC

##### 6.2B.4a.1.2	Intra-band non-contiguous NE-DC

##### 6.2B.4a.1.3	Inter-band NE-DC within FR1

For inter-band dual connectivity with one uplink serving cell per CG on E-UTRA and NR respectively, the UE is allowed to set its configured maximum output power PCMAX,c(i),i for serving cell c(i) of CG i, i = 1,2, and its total configured maximum transmission  power for NE-DC operation, PTotalNE-DC= 10log10(PtotalNE-DC) with PtotalNE-DC as specified in sub-clause 7.6.1A of [10].

The configured maximum output power PCMAX_ E-UTRA,c (p) in sub-frame p for the configured E-UTRA uplink carrier shall be set within the bounds:

PCMAX_L_ E-UTRA,c (p) ≤  PCMAX_ E-UTRA,c (p) ≤  PCMAX H _ E-UTRA,c (p)

where PCMAX_L_ E-UTRA,c and PCMAX H _ E-UTRA,c are the limits for a serving cell c as specified in TS 36.101 [4] sub-clause 6.2.5 modified by PLTE as follows:

PCMAX_L_ E-UTRA,c = MIN { PEMAX, NE-DC , (PPowerClass, NE-DC – ΔPPowerClass,NE-DC ), MIN(PEMAX,c , PLTE) – tC_ E-UTRA, c,  (PPowerClass – ΔPPowerClass) – MAX(MPRc + A-MPRc + ΔTIB,c  + TC_ E-UTRA, c + TProSe, P-MPRc)}

PCMAX H _ E-UTRA,c = MIN {PEMAX,c,  PEMAX, EN-DC  , (PPowerClass, NE-DC – ΔPPowerClass,NE-DC ), PLTE, PPowerClass – ΔPPowerClass}

with exception that

-	if no symbol of slot  of the NR that is indicated as uplink or flexible by TDD-UL-DL-ConfigurationCommon or TDD-UL-DL-ConfigDedicated overlaps with subframe  of the E-UTRA; or

-	if NR slot(s) that is indicated as downlink by TDD-UL-DL-ConfigurationCommon or TDD-UL-DL-ConfigDedicated does not overlap with subframe  of the E-UTRA; then

PCMAX_L_ E-UTRA,c = MIN { PEMAX, NE-DC , (PPowerClass, NE-DC – ΔPPowerClass,NE-DC ), PEMAX,c  – tC_ E-UTRA, c,  (PPowerClass – ΔPPowerClass) – MAX(MPRc + A-MPRc + ΔTIB,c  + TC_ E-UTRA, c + TProSe, P-MPRc)}

PCMAX H _ E-UTRA,c = MIN {PEMAX,c,  PEMAX, EN-DC  , (PPowerClass, NE-DC – ΔPPowerClass,NE-DC ), PPowerClass – ΔPPowerClass}

The configured maximum output power PCMAX_ NR,c (q) in physical-channel q for the configured NR carrier shall be set within the bounds:

PCMAX_L,f,c,,NR (q) ≤  PCMAX,f,c,NR (q) ≤  PCMAX_H,f,c,NR (q)

where PCMAX_L_ NR,c and PCMAX H _ NR,c are the limits for a serving cell c as specified in sub-clause 6.2.4 of TS 38.101-1 [2] modified by PNR as follows:

PCMAX_L,f,c,,NR = MIN { PEMAX, NE-DC  , (PPowerClass, NE-DC – ΔPPowerClass,NE-DC ), MIN(PEMAX,c , PNR ) - TC_NR, c,  (PPowerClass – ΔPPowerClass) – MAX(MPRc + A-MPRc+ ΔTIB,c + TC_NR, c + ∆TRxSRS,  P-MPRc) }

PCMAX_H,f,c,NR = MIN {PEMAX,c, PEMAX, NE-DC  , (PPowerClass, NE-DC – ΔPPowerClass,NE-DC ), PNR , PPowerClass – ΔPPowerClass }

-	PLTE signalled by RRC as p-MaxEUTRA in [36.331]

-	PNR signalled by RRC as p-NR-FR1 defined in [38.331]

-	ΔTc_E-UTRA, c = 1.5dB when NOTE 2 in Table 6.2.2-1 in TS 36.101 [4] applies for a serving cell c, otherwise TC_ E-UTRA,c = 0dB;

-	TC_NR,c = 1.5dB when NOTE 3 in Table 6.2.1-1 in TS 38.101-1 [2] applies for a serving cell c, otherwise TC_NR,c = 0dB;

-	ΔTIB,c specified in sub-clause  6.2Ba.4.2 for NE-DC, the individual Power Class defined in table 6.2Ba.1.1 and any other additional power reductions parameters specified in sub-clauses  6.2Ba.2 and 6.2Ba.3 for NE-DC are applicable to PCMAX_ E-UTRA,c and PCMAX_ NR,c evaluations.

-	ΔPPowerClass,NE-DC = 3 dB for a power class 2 capable NE-DC UE when  LTE UL/DL configuration is 0 or 6; or LTE UL/DL configuration is 1 and special subframe configuration is 0 or 5; or the IE p-maxUE-FR1 as defined in TS 38.331 [7] is provided and set to the maximum output power of the default power class or lower; otherwise ΔPPowerClass,NE-DC = 0 dB;

If the transmissions from NR and E-UTRA do not overlap, then the complete sub-clauses for configured transmitted power for E-UTRA and NR respectively from their own specifications apply with the modifications specified above. The lower value between PPowerClass, NE-DC or PEMAX, NE-DC shall not be exceeded at any time by UE.

PTotalNE-DC = 10log10(PtotalNE-DC) with PTotalNE-DC the configured maximum transmission power for NE-DC operation as specified in sub-clause 7.6 of [10].

The total configured maximum transmission power for both synchronous and non-synchronous operation is

PTotalNE-DC= MIN { PEMAX, NE-DC ,PPowerClass, NE-DC – ΔPPowerClass, NE-DC }

If the UE does not support dynamic power sharing,

PTotalNE-DC= MIN { PEMAX, NE-DC ,PPowerClass, NE-DC  – ΔPPowerClass, NE-DC } + 0.3 dB

If the NE-DC UE does not support dynamic power sharing, then the complete sub-clauses for configured transmitted power for E-UTRA and NR respectively from their own specifications TS 36.101 [4] and TS 38.101-1 [2] respectively apply with the modifications specified above and PTotalNE-DC applies.

When a UE supporting dynamic  sharing is configured for overlapping E-UTRA uplink and NR uplink transmissions, the UE can set its configured maximum output power PCMAX_ E-UTRA,c and PCMAX_ NR,c for the configured E-UTRA and NR uplink carriers, respectively, and its configured maximum transmission power for NE-DC operation, PTotalNE-DC, as specified above.

The measured total maximum output power PUMAX over both CGs/RATs, measured over the transmission reference time duration is

PUMAX = 10 log10 [pUMAX,c,E-UTRA + pUMAX,c,NR],

where pUMAX,c,E-UTRA and pUMAX,c,NR denotes the measured output power of serving cell c for E-UTRA and NR respectively, expressed in linear scale.

The measured total configured maximum output power PUMAX shall be within the following bounds:

PCMAX_L -TLOW (PCMAX_L)  ≤  PUMAX  ≤  PCMAX_H + THIGH (PCMAX_H)

with the tolerances TLOW(PCMAX_L) and THIGH(PCMAX_H) for applicable values of PCMAX specified in Table 6.2B.4a.1.3-2.

When an UL subframe transmission p from E-UTRA overlap with a physical-channel q from the NR, then for PUMAX evaluation, the E-UTRA subframe p is taken as reference period TREF and always considered as the reference measurement duration and the following rules are applicable.

TREF and Teval are specified in Table 6.2B.4a.1.3-1 when same or different subframe and physical-channel durations are used in aggregated carriers. PPowerClass ,NE-DC shall not be exceeded by the UE during any evaluation period of time.

Table 6.2B.4a.1.3-1: PCMAX evaluation window

| transmission duration | TREF | Teval |
| --- | --- | --- |
| Different transmission duration in different RAT carriers | LTE Subframe | Min(Tno_hopping, Physical Channel Length) |

For each TREF, the PCMAX_H is evaluated per Teval and given by the maximum value over the transmission(s) within the Teval as follows:

PCMAX_H  = MAX { PCMAX_ NE-DC _H (p,q) , PCMAX_ NE-DC _H (p,q+1), … , PCMAX_ NE-DC _H (p,q+n) }

where PCMAX_ NE-DC _H are the applicable upper limits for each overlapping scheduling unit pairs (p,q) , (p, q+1) , up to (p, q+n) for each applicable Teval duration, where q+n is the last NR UL physical-channel overlapping with LTE subframe p.

While PCMAX_L is computed as follows:

PCMAX_L = MIN { PCMAX_ NE-DC _L (p,q) , PCMAX_ NE-DC _L (p,q+1), … , PCMAX_ NE-DC _L (p,q+n)}

where PCMAX_NE-DC_L are the applicable lower limits for each overlapping scheduling unit pairs (p,q) , (p, q+1) , up to (p, q+n) for each applicable Teval duration, where q+n is the last NR UL physical-channel overlapping with LTE subframe p,

With

PCMAX_ NE-DC _H(p,q) = MIN {10 log10 [pCMAX H _ E-UTRA,c (p) + pCMAX H,f,c,NR c(q)], PEMAX, NE-DC ,PPowerClass, NE-DC}

And:

a = 10 log10 [pCMAX_ E-UTRA,c (p) +pCMAX,f,c,NR (q) ] > PTotalNE-DC

If a = TRUE

PCMAX_ NE-DC _L(p,q) = MIN {10 log10 [pCMAX L _ E-UTRA,c (p) ], PEMAX, NE-DC ,PPowerClass, NE-DC}

Else

PCMAX_ NE-DC _L(p,q) = MIN {10 log10 [pCMAX L _ E-UTRA,c (p) + pCMAX L,f,c,,NR c(q)], PEMAX, NE-DC ,PPowerClass, NE-DC}

where

-	pCMAX H _ E-UTRA,c (p) is the E-UTRA higher limit of the maximum configured power expressed in linear scale;

-	pCMAX H _ NR,c (q) is the NR higher limit of the maximum configured power expressed in linear scale;

-	pCMAX L _ E-UTRA,c (p) is the E-UTRA lower limit of the maximum configured power expressed in linear scale;

-	pCMAX L _ NR,c(q) is the NR lower limit of the maximum configured power expressed in linear scale;

-	PPowerClass, NE-DC is defined in sub-clause 6.2B.1a.3-1 for inter-band NE-DC;

-	pCMAX_ E-UTRA,c (p) is the linear value of PCMAX_ E-UTRA,c (p), the real configured max power for LTE

-	pCMAX,f,c,NR (q) is the linear value of PCMAX,f,c,NR (q), the real configured max power of NR

Table 6.2B.4a.1.3-2: PCMAX tolerance for Dual Connectivity LTE-NR

| PCMAX(dBm) | Tolerance TLOW (PCMAX_L) (dB) | Tolerance THIGH (PCMAX_H) (dB) |
| --- | --- | --- |
| 23 ≤ PCMAX ≤ 33 | [3.0] | [2.0] |
| 22 ≤ PCMAX < 23 | [5.0] | [2.0] |
| 21 ≤ PCMAX< 22 | [5.0] | [3.0] |
| 20 ≤ PCMAX < 21 | [6.0] | [4.0] |
| 16 ≤ PCMAX < 20 | [5.0] |  |
| 11 ≤ PCMAX < 16 | [6.0] |  |
| -40 ≤ PCMAX < 11 | [7.0] |  |
| NOTE 1: For UEs not indicating support of dynamic power sharing, the upper tolerance Thigh shall be reduced by 0.3 dB for P ≥ 20 dBm. |  |  |

When LTE and NR transmissions overlap and the condition a = TRUE, PUMAX,f,c,NR (q) for MCG, under nominal conditions, shall meet

PUMAX,f,c,NR (q) ≤  10log(pCMAX H, f,c,,NR c (q)) + THIGH (10log(pCMAX H, f,c,,NR c (q))).

with the tolerances TLOW and THIGH for applicable values of PCMAX specified in Table 6.2B.4a.1.3-2.

When LTE and NR transmissions overlap and the condition a = FALSE), then PUMAX, under nominal conditions, shall be within the following bounds:

PCMAX_L -TLOW (PCMAX_L)  ≤  PUMAX  ≤  PCMAX_H + THIGH (PCMAX_H)

where PCMAX_L, PCMAX_H, and PUMAX are specified above with the tolerances TLOW and THIGH specified in Table 6.2B.4a.1.3-2 for applicable values of PCMAX_L and PCMAX_H.

< end of changes >